| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D285 |
| --- | --- |

| Title: | Switching Interpolation Filter (SIF) scheme with low-complexity decision algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7587 to-yoshino@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

In the previous meeting, we proposed a Switching Interpolation Filter (SIF) approach [1] to improve the coding performance. However, SIF approach with selecting the best filter based on RDO requires large computational cost. In this contribution, we introduce low-complexity approach for estimating the best filter coefficients, and we report the coding performance of the proposed SIF scheme with the low-complexity filter decision. Furthermore, 8-tap filter framework was employed to evaluate the performance under the high coding efficiency. The experimental result shows the BD-rate Y improved 0.4% on average against TMuC0.9 under LoDe/LoCo condition while maintaining the encoding/decoding time.

# Introduction

In the previous meeting, we presented SIF approach that pre-defined 8 candidates of 6-tap filter coefficients. Also, we showed an example of low-complexity approach for estimating the best filter coefficients. This approach estimated motion compensated (MC) prediction error from the result of global MC prediction, and filter coefficients were determined from the frequency characteristics estimated by utilizing MC prediction error. However, this approach still increases the complexity by 30% compared to H.264.

To overcome this problem, this contribution introduces low-complexity approach for estimating the desired interpolation filter characteristics in the current frame. The detailed mechanism is described in Section 3.

Furthermore, 8-tap filter framework and high-accuracy generation for interpolated pixel value at quarter-pel position are also incorporated as shown in the following.

# Switching Interpolation Filter

First of all, switching interpolation filter approach is introduced under the following precondition. This contribution employs 8-tap filter, although only 6-tap filter was assumed in the JCTVC-C183 [1].

JCTVC-C183 [1] pre-defined up to 8 candidates of filter coefficients, while only 3 candidates are assumed as 8-tap filter coefficients in the proposed scheme.

- Filter set 1 : [-18  31 -54 169 169 -54  31 -18]/256

- Filter set 2 : [ -3  18 -44 157 157 -44  18  -3]/256

- Filter set 3 : [ 10   3 -31 146 146 -31   3  10]/256

- Filter set 4 : [ 20 -10 -18 136 136 -18 -10  20]/256

- Filter set 5 : [ 20 -24  -4 136 136  -4 -24  20]/256

- Filter set 6 : [ 13 -31  10 136 136  10 -31  13]/256

- Filter set 7 : [ -3 -33  28 136 136  28 -33  -3]/256

- Filter set 8 : [-18 -26  44 128 128  44 -26 -18]/256

In JCTVC-C183 [1], pixel value at quarter-pel position was generated by bi-liner process on pixel values at half-pel position. In this contribution, pixel value at quarter-pel position is generated by applying 8-tap filter to pixel values at half-pel and integer-pel position.

Our proposed filter is separable into horizontal and vertical filter. Therefore, interpolation filter type can be selected for horizontal and vertical direction, independently. The selected filter identifier is described as slice header information.

# Low-complexity approach for estimating filter coefficient

In order to estimate the MC prediction error at low complexity, an enhanced approach is introduced. In this approach, MC prediction error in the current frame is regarded as identical to the prediction error preserved while encoding the reference frame. The desired interpolation filter characteristics for both horizontal and vertical directions are determined considering the estimated MC prediction error. A pre-defined filter whose frequency characteristics are close to the desirable case is adopted frame by frame.

# Experimental Results

In order to evaluate the coding performance, coding experiment was conducted. The proposed scheme introduced in Section 2 and 3 was implemented on TMuC0.9. Table 1 and Table 2 show the BD-rateY against TMuC0.9 under LoDe/LoCo condition.

Table 1 : summary of the results for LoDe/LoCo

|  | LoDe/LoCo |
| --- | --- |
|  | Y BD-rate |
| Class A |  |
| Class B | -0.4 |
| Class C | -0.4 |
| Class D | -0.5 |
| Class E | -0.5 |
| All | -0.4 |
| Enc Time[%] | 100% |
| Dec Time[%] | 100% |

Table 2 : Experimental results (Y BD-rate [%]) for LoDe/LoCo

| Class | Sequence | BD-rate Y |
| --- | --- | --- |
| B1 | Kimono | -0.01 |
|  | ParkScene | -0.19 |
| B2 | Cactus | -0.27 |
|  | BasketBallDrive | -0.08 |
|  | BQTerrace | -1.23 |
| C | BasketBallDrill | -0.49 |
|  | BQMall | -0.11 |
|  | PartyScene | -0.84 |
|  | RaceHorses | -0.05 |
| D | BasketBallPass | -0.05 |
|  | BQSquare | -1.06 |
|  | BlowingBubbles | -0.65 |
|  | RaceHorses | -0.06 |
| E | Vidyo1 | -0.26 |
|  | Vidyo3 | -0.85 |
|  | Vidyo4 | -0.32 |
| Average for all sequences |  | -0.41 |

# Conclusion

In this contribution, low-complexity approach for selecting the best filter among the pre-defined filter candidates was proposed. From experimental results, the coding performance of the proposed SIF scheme with the low-complexity filter decision was evaluated. The BD-rateY against TMuC0.9 reached 0.4% in the case of LoDe/LoCo while maintaining encoding/decoding time. Actually, increase of encoding time was 0.3% against TMuC LoDe/LoCo.

This contribution shows the low-complexity solution for SIF scheme.

# Patent rights declaration(s)

KDDI Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Tomonobu Yoshino et. al, "Enhanced Switching of Interpolation Filter for HEVC," JCTVC-C183, October 2010.

[2] Tomonobu Yoshino et. al, "Low-complexity Scheme for Adaptive Interpolation Filter Based on Amplitude Characteristic Analysis”, ICIP2010, Sep. 2010