| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D286 |
| --- | --- |

| Title: | Simplified multiplierless 4x4 DST for intra prediction residue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Li Zhang, Siwei Ma, Wen Gao Institute of Digital Media, Peking University Beijing, China | Tel: Email: | +86-010-62753424 xzhao@jdl.ac.cn zhanglili@jdl.ac.cn swma@jdl.ac.cn |
| Source: | Peking University |  |  |

_____________________________

# Abstract

In this contribution, a simplified 4x4 discrete sine transform (DST) scheme for intra prediction residue is shown. The proposed 4x4 DST is featured by simple integer transform matrix, multiplierless arithmetic and low dynamic range of internal results. Also a simplified mode-dependent coefficient scanning (SMDS) method is proposed together with the proposed DST.

# Introduction

Mode-dependent directional transform (MDDT) [1] is mainly featured by the mode-dependent design of transform and coefficient scanning and improves the intra coding performance. However, MDDT also introduces higher complexity which is mainly introduced by the following issues:

- Additional memory requirement for storing the 16 4x4 and 8x8 KLT matrices

- Additional memory requirement for storing the 18 4x4 and 8x8 coefficient scanning orders

- High dynamic range of internal results introduced by large transform matrix components

- No fast implementation for the proposed KLT

To achieve similar coding performance with lower complexity, Tool Experiment TE7 [2] was set up for a further investigation on MDDT simplification in the 2nd JCT-VC meeting. In the related contributions, a DST scheme is proposed by [3] and [4]. The basis of proposed N×N DST is composed of sinusoids with following formulation,

(1)

In both [3] and [4], the basis functions are approximated with large integers and multiplications are needed. To further simplify the implementation of 4x4 DST, we introduce a simple integer matrix composed of “3”, “5”, “7”, “8” and their negative values. The integer matrix is actually an approximation of DST, but featured by low dynamic range of internal results and multiplierless arithmetic.

# Algorithm description

For intra prediction residue, a separable transform scheme with following formulation is used.

F=D·X·DT							(2)

where X indicates the residual block of N×N size, D denotes the transform matrix. Two transform matrices are utilized in the proposed scheme, one is DCT defined in TMuC 0.9, and the other is a proposed integer matrix S. For 4×4 transform, S is designed as shown below

.					(3)

S4 is actually an approximation of 4×4 DST matrix proposed in [3], [4]. The basis of N×N DST is composed of sinusoids with formulation (1). Comparisons between the proposed integer basis and standard DST are shown in Figure 1. From Figure 1, it is shown that row vectors of S in (2) are good approximations of DST basis, but the components are small integers which are advantageous for low-complexity implementations.

Figure 1 Transform basis of continuous sine transform defined in (3) and the discrete version, and proposed transform

The calculation of proposed transform using S4 require only shifts and adds, and no multiplication is needed. For calculating the 1-D forward transform of a 4 point vector x, i.e., f=S4·x, a supposed low-complexity implementation is

a0=x0- x1；a1= x1+x3；a2=(x2<<3)- x2；a3=(x0+ x1)- x3

b0= a0<<2；b1= a1<<2

c0=b0+ a2

Finally, the transform coefficient vector f=[f0, f1, f2, f3]is obtained as

f0= c0- a0+(a1<<3)；f1=(a3<<3)-a3； f2=(a0<<3)-a2+b1+a1； f3=b1-a0-c0 -a1

With the above implementation, totally 15 adds and 6 shifts are needed for doing a 1-D 4-point forward transform.

For 1-D inverse transform of 4 point vector f, i.e., x=S4T·f, a similar supposed implementation is

a0=f0+ f2；a1= f2-f3；a2=(f1<<3)- f1；a3=f0- f2- f3

b0= a0<<2；b1= a1<<2

c0=b0+ a2

Finally, the transform coefficient vector x=[x0, x1, x2, x3] is obtained as

x0= c0- a0+ b1+a1；x1= a0 +c0- (a3<<3)； x2=(a3<<3)-a3； x3=(a0<<3)-a2-b1+a1

With the above implementation, totally 15 adds and 6 shifts are needed for doing a 1-D 4-point inverse transform. The utilization of DCT matrix and proposed transform matrix is specified by the intra mode as shown Table 1

Table 1 Transform for different Intra modes

|  | DC prediction | Others |
| --- | --- | --- |
| I4MB modes (0~8) | DCT (H.264/AVC) | S4 |

After the transform coefficients are quantized, a simplified version of mode-dependent coefficient scanning proposed in [1] is implemented to map the 2-D coefficient matrix into 1-D vector. Differences compared to the implementations in [1] include:

- Fixed coefficient scanning orders are used instead of updating one in [1] to reduce complexity.

- There are 2 coefficient scanning methods in the proposed scheme, instead of 9 in [1].

The coefficient scanning method is selected according to intra prediction direction, as specified in Table 2

Table 2 Prediction modes grouping in different Intra modes

|  | Logical prediction modes (0~33) |
| --- | --- |
| Scanning Method #0 | 5~13, 21~29 (transposed version of method #0) |
| Scanning Method #1 | 1~4, 14~20, 30~33 |

# Simulation results

## RD results

The rate-distortion performance compared to the anchor in terms of BD Rate [5] for Y, U and V components are shown in Table 3. As Table 3 shows, for “Intra High Efficiency” test condition, the proposed algorithm together with SMDS achieves in average 1.8%, 1.9% and 1.8% BD-Rate reduction for Y, U and V components, respectively. Also average 1.0%, 0.8% and 0.8% BD-Rate reduction for Y, U and V components is achieved when SMDS is disabled, respectively

Table 3 RD results for Intra configurations

| Sequence name | Intra High Efficiency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | with SMDS |  |  | no SMDS |  |  |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Class A avg | -2.5 | -2.7 | -2.5 | -1.7 | -1.6 | -1.7 |
| Class B avg | -1.2 | -1.6 | -1.5 | -0.7 | -0.7 | -0.7 |
| Class C avg | -1.7 | -1.7 | -1.6 | -0.8 | -0.5 | -0.5 |
| Class D avg | -1.8 | -1.6 | -1.7 | -1.0 | -0.7 | -0.6 |
| Class E avg | -2.7 | -2.5 | -2.3 | -1.1 | -1.1 | -1.0 |
| All average | -1.8 | -1.9 | -1.8 | -1.0 | -0.8 | -0.8 |

## Complexity

For complexity, the additional complexity in sense of arithmetic operation counts, memory requirements and bit-precision change introduced by the proposed method is shown in Table 4.

| Complexity Metric | 4×4 Transform |
| --- | --- |
| operation counts for transform a N-point 1-D vector | DC: no change Others: 15 adds, 6 shifts |
| memory requirements for transforms | no change |
| memory requirements for coefficient scanning | 2×4×16/8=16 byte (4 bits are needed for one scan position, 2 for x, 2 for y) |
| minimum bit-precision (9-bit input) | DC: no change Others: 19bits |

Table 4 Complexity measurement of proposed scheme compared with TMuC 0.9 (HM) anchor

# Conclusion

Based on the above simulation results, it is observed that the coding efficiency can be further promoted with applicable tradeoff with additional complexity. We recommend further study on low-complexity mode-dependent transform and scanning methods.

# References

[1]	Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2] Robert Cohen, Chuohao Yeo, Rajan Joshi, Wenpeng Ding, Siwei Ma, Akiyuki Tanizawa, Haitao Yang, Xin Zhao, Li Zhang, “Tool Experiment 7: MDDT Simplification”, Doc. JCTVC-B307_r4, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, 21-28 July, 2010.

[2] Chuohao Yeo, Yih Han Tan, Zhengguo Li, Susanto Rahardja, “TE7: Results for Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, Doc. JCTVC-C037, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[3] Ankur Saxena, Felix C. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, Doc. JCTVC-C108, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[4] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Peking University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).