| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D287 |
| --- | --- |

| Title: | Bi-Intra Prediction using slope information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C.W. Seo, J.K. Han (Sejong Univ.), J. Lim, J. Song (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: Tel: Email: | Tel: +85-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr +82-31-710-5196 jeogyeon@sktelecom.com jinhan.song@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This contribution presents bi-intra prediction scheme which is used with offset and reference smoothing.  Since the proposed technique considers the concept of slope and uses bi-prediction like bilinear interpolation, it increases the coding efficiency of HM. In this document, the experimental results under the common test condition defined by JCTVC-C500 [3] are reported.  For I slice only coding structure, the average BD-rate gains are 2.4% on low complexity condition and 0.6% on high efficiency condition. Based on the results presented in this contribution, we recommend that this tool will be studied and discussed in CE6.

# Introduction

In intra prediction of HM-0.9, pixels in current block are predicted from their neighboring left and top pixels by UIP (unified intra prediction)[1] where the number of directions for prediction is as in Fig. 1. The number of these prediction modes is larger than that of H.264/AVC.

In this contribution, we propose some advanced schemes, (1) using offset, (2) bi-intra prediction, (3) reference pixel smoothing, and (4) embedded flags. These tools decrease the residual data resulting from intra prediction. Thus, the coding efficiency is increased by them.

- Directions used in UIP

- The number of prediction directions  in UIP

Fig.  1. Available prediction directions in UIP

# Algorithm description

2.1. Bi-Intra Prediction (BIP)

The proposed BIP uses the concept of slope to increase the performance of UIP as in Fig. 2. For all direction modes used in UIP except for DC mode, we calculate the difference between two neighboring parts of the current block. One neighboring part is next to the reference line and other is next to the last horizontal (or vertical) line of the current block according to the prediction direction. In case of vertical prediction directions, the difference is calculated between two left neighboring parts of the last horizontal line and the corresponding reference line as in Fig. 2. For horizontal prediction directions, upper-extended parts of the last vertical line and the corresponding vertical reference line are used to calculate the difference. The lines to get the difference value are denoted by red blocks in Fig. 2. The averaged difference between them (red pixels) is offset which provides the information for slope. The slope is not encoded since the decoder can calculate it.

After offsets are calculated, the last line of the current block is predicted by adding the offset to the precition values provided by UIP. And then, the other lines of the current block are predicted by bi-linear interpolation with the neighboring pixels and the prediction values of the last line. Note that the size of the neighbor lines (red block) used to calculate offset is half of that of the current block.

Fig.  2. Example of BIP for a 4x4 block

2.2. Offset Clipping

When absolute value of the offset is too small, the scheme using the offset does not give any coding gain. Thus, the offest value is clipped as follows.

If( |offset| < threshold ), then offset = 0

The threshold depends on the number of the increased bit-depths of IBDI. In the following equation, B_inc denotes the number of the increased bit-depths.

threshold = 2 << B_inc

On the other hand, when the offset is too large, the value is clipped as follows.

Offset = Clip( -10<<B_inc, 10<<B_inc, offset)

2.3. Reference Pixel Smoothing

In bi-intra prediction, the coding efficiency can be degraded when the difference between the predicted last line and their adjacent reference is large. This case is occurred when the difference between B and C is large in Fig. 3. This coding loss can be prevented by smoothing them as follows.

B‘=(A + 2 * B + C)/4

C‘=(B + 2 * C + D)/4

Fig.  3. Reference pixel smoothing

2.4. Flag embedded in quantized coefficeints

Intra coding blocks are encoded by UIP or the proposed BIP. Mode and algorithm are selected based on RD optimization. In this contribution, flag to indicate the selected algorithm is not encoded to reduce the generated bit rate. Instead of encoding the flag, the flag is embedded in the quantized coefficeints. When BIP is used, the sum of absolute values of quantized coefficients are even number, i.e., 0, 2, 4, ..... On the other hand, when UIP is used, the sum of absolute values of quantized coefficients is odd, i.e., 1, 3, 5, .....

# Simulation results

As for anchor, HM-0.9 software is used in this simulation. Intra only, Random access, Low delay coding structures are tested. The experiments in this contribution are performed under the common test condition defined by JCTVC-C500 [3].

BD-rate (ΔBitrate [%]) [4] is measured by Excel sheet (TE12_results_template_r6.xls) provided on the JCTVC reflector.  The table 1 shows the experimental results of BIP for I slice only coding structure. The table 2 shows the experimental results of BIP for the random access coding structure. The BD-rate of BIP compared to the HM anchor is shown in Table 3 for low delay coding structure. These tables show that BIP provides coding gain with the reasonable increase of the encoding and decoding time.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.4 | -1.6 | -2.8 | -2.3 | -2.2 |
| Class B | -0.6 | -0.5 | -0.6 | -2.2 | -1.8 | -1.7 |
| Class C | -0.6 | -0.8 | -0.8 | -2.2 | -1.9 | -1.9 |
| Class D | -0.7 | -1.1 | -1.1 | -2.3 | -1.8 | -1.8 |
| Class E | -0.6 | -1.1 | -1.0 | -2.9 | -2.4 | -2.4 |
| All | -0.6 | -0.9 | -0.9 | -2.4 | -2.0 | -1.9 |
| Enc Time[%] | 150% |  |  | 155% |  |  |
| Dec Time[%] | 101% |  |  | 106% |  |  |

Table 1. Intra only case

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.7 | -0.8 | -1.0 | -0.8 | -0.8 |
| Class B | -0.3 | -0.4 | -0.6 | -0.8 | -0.9 | -0.8 |
| Class C | -0.4 | -0.6 | -0.7 | -0.8 | -0.7 | -0.7 |
| Class D | -0.3 | -0.5 | -0.8 | -0.8 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.5 | -0.7 | -0.8 | -0.8 | -0.8 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2. Random access case

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.3 | 0.0 | -0.2 | -0.4 | -0.2 |
| Class C | -0.1 | -0.3 | -0.3 | -0.2 | -0.4 | -0.2 |
| Class D | 0.0 | -0.5 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class E | 0.0 | 0.4 | 0.4 | 0.0 | -0.4 | 0.0 |
| All | -0.1 | -0.2 | 0.0 | -0.2 | -0.3 | -0.1 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3. Low delay case

# References

- JCT-VC, “WD1: Working Draft 1 of High-Efficiency Video Coding,” JCTVC-C403, 3rd JCT-VC Meeting, Guangzhou, CN, Jul 2010.

- JCT-VC, “Core Experiment 6: Intra Prediction Improvement,” JCTVC-C506, 3rd JCT-VC Meeting, Guangzhou, CN, Jul 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd JCT-VC Meeting, Guangzhou, CN, Jul 2010.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

# Patent rights declaration(s)

Sejong University and SK Telecom may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).