| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D295 WG11 Number: m19061 |
| --- | --- |

| Title: | CE1：Huawei report on TMDMVD and STDM in HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Report |  |  |
| Author(s) or Contact(s): | Mingyuan Yang Sixin Lin Haoping Yu Huawei Technologies Co., Ltd. Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86 755 28567191 myyang@huawei.com linsx@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This core experiment (CE) is to prove the performance of the two Decoder-side Motion Vector Derivation (DMVD) techniques, Template Matching based DMVD (TM-DMVD) and the Spatial and Temporal Direct Mode (STDM), which is a continuation of the work in [1][2]. Both techniques have been implemented in the TMuC v0.9 software and tested following the common test requirements (JCTVC-C300). This document describes these techniques in details, which includes descriptions of coding algorithms and their implementation, coding performance, and complexity evaluation and analysis. According to the test results, an average bit-saving of 2.28% has been achieved with very little additional decoding complexity, the decoding time is about 10% more than that of the anchor.

- Algorithm Description

## TM-DMVD

The motion vectors and the selection of the reference pictures are derived as described below.

- Template composition

The current template is created by the reconstructed pixels nearby the current block as shown in Fig. 1, where an L-shaped symmetric region around the upper-left corner of the target block is selected as the template whose maximum size is equal to that of the PU. However, when TM-DMVD is used in a Prediction Unit (PU), in order to make parsing and decoding process separated, the part from the available reconstructed region is used or the reference prediction instead of the unavailable reconstructed region from the neighboing block is used to construct the current template as illustrated in Fig. 2. For instance, in the first PU of a SIZE_NxN CU the current template is constructed by the reconstructed pixels. For the second PU only the reconstructed pixels in the above and the above left neighboring region is used to form the current template. However for the forth PU, since there’s no available reconstructed pixels, the reference prediction pixels pointed by the above or the left motion vectors are used.

Fig. 1 Template composition for PU with different size

Fig. 2 Template composition when a part of the template is not available

- Candidates based template matching

The motion information candidates similar with those of AMVP mode from the spatial and temporal neighboring are used, which are shown in Fig.3,e.g. The unique motion vectors from the above, left, above left, above right, median and collocated are used as the candidates.

Fig. 3 Composition of candidate motion vector set

c)  Reference template interpolated by bilinear filter

After the motion vector candidates set are selected, the reference template can be determined by one of these candidates. In order to reduce the complexity, the reference template is generated by interpolating the reference block pointed by a candidate with a bilinear filter(shown as Fig.4), which is much simpler than the filter used in motion compensation in TMuC software.

Fig. 4  Bilinear filter to interpolate a reference template

In Fig.4 half-pixels and quarter-pixels are interpolated using the integer pixels A, B, C and D.

d) Multiple reference pictures and multi-hypothesis prediction

Template matching is used based on the distortion cost function (Equation. 1) between the current template and the reference template. In order to reduce the complexity of cost calculation a downsampling techinique is used, which means a sample in every two samples in rows and in columns of the templates is used to calculate the SAD. Consequently, at most two motion vectors in the candidates set for reference pictures which produce the minimal distortion costs are used to predict the current coding block(Equation. 2). And then multi-hypothesis prediction is performed.

SAD=x,y=0,2,..2N|curTM(x,y)−refTM(x+u,y+v)|            (1)

where, (u,v) is the selected motion vector candidate, x, y is the coordinate of the sample in the template, which can set 0, 2, 4,..,2N.2N is the size of the PU.

pred(x,y)=(i=0,1ref(x+ui,y+vi)×(4>>i))6             (2)

Where, ref(x+ui,y+vi)means the reference block sample pointed by the candidate (ui, vi), whose SAD is the top i minimal distortion cost.

Finally, set the motion vector of TM-DMVD mode PU as the motion vector with minimal distortion cost.

## STDM

Here the algorithm process is depicted and the flowing chart is as below.

Fig. 5 Procedures of STDM

The procedure of STDM is shown in Fig. 5.

- Firstly two reference slices should be specified for the current encoding block. In B slice, these two reference slices are the nearest forward and backward reference slices in List0 and List1.

- Spatial-temporal neighboring motion vectors are collected in this step. As illustrated in Fig. 6, five spatial motion vectors are obtained from adjacent blocks of the current encoding PU. Another two spatial motion vectors are the median motion vectors of these neighboring motion vectors and zero motion vectors. The temporal motion vectors are obtained from the collocated motion vector in the forward reference slice. All these motion vectors are scaled based on their POC number.

Fig. 6 Spatial Neighboring Motion Vectors

- Motion vector candidate sets are setup based on the motion vectors collected in step 2. For each spatial neighboring PU if it is only forward prediction or only backward prediction, the motion vector for opposite direction is derived according to the following equation (3) based on the simple translation motion model. If neighboring PU is biprediction, a new backward motion vector is derived from the existing forward motion vector of this PU and a new forward motion vector is derived from the backward motion vector of this PU. The derivation uses the same equation (3).

(3)

represents the temporal distance between the current encoding slice and the reference slice from which MV is obtained.  represents the temporal distance between the current encoding slice and the opposite reference slice.

- After the candidate set of motion vector pairs is set up, each pair of motion vectors in this set will be checked. The one that minimize the following distortion cost function is selected as the final motion vectors pair of the current coding block.

(4)

In equation 4,  calculates the sum of absolute difference of two blocks  and ,  and  are the corresponding reference blocks in the forward and the backward reference slices, respectively.

In order to reduce the complexity further, STDM uses the rounded motion vector technique mentioned in [3] to avoid the complexity of reference block interpolation.

- Software Implementation

STDM is applied to inter modes (Skip/Direct, 2NxN, Nx2N and NxN mode) in B (NON-GPB) slices. So STDM mode only works for random access cases. Flag is needed to indicate whether STDM mode is used. For Skip/Direct and NxN mode only one bit is used to indicate whether STDM mode is used for all PUs. For other modes two bits are needed to indicate which PU uses the STDM modes.

TM-DMVD is applied to the inter modes (2Nx2N, 2NxN, Nx2N and NxN mode) in P slices or GBP slices. For 2Nx2N and NxN mode only one bit is used to indicate whether TM-DMVD mode is used for all PUs. For other modes two bits are needed to indicate which PU uses the TM-DMVD modes.

CU-level rate-distortion (RD) evaluation is performed to decide whether STDM and TM-DMVD are chosen.

- Syntax and Semantic Change

New syntax elements “STDM_Flag” and “DMVD_Flag” are added as follows in TMuC software. (The green part is modification of new syntax elements.)

Prediction unit syntax

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if(!slice_is_P_or_GPB) |  |  |
| STDM_flag | 2 | ue(1) ¦ ae(v) |
| if( mv_competition_flag && ! stdm_flag) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(!entropy_coding_mode_flag) |  |  |
| mode_table_idx |  | vlc(n,v) |
| else { |  |  |
| if( slice_type != I && PredMode!=MODE_SKIP ){ |  |  |
| merge_mode | 2 | u(1) ¦ ae(v) |
| if(PredMode==MODE_MERGE && uiNeighbourInfo==3) |  |  |
| Merge_idx | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if(slice_type != I && PredMode!=MODE_MERGE ) |  |  |
| pred_mode | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| planar_flag | 2 | u(1) ¦ ae(v) |
| if(planar_flag) { |  |  |
| planar_delta_y = getPlanarDelta() |  |  |
| planar_delta_uv_present_flag | 2 | u(1) ¦ ae(v) |
| if(planar_delta_uv_present_flag) { |  |  |
| planar_delta_u = getPlanarDelta() |  |  |
| planar_delta_v = getPlanarDelta() |  |  |
| } |  |  |
| } else { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| intra_split_flag | 2 | ae(v) |
| combined_intra_pred_flag | 2 | u(1) ¦ ae(v) |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) { |  |  |
| prev_intra_luma_pred_ flag | 2 | u(1) ¦ ae(v) |
| if( !prev_intra_pred_luma_flag ) |  |  |
| rem_intra_luma_pred_mode | 2 | ue(v) ¦ ae(v) |
| if( adaptive_intra_smoothing_flag ) |  |  |
| intra_luma_filt_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if(!slice_is_P_or_GPB&&(inter_partitioning_idc!=SIZE_NxN)) |  |  |
| STDM_flag[i] | 2 | ue(1) ¦ ae(v) |
| else if(!slice_is_P_or_GPB&& i==0) |  |  |
| STDM_flag[i] | 2 | ue(1) ¦ ae(v) |
| else if(!slice_is_P_or_GPB) |  |  |
| stdm_flag [i] = stdm_flag [0] |  |  |
| if(slice_is_P_or_GPB && (inter_partitioning_idc != SIZE_NxN)) |  |  |
| DMVD_flag[i] | 2 | ue(1) ¦ ae(v) |
| else if(slice_is_P_or_GPB&& i==0) |  |  |
| DMVD_flag [i] | 2 | ue(1) ¦ ae(v) |
| else if(slice_is_P_or_GPB) |  |  |
| dmvd_flag [i] = dmvd_flag [0] |  |  |
| if( slice_type == B && ((!stdm_flag[i]) ¦¦ (!dmvd_flag[i]))) |  |  |
| inter_pred_idc[ i ] | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ i ] != Pred_L1 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l0_active_minus1 > 0 && ((!stdm_flag[i]) ¦¦ (!dmvd_flag[i]))) |  |  |
| ref_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l0 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| ref_idx_mvres_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| mvd_l0[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| mvd_l0[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L0, i ) > 1 ) |  |  |
| mvp_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if( inter_pred_idc[ i ] != Pred_L0 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l1_active_minus1 >0 && ((!stdm_flag[i] ¦¦ (!dmvd_flag[i]) ) |  |  |
| ref_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l1 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| ref_idx_mvres_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| mvd_l1[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| if((!stdm_flag[i]) ¦¦ (!dmvd_flag[i])) |  |  |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L1, i ) > 1 ) |  |  |
| mvp_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if(!slice_is_P_or_GPB) |  |  |
| STDM_flag | 2 | ue(1) ¦ ae(v) |
| if( slice_type == B) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag && ! stdm_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

STDM_flag specifies the approach to derive the motion information in Skip/Direct and Inter modes.

- If stdm_flag is equal to 1, STDM derivation method is used.

If inter_partition_idc is SIZE_NxN, STDM derivation method is applied in all predition units of this Coding Unit.

- Otherwise TMuC original derivation method is used.

If inter_partition_idc is SIZE_NxN, TMuC original derivation method is applied in all predition units of this Coding Unit

DMVD_flag specifies the approach to derive the motion information in Inter modes.

- If DMVD_flag is equal to 1, TM-DMVD derivation method is used.

If inter_partition_idc is SIZE_NxN, TM-DMVD derivation method is applied in all predition units of this Coding Unit.

- Otherwise TMuC original derivation method is used.

If inter_partition_idc is SIZE_NxN, TMuC original derivation method is applied in all predition units of this Coding Unit

Decoding Process Conditions

slice_is_P_or_GPB specifies whether current slice is GPB or P slice

- If slice_is_P_or_GPB is true, current slice is P slice or GPB slice,  and the POC numbers of reference slices in List 1 is smaller than the POC number of the current slice

- Otherwise current slice is B(non-GPB) slice and the POC numbers of reference slices in List 1 is bigger than the POC number of the current slice.

stdm_flag specifies whether current prediction unit is coded by STDM

- If stdm_flag is true, current prediction unit is coded by STDM

- Otherwise current prediction unit is not coded by STDM

slice_is_P_or_GPB specifies whether current slice is GPB or P slice

- If slice_is_P_or_GPB is true, current slice is P slice or B slice and the POC numbers of reference slices in List 1 is smaller than the POC number of the current slice

- Otherwise current slice is B slice and the POC numbers of reference slices in List 1 is bigger than the POC number of the current slice.

dmvd_flag specifies whether current prediction unit is coded by TM-DMVD

- If dmvd_flag is true, current prediction unit iscoded by TM-DMVD

- Otherwise current prediction unit is not coded by TM-DMVD

- Compression Performance and Complexity analysis

TM-DMVD works for GPB slices which means TM-DMVD works in low delay and random access cases. STDM works for B (non-GPB) slices which means STDM works only in random access cases. In order to provide the performance and complexity in detail, in random access cases performance and complexity of TM-DMVD plus STDM and STDM only will be provided separately.

In the tables below, all of the data are measured on the same computer: MS windows Server 2003R2, Enterprise x64 Edition, SP2. Inter® Xeon® CPU E5650 @2.67GHz, 24 GB of RAM. And the source code is written in C++ language based on TMuC 0.9, without any optimization by assemble language, such as MMX or SSE2 instruction, etc. The exe encoder and decoder are complied with 32 bits visual C++ compiler.

The simulation complies with the common test conditions defined in JCTVC-C500. Detailed test conditions are available in attached configure files.

The accompanied spreadsheets show the detailed test results.

## Compression Performance Discussion

In the tables below, average BDRATE saving is used. BD-rate Y represents the bit rate saving in luma component. BD-rate U and BD-rate V represent the bit rate saving in each chroma component. The detailed data is provided in the associated excel data sheet.

### Test Results for Random Access High Efficiency Configuration Relative toCE1 Anchor

In table 1, TMDMVD and STDM are enabled. STDM works in B (non-GPB) slices, and TMDMVD works in GPB slices.

| Video Sequence | TMDMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -3.16 | -3.06 | -2.89 |
| Class B | -1.98 | -1.60 | -1.79 |
| Class C | -2.01 | -2.01 | -2.23 |
| Class D | -2.49 | -2.17 | -2.43 |
| Total average | -2.28 | -2.06 | -2.23 |

Table 1.	TMDMVD+STDM RD Performance in random access high efficiency Configuration

From Table 1 the total gain for proposed techniques is about 2.28% bitrate saving.

In table 2, STDM is enabled B (non-GPB) slices.

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.91 | -2.85 | -2.89 |
| Class B | -1.72 | -1.48 | -1.54 |
| Class C | -1.79 | -1.98 | -2.00 |
| Class D | -2.13 | -1.95 | -2.16 |
| Total average | -2.00 | -1.92 | -2.01 |

Table 2.	STDM RD Performance in random access high efficiency Configuration

From Table 2 the total gain for proposed techniques is about 2% bitrate saving.

### Test Results for Random Access Low Complexity Configuration Relative to CE1 Anchor

In table 3, TM-DMVD and STDM are enabled. STDM works in B (non-GPB) slices, and TMDMVD works in GPB slices.

| Video Sequence | TMDMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.24 | -2.17 | -2.23 |
| Class B | -1.59 | -1.49 | -1.67 |
| Class C | -1.49 | -1.64 | -1.86 |
| Class D | -1.96 | -1.97 | -1.91 |
| Total average | -1.75 | -1.75 | -1.86 |

Table 3.	TM-DMVD+STDM RD Performance in random access low complexity Configuration

From Table 3 the total gain for proposed techniques is about 1.75% bitrate saving.

In table 4, STDM is enabled B (non-GPB) slices.

| Video Sequence | STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class A | -2.16 | -2.11 | -2.08 |
| Class B | -1.47 | -1.24 | -1.37 |
| Class C | -1.36 | -1.40 | -1.59 |
| Class D | -1.86 | -1.78 | -1.75 |
| Total average | -1.64 | -1.54 | -1.63 |

Table 4.	STDM RD Performance in random access low complexity Configuration

From Table 4 the total gain for proposed techniques is about 1.64% bitrate saving.

### Test results for Low Delay High Efficiency Configuration Relative to CE1 Anchor

In table 5, TM-DMVD is enabled GPB slices, no matter STDM is enable or disable.

| Video Sequence | TM-DMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | -0.88 | -0.98 | -1.48 |
| Class C | -1.03 | -1.13 | -1.34 |
| Class D | -0.92 | -1.55 | -1.13 |
| Class E | -1.66 | -3.35 | -3.45 |
| Total average | 1.08 | -1.6 | -1.7 |

Table 5.	TMDMVD RD Performance in low delay high efficiency Configuration

Compared with CE1 anchor, the average gain for proposed techniques is about 1.08% bitrate saving.

### Test results for Low Delay Low Complexity Configuration Relative to CE1 Anchor

In table 6, TM-DMVD is enabled GPB slices, no matter STDM on or off.

| Video Sequence | TM-DMVD+STDM |  |  |
| --- | --- | --- | --- |
|  | BD-rate Y(%) | BD-rate U(%) | BD-rate V(%) |
| Class B | -0.48 | -0.35 | -0.32 |
| Class C | -0.28 | -0.69 | -0.71 |
| Class D | -0.23 | -0.45 | -0.36 |
| Class E | -0.82 | -0.33 | -0.52 |
| Total average | -0.43 | -0.46 | -0.47 |

Table 6.	TM-DMVD RD Performance in low delay low complexity Configuration

Compared with CE1 anchor, the average gain for proposed techniques is about 0.43% bitrate saving.

## Complexity Analysis

In the tables below, the data means the percentage of encoding and decoding time compared with CE1 anchor. The data in the tables are the average time for each sequence at all rate points. Not only the decoding time with the writing of yuv files is provide, but also the decoding time without the writing of yuv files is provided as the supplementary information. You can see the details in the associated excel data sheets.

### Random Access High Efficiency Configuration Relative to CE1 Anchor

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 194% | 111.20% | 111.55% |

Table 7.	TM-DMVD+STDM Complexity Analysis in random access high efficiency Configuration

From Table 7, it can be seen that the decoding time of TM-DMVD+STDM is about 11% increasing over  that of CE1 anchor. The encoding time is about 94% increasing over that of CE1 anchor.

| Video Sequence | STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 170.78% | 105.69% | 106.00% |

Table 8.	STDM Complexity Analysis in random access high efficiency Configuration

From Table 8, it can be seen that the decoding time of STDM is about 6% increasing over  that of anchor. The encoding time is about 70% increasing over that of CE1 anchor.

### Random Access Low Complexity Configuration Relative to CE1 Anchor

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 207.90% | 123.12% | 125.49% |

Table 9.	TM-DMVD+STDM Complexity Analysis in random access low complexity Configuration

From Table 9, it can be seen that the decoding time of TM-DMVD+STDM is about 25% increasing over that of CE1 anchor. The encoding time is about 107% increasing over that of CE1 anchor.

| Video Sequence | STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 176.38% | 116.99% | 119.47% |

Table 10.	STDM Complexity Analysis in random access low complexity Configuration

From Table 10, it can be seen that the decoding time of STDM is about 19% increasing over that of CE1 anchor. The encoding time is about % increasing over that of CE1 anchor.

### Low Delay High Efficiency Configuration Relative to CE1 Anchor

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 178.22% | 113.96% | 114.022% |

Table 11.	TM-DMVD Complexity Analysis in low delay high efficiency Configuration

From Table 11, it can be seen that the decoding time of TM-DMVD is about 14% increasing over that of CE1 anchor. The encoding time is about 78% increasing over that of CE1 anchor.

### Low Delay Low Complexity Configuration Relative to CE1 Anchor

| Video Sequence | TM-DMVD+STDM (%) |  |  |
| --- | --- | --- | --- |
|  | Encoding Time (%) | Decoding Time(%) Writing YUV files | Decoding Time(%) Not writing YUV files |
| Average | 180.38% | 124.15% | 126.07% |

Table 12.	TM-DMVD Complexity Analysis in low delay low complexity Configuration

From Table 12, it can be seen that the decoding time of TM-DMVD is about 26% more than that of CE1 anchor. The encoding time is about 80% increasing over that of CE1 anchor.

Complexity of TM-DMVD and STDM in encoder side can be further decreased. Currently each TM-DMVD and STDM mode are independent in TMuC. When the partition size is 2NxN or Nx2N, some of PU is doing motion estimation in TM-DMVD or STDM mode. The same motion vectors achieved from conventional 2NxN and Nx2N in same PU could be used again and motion estimation in this PU in TM-DMVD and STDM mode could be saved.

According to the complexity of TM-DMVD and STDM in decoder side, all the calculations of cost function are independently and could be processed in parallel. Also this kind of SAD calculation is very easy to be speeded up by using MMX or SSE optimization.

- Conclusion

In this proposal we present the brief description of TM-DMVD and STDM together with the analysis of coding performance and complexity. From the simulation results, we can see that the coding performance of anchor can be much improved by the proposed techniques with a limited additional complexity in decoder.

We suggest JCT adopt the decoder side motion vector derivation techniques into the HM model.

- Reference

[1] Sixin Lin, Mingyuan Yang, Jiantong Zhou, Jin Song, Dong Wang, Haitao Yang, Jiali Fu, Haoping Yu, Wang Yue, Li Zhang, Siwei Ma, Wen Gao, “TE1: Huawei report on DMVD improvements”, JCTVC-B037, 2nd Meeting: Geneva, CH, 21-28 July, 2010;

[2] Mingyuan Yang, Sixin Lin, Dong Wang, Jiantong zhou, Haoping Yu, “TE1：Huawei report on DMVD improvements in TMuC”, JCTVC-C097, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010;

[3] Yi-Jen Chiu, Lidong Xu, Wenhao Zhang, Hong Jiang, “TE1：Report of self derivation of motion estimation improvement in TMuC”, JCTVC-C127, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010;

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Huawei Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).