| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D296 WG11 Number: m19062 |
| --- | --- |

| Title: | Unbiased clipping for IBDI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Kenta Senzaki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

In this contribution, it is proposed that the current biased clipping for IBDI, i.e., in the range of 0 to 2(8+bit_depth_minus8+bit_depth_increment) - 2bit_depth_ increment, is replaced with unbiased full-range clipping, in the range of 0 to 2(8+bit_depth _minus8+bit_depth _increment) – 1. The revised scaling for leveraging the extended range is also presented. Experimental results have shown that the performance difference is negligible. It is recommended that the proposal is discussed jointly with other IBDI-related issues.

# Introduction

In TMuC 0.9 and also in HM 1.0, the reconstructed pixel values are clipped in the range of 0 to 2(8+bit_depth_minus8+bit_depth_increment) - 2bit_depth_ increment at the default setting (#define IBDI_NOCLIP_RANGE 1 in CommonDef.h). It is described in the source code that it is aimed at avoiding the clipping when scaling reconstructed pixel values for image output. However, how to make output images is an implementation issue and should be beyond the scope of the normative part of the standard. Employing a biased clipping in the normative part considering informative image output should be avoided. It is therefore proposed in this contribution that the current biased clipping for IBDI is replaced with unbiased full-range clipping, i.e., in the range of 0 to 2(8+bit_depth _minus8+bit_depth _increment) - 1.

In order to leverage this wider range, revising the scaling method for IBDI both in encoder and decoder is also useful. The current HM implementation is that:

PixelToBeCoded = InputPixel << bit_depth_increment

PixelToBeOutput = (ReconstructedPixel + (1<<(bit_depth_increment-1)) >> bit_depth_increment.

Another scaling, which is likely to be more suitable for the revised clipping, is that:

PixelToBeCoded = (InputPixel << bit_depth_increment) + (1<<(bit_depth_increment-1))

PixelToBeOutput = ReconstructedPixel >> bit_depth_increment.

Although the scaling issue itself seems to be beyond the normative part of the standard as described above, it should be noted that it may affect other IBDI-related issues such as reference frame memory compression by fixed rounding [1], PCM coding for IBDI [2], and so on, since they need rounding and recovering bits extended by IBDI.

# Simulation results

The combination of the proposed full-range clipping and the revised scaling has been implemented on TMuC 0.9 and tested according to common test conditions defined in JCTVC-C500 [3]. Since low complexity configurations are not related with the proposal, they were skipped and following three configurations were used in this experiment.

- Intra, high efficiency

- Random access, high efficiency

- Low delay, high efficiency

Our computing platform used for encodings is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Core i7 930 2.80GHz, 4 physical cores

- Memory: 12GiB

As for measurement of decoding times, the following computing platform was used.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

The encoder and decoder executables were generated with Microsoft Visual C++ 2008 SP1. We ran up to 12 encodings at the same time. Note that individual encoding time can be significantly varied since Xeon and Core i7 CPUs have automated over-clocking functionality.

The simulation results are shown below. It is observed that differences in coding efficiency and complexity by the proposal are negligible.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.2 | 0.1 |
| Class E | 0.0 | 0.4 | 0.1 |
| All | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 |
| Class B | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | -0.3 | 0.0 |
| Class C | 0.0 | 0.2 | 0.0 |
| Class D | 0.0 | -0.2 | 0.0 |
| Class E | -0.2 | 0.2 | -0.7 |
| All | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusions

In this contribution, it has been proposed that the current biased clipping for IBDI, i.e., in the range of 0 to 2(8+bit_depth_minus8+bit_depth_increment) - 2bit_depth_ increment, is replaced with unbiased full-range clipping, in the range of 0 to 2(8+bit_depth _minus8+bit_depth _increment) – 1. The revised scaling for leveraging the extended range is also presented. Experimental results have shown that the performance difference is negligible. It is recommended that the proposal is discussed jointly with other IBDI-related issues.

# References

[1] K. Chono, et al., “Rounding-error conscious memory compression method for IBDI,” JCT-VC document JCTVC-D045, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, January 2011.

[2] K. Chono, et al., “Pulse code modulation mode for HEVC,” JCT-VC document JCTVC-D044, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, January 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).