| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D297 WG11 Number: m19063 |
| --- | --- |

| Title: | CE2: Huawei & HiSilicon report on Flexible Motion Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, HiSilicon Haoping Yu, Huawei Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: Email: | +86-0755-28422009 zhengxiaozhen@huawei.com haopingyu@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd., Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports Huawei & HiSilicon's implementation status of flexible motion partitioning on the TMuC platform. At the current stage, the horizontal & vertical asymmetric partitioning and non-rectangular partitioning have been implemented in the TMuC0.9. Besides, signaling method, the motion vector prediction mechanism for non-rectangular partition has also been modified and improved.

Using the CE2 test condition, the new partitioning method has achieved about 3.2% and 3.7% bitrate saving on average for random access and low-delay cases with the 'high-efficiency' configuration, achieved 3.0% and 3.4% bitrate saving on average for random access and low-delay case at 'fast' version, and achieve about 1.6% and 1.7% bitrate saving on average for random access and low-delay case at 'diagonal-only' version.

# Introduction

In natural images, object’s boundary or contour usually divides a block into 2 regions with arbitrary shape. Conventional square-structured representation of motion partitioning can not deal with this situation efficiently. To solve the problem, the concept of flexible motion partitioning has been introduced in the past few years [1 ~ 6]. It has show satisfied coding gain for inter-frame coding, especially in the region that contains different object’s content.

To further investigate flexible motion partitioning technique, Core Experiment 2 was setup in the third JCTVC meeting in Guangzhou, October 2010 [7]. In the following sections, the implementation status on TMuC platform, signaling method, motion vector prediction mechanism for non-rectangular partition and coding performance will be discussed.

# Coding structure and signaling method

Following HM's design, the partitioning type 'PART_2Nx2N', 'PART_2NxN' and 'PART_Nx2N' are used for signaling the non-rectangular partitioning, asymmetric horizontal partitioning and asymmetric vertical partitioning respectively. A set of signaling parameters are coded followed with the partitioning type.

For the cases of asymmetric horizontal & vertical partitioning coding, 12 asymmetric partitioning modes are added to HM. The 'PART_2NxN' partitioning and other 6 asymmetric horizontal partitioning modes share the same partitioning type (inter_partitioning_idc == '1'), and the 'PART_Nx2N' and other 6 asymmetric vertical partitioning modes share the same partitioning type (inter_partitioning_idc == '2') [6].

For non-rectangular partitioning, a 2Nx2N CU can be split to two PU with arbitrary shape. A signaling flag is used to identify the usage of 2Nx2N square CU coding or non-rectangular CU coding [6]. As illustrated at Fig. 1, non-rectangular partitioning can be identified by the position of point A and B. The parameters ‘scalar’, ‘pos’ and directional flag are used to represent the positions.

‘scalar’ is used to represent the ration between ‘BO’ and ‘AO’. ‘pos’ is used to indicate the point A’s position. The direction of the boundary between two partitions is signaled by the direction flag. In the current design, the value of ‘scalar’ can be 1:1, 1:2, 2:1, 1:4 and 4:1, which represent 45 degree, 26.6 degree, 14 degree, 63.4 degree and 76 degree partitioning respectively.

Figure 1: Illustration of non-rectangular partitioning

# Method vector prediction for flexible motion partitioning

To improve the motion vector coding efficiency, AMVP mechanism is modified that is used to save coding bits of motion vector. The indices of AMVP candidates are reordered based on the shape of non-rectangular partitions.

# Summary of Results

To exactly evaluate the encoding and decoding complexity of the proposed method, both of anchor data and proposal data are run on the same computing platform. Three version data are proposed with different encoding complexity and coding efficiency.

For the ‘fast’ version, the number of scalars is same as ‘high-efficiency’ version whereas the encoding speed-up algorithm is added. For the ‘diagonal-only’ version, only 4 asymmetric horizontal & vertical partition modes, and 6 45 degree partition modes are added.

High-efficiency version – Reference: TMuC0.9

|  | Random access, high efficiency |  |  | Lowdelay, high efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4.3 | -5.0 | -4.6 |  |  |  |
| Class B | -2.4 | -3.1 | -2.9 | -2.5 | -3.8 | -4.0 |
| Class C | -3.8 | -4.7 | -4.9 | -3.3 | -4.3 | -4.6 |
| Class D | -3.2 | -3.6 | -3.2 | -3.3 | -5.2 | -4.6 |
| Class E |  |  |  | -6.7 | -7.6 | -7.4 |
| All | -3.2 | -3.9 | -3.7 | -3.7 | -5.0 | -5.0 |
| Enc Time[%] | 343% |  |  | 303% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |

Fast version – Reference: TMuC0.9

|  | Random access, high efficiency |  |  | Lowdelay, high efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.9 | -4.7 | -4.2 |  |  |  |
| Class B | -2.1 | -2.8 | -2.9 | -2.3 | -3.4 | -3.8 |
| Class C | -3.6 | -4.5 | -4.7 | -3.2 | -4.1 | -4.4 |
| Class D | -3.2 | -3.7 | -3.7 | -3.3 | -5.1 | -4.4 |
| Class E |  |  |  | -6.0 | -6.7 | -6.6 |
| All | -3.0 | -3.7 | -3.8 | -3.4 | -4.6 | -4.7 |
| Enc Time[%] | 237% |  |  | 214% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

Diagonal-only version – Reference: TMuC0.9

|  | Random access, high efficiency |  |  | Lowdelay, high efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.9 | -2.5 | -2.1 |  |  |  |
| Class B | -1.1 | -1.4 | -1.5 | -1.1 | -1.7 | -2.0 |
| Class C | -1.8 | -2.3 | -2.5 | -1.6 | -2.2 | -2.3 |
| Class D | -1.8 | -2.2 | -2.1 | -1.8 | -3.1 | -2.4 |
| Class E |  |  |  | -2.7 | -3.2 | -2.8 |
| All | -1.6 | -2.0 | -2.1 | -1.7 | -2.4 | -2.3 |
| Enc Time[%] | 161% |  |  | 152% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusion

According the test results, the ‘diagonal-only’ partitioning method can improve enhance the performance of the existing motion partitioning in HM whereas the complexity is low. It recommends that:

- Create the unified solution and further investigate the performance of non-rectangular partitions and asymmetric horizontal & vertical partitions with more flexible partitioning mechanism in the core experiment

# References

- Satoru Adachi, Shun-ichi Sekiguchi, Sadaatsu Kato, Mitsuru Kobayashi and Minoru Etoh, “Refined Results on the Low-overhead Prediction Modes”, document VCEG-O22, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

- Ò. Divorra, P. Yin and C. Gomila "Geometry-adaptive Block Partitioning", ITU-T Q.6/SG16 VCEG, VCEG-AF10, April 2007

- K. Vermeirsch, J. De Cock, S. Notebaert, and R. Van de Walle, “New macroblock bipartitioning modes for inter coding,” ITU-T Q.6/SG16 VCEG, VCEG-AH25, January 2008.

- Ken McCann, Woo-Jin Han, Il-Koo Kim, "Samsung’s Response to the Call for Proposals on Video Compression Technology" JCTVC Contribution JCTVC-A124, Dresden, Germany, April 2010.

- P.Chen, L.Guo, M.Karczewicz , (Qualcomm), “Geometry motion partition”, Doc. JCTVC-C301 (m18516), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, October 2010.

- X. Zheng, H. Yu, “TE3.2: Huawei & HiSilicon report on flexible motion partitioning coding”, JCTVC Contribution JCTVC- C099, Guangzhou, China, Octorber 2010.

- E. Francois, P. Chen, X. Zheng, “Description of Core Experiment 2: Flexible Motion Partitioning,” Doc. JCTVC-C099, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Octorber 2010

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).