| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4rd Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D298 WG11 Number: m19064 |
| --- | --- |

| Title: | CE2: Cross-verification of AMP from Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, HiSilicon Haoping Yu, Huawei Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: Email: | +86-0755-28422009 zhengxiaozhen@huawei.com haopingyu@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd., Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document summarizes the CE2 activities related to the cross-verification of fast asymmetric motion partitioning (AMP) from Samsung[1].

# Verification

Samsung has provided the source code of fast asymmetric motion partitioning (AMP) and the test results corresponding to TMuC 0.9 anchor. Both anchor and Samsung's proposed algorithm are run on the same computing platform. The following platform and compiler has been used at the verification:

OS: Windows Server 2003 64 bits

Compiler: VS2005

CPU: 2xE5520, 4 cores

RAM: 4G per core

# Cross-Verification results

TMuC 0.9 software and Samsung's software are used to generate anchor and comparison data. The following table summarize the testing result. Compared to Samsung's result, all of data are matched.

|  | Random access, high efficiency |  |  | Lowdelay, high efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.8 | -0.6 |  |  |  |
| Class B | -0.6 | -0.7 | -0.6 | -0.6 | -0.9 | -0.9 |
| Class C | -0.7 | -0.8 | -0.9 | -0.7 | -1.0 | -1.1 |
| Class D | -0.7 | -0.6 | -0. | -0.8 | -1.7 | -1.0 |
| Class E |  |  |  | -1.5 | -1.9 | -2.3 |
| All | -0.7 | -0.7 | -0.7 | -0.9 | -1.3 | -1.2 |
| Enc Time[%] | 111% |  |  | 113% |  |  |
| Dec Time[%] | 107& |  |  | 100% |  |  |

# Conclusion

The fast AMP algorithm proposed by Samsung can achieve 0.7% and 0.9% bitrate saving for random access and lowdelay case with about 10% encoding time increase. The decoding time has slight increase compared to TMuC 0.9 anchor.

# References