| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D299 WG11 Number: m19605 |
| --- | --- |

| Title: | CE6.b Report on Short Distance Intra Prediction Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Yunfei Wang, Tsinghua Yongbing Lin, HiSilicon Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei Feng Wu, Microsoft | Email: | laichangcai@huawei.com jzxu@microsoft.com haopingyu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This document reports the experimental results of the short distance intra prediction (SDIP) scheme for core Experiment 6 on intra prediction improvement. SDIP was presented in JCTVC-C101 and JCTVC-C270. By dividing the NxN block into lines or non-square blocks, SDIP can reduce the energy of the prediction residuals by reducing the distance of predicted pixel and its reference pixels. When integrated into the TMuC 0.9 (HM) software, it shows 4.3% and 5.7% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively, with about 50% encoding time increase and no obviously decoding time increase. Up to 8.9% bit rate saving is achieved on sequences with rich textures.

# Introduction

The traditional block-based intra coding in AVC/KTA/HM uses one NxN square block as the reconstruction unit and prediction unit. The pixels inside a square block are all predicted from the boundaries of neighboring reconstructed blocks, producing poor predictions for pixels on the right-bottom part than the others in some regions of sequences. To better exploit spatial correlations, the short distance intra prediction coding scheme is proposed by partitioning one NxN square block into several lines or non-square blocks with rectangle shape. In the block, pixels are predicted and reconstructed line by line or rectangle by rectangle. Therefore, the prediction distance can be obviously shortened.

# Algorithm description

In SDIP, one NxN square block which is smaller than 32x32 is divided into several lines or non-square blocks with rectangle shape. In the block, pixels are predicted and reconstructed line by line or rectangle by rectangle.

## Block Partitions

In SDIP modes, one CU that smaller than 32x32 can be partitioned as lines or non-square blocks with rectangle shape as Fig.1 shows. One 16x16 CU can not only be divided into four 8x8 PU as did in HM, but also be divided into four 4x16/16x4 PU, and a 4x16/16x4 PU can be further split into four 1x16/16x1 partitions. As similar, one 8x8 CU can also be divided into four 2x8/8x2 PU, and every 4x4 PU can be further divided into four 1x4/4x1 partitions.

8x8

1x4

16x16

4x16

1x16

2x8

116形6

Fig. 1 SDIP block partitions based on HM

## SDIP intra prediction and coding

Fig.2 UDI prediction for 2x8

In the Intra prediction for SDIP partitions, UDI of HM is used as Fig.2 and the UDI modes can be from 9 to 34 and the number of modes is configurable as HM. One UDI mode flag is identified for every 4x16/16x4 or 2x8/8x2 partition. If a 4x16/16x4 is divided into four 1x16/16x1, the 1x16/16x1 partitions used the same modes that identified in 4x16/16x4 and similar for 1x4/4x1 partitions in 4x4 PU.

SDIP is not directly used for chroma intra prediction currently, however, all four luma prediction modes in the current CU and the four modes 0-3 are tested in mode decision, among which the four luma modes are mapped to low indices and modes 0-3 are mapped to high ones. Only the first five modes by the new indices are searched in mode decision. The new index is then transmitted in the bitstream instead of the original modes. This approach is also applied to blocks not selecting SDIP mode, where SIZE_NxN partitions might benefit from it.

Similar transforms and quantization as HM were used in SDIP modes but only the transform size is partition size related and the scale matrix is modified respectively. At the entropy coding stage, different scanning orders and contexts are designed for different partitions. Generally, for 2x8, 1x16 and 4x16 partitions, the coefficients are first scanned from high frequency to low frequency into a 1D buffer and then reorganized into a 4x4 or 8x8 block and coded as it does as illustrated in Fig. 3. For 1x16 partitions, since the transform is one dimensional, there is no need to scan from high to low. The coefficients are directly reorganized into 4x4 and coded. For 4x4 using 1x4 transforms, the coefficients of a 4x4 block are coded as a whole. They are scanned from four DC coefficients to other AC coefficients. The significant map is coded using the frequency as a context, i.e. there are four context models for 4x4. The coefficient levels are coded similar to 4x4 in HM. The following figure illustrates the scanning for each case. In SDIP modes, RDOQ is used in CABAC case, in LCEC case, RDOQ was also implemented but turned off considering the tradeoff between the gain and encoding complexity.

Fig. 3 Scanning order for 16x4 partition

# Test conditions and Results

The proposal has been integrated into HM0.9. We have run two sets of results with different SDIP intra prediction modes configurations. Both of two sets of results were tested in Intra and Intra LoCo cases under the recommended test conditions of intra-only configuration for CE6 in [1]. The following summarizes the simulation results.

Results_1: Normal configurations

In this set of configurations, only a simple SDIP early skip method was used to reduce the encoding complexity. The following platform and compiler have been used and results were shown in Table1.

| Platform | Compiler |
| --- | --- |

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.7 | -1.9 | -1.9 | -4.9 | -3.6 | -3.9 |
| Class B | -3.2 | -1.5 | -1.7 | -4.9 | -2.6 | -3.5 |
| Class C | -5.5 | -2.9 | -3.1 | -7.4 | -4.0 | -4.8 |
| Class D | -5.6 | -3.0 | -3.0 | -7.1 | -4.1 | -4.5 |
| Class E | -4.8 | -3.2 | -3.0 | -5.8 | -7.2 | -6.3 |
| All | -4.6 | -2.5 | -2.5 | -6.1 | -4.1 | -4.5 |
| Enc Time[%] | 200% |  |  | 182% |  |  |
| Dec Time[%] | 104% |  |  | 112% |  |  |

The test results show that 4.6% and 6.1% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively. Up to 9.3% bit rate saving is achieved on sequences with rich textures.

Results_2: faster configurations

The purpose of Set2 is to further reduce the encoding complexity while maintaining the coding efficiency. In this set of configurations, in addition to the SDIP early skip method, some other fast intra prediction modes search methods were used as following:

The following platform and compiler have been used and Table2 shows the results of faster configurations.

| Platform | Compiler |
| --- | --- |

Table 2. Resutls of faster setting

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.5 | -1.9 | -1.9 | -4.5 | -3.3 | -3.4 |
| Class B | -3.0 | -1.4 | -1.7 | -4.4 | -2.5 | -3.2 |
| Class C | -5.2 | -2.8 | -3.1 | -7.0 | -3.8 | -4.4 |
| Class D | -5.4 | -2.9 | -2.9 | -6.8 | -3.9 | -4.2 |
| Class E | -4.5 | -3.0 | -2.7 | -5.3 | -6.3 | -5.6 |
| All | -4.3 | -2.4 | -2.5 | -5.7 | -3.8 | -4.1 |
| Enc Time[%] | 161% |  |  | 146% |  |  |
| Dec Time[%] | 100% |  |  | 105% |  |  |

The test results show that 4.3% and 5.7% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively. Up to 8.9% bit rate saving is achieved on sequences with rich textures.

# Conclusion

This document reports the experimental results of SDIP scheme for core Experiment 6 on intra prediction improvement. When integrated into the TMuC 0.9 software, it shows 4.3% and 5.7% bit rate saving on average, under all intra high efficiency and low complexity conditions, respectively, with about 50% encoding time increase and no obviously decoding time increase. Up to 8.9% bit rate saving is achieved on sequences with rich edges.

# References

A. Tabatabai, “Core Experiment 6: Intra Prediction Improvement”, Docx. JCTVC-C506_r2, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, China, 07-15 October, 2010.

# Patent rights declaration(s)

Tsinghua University, University of Science and Technology of China, Microsoft, Huawei and HiSilicon may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).