| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D303r2 WG11 Number: m19069 |
| --- | --- |

| Title: | Consideration on AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato GCO W12F 1-11-1 Osaki Shinagawa Tokyo 141-0032 Japan | Tel: Email: | +81-3-5435-3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Current HM adopts AMVP as motion vector coding. In AMVP scheme candidates of a motion vector predictor are Median, MV_a, MV_b, MV_c and ColMV.

However the horizontal and the vertical component of the Median PMV is either MV_a, MV_b or MV_c. The one advantage of the Median PMV is that it will provide more degree of freedom: for example, the horizontal component can be MV_a whereas the vertical component can be MV_b, but it may be redundant. In addition, having more candidates will cause increase in code_nunber bits and loss in coding efficiency, and Median PMV requires sorting that causes increase in complexity.

In this document we will show that omitting Median PMV as a candidate of AMVP is not harmful in terms of coding efficiency.

- Introduction

Other than temporal direct mode, AVC employs Median Prediction for PMV generation. Median Prediction requires sorting and causes increase in coding efficiency.

In current HM, AMVP is adopted as Motion Vector Coding. In AMVP scheme, not only Median PMV but also MV_a, MV_b, MV_c and ColMV are defined as a candidate of PMV. The horizontal and the vertical component of the Median PMV is either MV_a, MV_b or MV_c so having Median PMV as a candidate may be redundant in AMVP.

# Simulation

## Simulation Condition

Simulation condition is based on Random_Access, RandomAccess_Loco, LowDelay, and LowDelay_Loco, except that MRG is set as 0.

This is because I would like to test with the condition that AMVP is the only tool for MV Coding.

In the data shown in the next section anchor is MedianPMV-off and to be tested is MedianPMV-on.

## Simulation Result

Summary of simulation results are shown in Table 3.2.1 and Table 3.2.2. Complete results are shown in “D303.xls”. The verification result is reported by [3] and [4].

As can be seen difference in coding efficiency is very small between MedianPMV-off and MedianPMV-on.

Table 2.2.1

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class D | -0.1 | 0.1 | -0.1 | -0.1 | -0.1 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 | -0.1 |

Table 2.2.2

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.2 | 0.1 | 0.2 | 0.3 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | 0.2 | 0.1 |
| Class E | -0.2 | -1.1 | 0.0 | -0.2 | -0.2 | -0.3 |
| All | 0.0 | -0.1 | 0.1 | 0.0 | 0.1 | 0.1 |

# Conclusion

The simulation results show that in AMVP scheme omitting Median PMV does not affect coding efficiency, as the result of Median is either MV_a, MV_b or MV_c, that are included in the candidate of PMV in AMVP scheme. In addition, Median Prediction causes increase in complexity.

Removal of Median PMV from AMVP needs to be considered by the Core Experiment on Motion Vector Coding.

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, October 2010

- “Common test conditions and software reference configurations”, JCTVC-C500, October 2010

- “Cross-check report of Sony’s AMVP modification (JCTVC-D303)”, JCTVD-C417, January 2011

- “Cross-check report of Sony's proposal JCTVC-D303 by Panasonic”, JCTVC-D428, January, 2011