| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D305r1 WG11 Number: m19071 |
| --- | --- |

| Title: | CE13: Cross-Check Result of 3.2d |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato GCO W12F 1-11-1 Osaki Shinagawa Tokyo 141-0032 Japan E. Maani 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +81-3-5435-3308 Kazushi.Sato@jp.sony.com +1-408- 352-4710 ehsan.maani@am.sony.com |
| Source: | Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check result of the experiment 3.2d in CE13 and in addition cross-check result of mode-dependent intra smoothing and mode-dependent hybrid intra smoothing, which is also part of CE13.

- Description

At the 3rd JCTVC mtg, fast intra smoothing has been proposed [4] and it is part of CE13: Intra Smoothing [3].

Software based on HM1.0 has been provided to CE participants by the proponent of [4], and experiment for cross-checking of 3.2d in the document [3] has been conducted by Sony.

In addition, we have also conducted cross-check of mode-dependent intra smoothing and mode-dependent hybrid intra smoothing proposed by [6] with Intra and Intra_Loco conditions.

# Simulation

## Fast Intrta Smoothing

2.1.1 Simulation Condition

Basic setting is same as the one described in [2] with Intra, Intra_Loco, RandomAccess, RandomAccess_Loco, LowDelay, and LowDelay_Loco.

HM1.0 is used as anchor and the software distributed to CE13 participants is use as to be tested with the following macro setting:

#define HHI_AIS_ANGULAR_FIX	1

#define	HHI_AIS_SPEEDUP	1

2.1.2 Simulation Result

Summary of the result is shown in Table 2.1.2.1, 2.1.2.2 and 2.1.2.3. Complete result is shown in “JCTVC-D305_1.xls”. The result matches with the one provided by the proponent.

As can be seen gain is observed with larger images like Class A, B, or E in Intra_Loco setting even if fast mode [4] is employed.

Table 2.1.2.1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.6 | -0.8 | -1.7 | -0.3 | -0.6 |
| Class B | -0.5 | -0.5 | -0.6 | -0.7 | 0.0 | -0.2 |
| Class C | -0.1 | 0.1 | -0.1 | 0.1 | 0.7 | 0.6 |
| Class D | -0.1 | 0.1 | 0.1 | 0.1 | 0.6 | 0.6 |
| Class E | -0.4 | -0.4 | -0.4 | -0.7 | -0.2 | 0.3 |
| All | -0.3 | -0.2 | -0.3 | -0.5 | 0.2 | 0.2 |

Table 2.1.2.2

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.5 | -0.5 | -0.1 | -0.1 | -0.3 |
| Class B | -0.2 | -0.6 | -0.9 | 0.0 | -0.2 | -0.3 |
| Class C | -0.1 | -0.2 | -0.3 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | -0.1 | 0.2 | 0.1 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.4 | -0.4 | 0.1 | 0.0 | 0.0 |

Table 2.1.2.3

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | -0.3 | 0.1 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.2 | -0.1 | 0.1 |
| Class D | 0.0 | -0.4 | -0.1 | 0.1 | 0.3 | 0.1 |
| Class E | 0.1 | -0.1 | 0.1 | 0.3 | -0.2 | -0.1 |
| All | 0.0 | -0.1 | -0.2 | 0.1 | 0.0 | 0.0 |

## Mode-dependent itnra smoothing and mode-dependent hybrid intra smoothing

2.2.1 Simulation Condition

Basic setting is same as the one described in [2] with Intra, Intra_Loco.

HM1.0 is used as anchor and the software distributed to CE13 participants is use as to be tested.

Both mode-dependent intra smoothing and mode-dependent hybrid intra smoothing have been tested.

2.2.2 Simulation Result

Summary of result with mode-dependent intra smoothing and mode-dependent hybrid intra smoothing are shown in Table 2.2.2.1 and 2.2.2.2 respectively. Complete result is shown in “JCTVC-D305_2.xls” and in “JCTVC-D305_3.xls”. The result matches with the one provided by the proponent.

Table 2.2.2.1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.7 | -0.8 | -1.5 | -0.8 | -0.9 |
| Class B | -0.4 | -0.3 | -0.3 | -0.9 | -0.3 | -0.3 |
| Class C | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | -0.2 | -0.2 | -0.3 | -0.4 | -0.2 | -0.2 |
| Class E | -0.3 | -0.5 | -0.6 | -1.2 | -0.4 | -0.4 |
| All | -0.3 | -0.3 | -0.4 | -0.7 | -0.3 | -0.3 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

Table 2.2.2.2

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.9 | -1.0 | -1.7 | -0.9 | -1.0 |
| Class B | -0.7 | -0.7 | -0.7 | -1.0 | -0.5 | -0.6 |
| Class C | -0.2 | -0.2 | -0.3 | -0.2 | -0.1 | -0.2 |
| Class D | -0.2 | -0.4 | -0.3 | -0.4 | -0.2 | -0.2 |
| Class E | -0.7 | -0.8 | -0.8 | -1.4 | -0.6 | -0.5 |
| All | -0.5 | -0.5 | -0.6 | -0.9 | -0.4 | -0.4 |
| Enc Time[%] | 104% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Conclusion

Experimental result of 3.2d in CE13 has been verified. Gain is observed with larger images like Class A, B, or E in Intra_Loco setting even if fast mode is employed.

Additionally we have tested mode-dependent intra smoothing and mode-dependent hybrid intra smoothing with Intra and Intra_Loco conditions. This is also part of CE13.

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, October 2010

- “Common test conditions and software reference configurations”, JCTVC-C500, October 2010

- “Core Experiment 13: Intra Smoothing”, JCTVC-C513, October 2010

- “Improved intra smoothing for UDI and new AIS fast mode”, JCTVC-C302, October 2010

- “Improvement of Adaptive Intra Smoothing by switching interpolation filters”, JCTVC-D391, January 2011

- “Simplified Intra Smoothing”, JCTVC-C234, Guangzhou, October, 2010.