| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D307_r2 |
| --- | --- |

| Title: | CE7: Cross-checking of MDDT results from Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Jin Song, Haitao Yang, Haoping Yu, Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District,Beijing, China. | Tel: Email: | +86 10 82882583 andy.songjin@huawei.com haitaoyang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by Huawei on Qualcomm’s MDDT proposal. The simulation results match the results provided by Qualcomm.

# Introduction

Basically, the MDDT is implemented as combinations of two transforms. The two transforms are
1. A transform (seems like the trained KLT) and the DCT matrices with 7-bit precision for 4x4 and 8x8 transforms

2. The DST matrix and the DST matrices with 8-bit precision for 16x16 transforms

For the fixed scanning order, the row-wise horizontal scan and the column-wise vertical scan as well as the zigzag scan are used.  The horizontal scan and the vertical scan are fixed. However, the zigzag scan is adaptive, which was proposed by HHI and is actually the default behavior of TMuC0.9.

# Simulation Results

The test conditions as specified in CE7 [1] are followed. Qualcomm’s implementation appears to be based on TMuC 0.9. Tests were conducted using the intra high efficiency and random access high efficiency coding configurations [2].

## RD results

Table 1 shows the RD results when all the frames are coded as intra, for all the test sequences specified in the common coding conditions [2].

Table 1. RD results for intra high efficiency configuration.

|  | results from Qualcomm |  |  | results from Huawei |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.00 | -3.55 | -3.61 | -3.00 | -3.55 | -3.61 |
| Class B | -1.57 | -2.27 | -2.31 | -1.57 | -2.27 | -2.30 |
| Class C | -2.33 | -2.29 | -2.26 | -2.33 | -2.29 | -2.26 |
| Class D | -2.27 | -2.13 | -2.12 | -2.27 | -2.13 | -2.12 |
| Class E | -3.10 | -3.59 | -3.36 | -3.10 | -3.59 | -3.36 |
| All | -2.31 | -2.61 | -2.57 | -2.31 | -2.61 | -2.57 |
| Enc Time[%] |  |  |  | 105% |  |  |
| Dec Time[%] |  |  |  | 106% |  |  |

Table 2 shows the RD results when the random access high efficiency configuration is used [2].

Table 2. RD results for random access high efficiency configuration

|  | results from Qualcomm |  |  | results from Huawei |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.26 | -0.78 | -0.62 | -1.26 | -0.78 | -0.62 |
| Class B | -0.85 | -0.74 | -0.84 | -0.85 | -0.74 | -0.84 |
| Class C | -1.08 | -0.86 | -0.99 | -1.08 | -0.86 | -1.00 |
| Class D | -0.94 | -0.73 | -0.38 | -0.94 | -0.73 | -0.38 |
| Class E |  |  |  |  |  |  |
| All | -0.99 | -0.77 | -0.73 | -0.99 | -0.77 | -0.73 |
| Enc Time[%] |  |  |  | 105% |  |  |
| Dec Time[%] |  |  |  | 100% |  |  |

More detailed coding results can be found in the Excel attachment.

In cross-checking, the results we obtained match those provided by Qualcomm. The small difference in bit rate can be considered due to different operating systems.

# Conclusion

We have cross-checked Qualcomm’s MDDT proposal, studied the algorithm and verified their coding simulation results.

# References

[1]R. Cohen et al., “Description of Core Experiment 7: Alternative Transforms,” JCTVC-C507, Guangzhou, October 2010.

[2]F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.