| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D311 WG11 Number: m19077 |
| --- | --- |

| Title: | Adaptive coefficients scanning for inter-frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Song, Mingyuan Yang, Haitao Yang, Jiantong Zhou, Dong Wang, Sixin Lin, Haoping Yu Jiali Fu, Jie Liu Huawei Technologies Co., Ltd. Huawei Building, 3 Xinxi Road, Haidian District,Beijing, China. | Tel: Email: | +86 10 82882583 andy.songjin@huawei.com yangmingyuan@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes an adaptive scanning method for transform coefficients in inter-slices. The best scanning mode for every Transform Unit (TU) is chosen based on the texture direction of reference block and no flag is sent to the decoder side. The proposed technique was implemented on top of TMuC 0.9 and the comparison tests were done with the existing methods in TMuC 0.9 version. The performance of this new scanning order method is evaluated based on the common test conditions specified in JCTVC-C500 [1]. Proposed method provides 1.1% and 0.4% improvements in high efficiency low-delay and random access configurations, respectively.

# Motivation

As illustrated in Fig. 1, it is observed that structured textures still exist in the residual images after motion compensation. And it is well known that the texture of reference block is very similar with that of the current coding block. So the texture of reference block can represent the texture of the residual block.

Fig. 1 the coding image and its residual image

On the other hand, as illustrated in Fig. 2, it is also observed that there is a relationship between residual block’s texture and transform coefficient distribution. When the residual block’s texture contains vertical edges, the non-zero transformed coefficients will fall along the horizontal direction. The reverse is also true.

(a) Horizontal texture                                       (b) Vertical texture

Fig. 2 Relationship of residual texture and transform coefficients distribution

# Adaptive Coefficient Scanning (ACS) method based on reference block

In this proposal, an adaptive coefficient scanning (ACS) method is introduced. Currently, this ACS method consists of three scanning patterns: new zigzag, horizontal and vertical patterns (Fig. 3).

(a) New zigzag scan                   (b) Horizontal scan                    (c) Vertical scan

Fig. 3 ACS patterns for 4x4 transform unit

First, the texture directions of the reference block need to be detected (Fig. 4). The sum of the absolute value of the difference between two adjacent pixels in horizontal direction is calculated, which is called as horizontal gradient. The sum of the absolute value of the difference between two adjacent pixels in vertical direction is calculated, which is called as vertical gradient. A threshold is used here for the choice of scanning pattern. When the ratio of horizontal gradient and vertical gradient is bigger than the threshold, the texture direction is vertical. When the ratio of vertical gradient and horizontal gradient is bigger than the threshold, the texture direction is horizontal. In other case, the texture direction is diagonal. The threshold is set to be 1.5 for WQVGA and WVGA and 2 for other bigger resolution in our current implementation.

(a) calculating horizontal gradient      (b) calculating vertical gradient

Fig. 4 calculating directional gradient

Then the best scanning pattern is chosen based on the texture direction. The relationship between the texture direction and scanning pattern is given in Table 1.

Table 1: Relationship between the texture direction and scanning pattern

| texture direction | Scanning pattern |
| --- | --- |
| horizontal | vertical |
| vertical | horizontal |
| diagonal | new zigzag |

There is no need to transmit any flag to indicate which scanning pattern is used, because the same process above is carried out in both encoder and decoder.

# Experimental results of ACS and complexity analysis

TMuC 0.9 with default configuration was used to generate the anchor data, which is the same as the anchor of CE11. Following table presents the total average BD-rate saving for each testing configuration. ACS has improvements comparing with anchor by 0.4% and 1.1% in high-efficiency random-access and low delay configurations, respectively.

Table 2. Average performance in low delay configuration

|  | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | BD-PSNR | BD-rate | BD-PSNR | BD-rate | BD-PSNR |
| Class B | -1.2 | 0.03 | -0.2 | 0.00 | -0.1 | 0.00 |
| Class C | -1.2 | 0.05 | 0.0 | 0.00 | 0.1 | 0.00 |
| Class D | -1.2 | 0.06 | -0.5 | 0.01 | 0.0 | 0.00 |
| Class E | -0.7 | 0.02 | -0.2 | 0.01 | 0.1 | 0.00 |
| Average | -1.1 | 0.04 | -0.2 | 0.00 | 0.0 | 0.00 |

Table 3. Average performance in random access configuration

|  | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | BD-PSNR | BD-rate | BD-PSNR | BD-rate | BD-PSNR |
| Class A | -0.3 | 0.01 | 0.1 | 0.00 | 0.1 | 0.00 |
| Class B | -0.4 | 0.01 | -0.1 | 0.00 | -0.1 | 0.00 |
| Class C | -0.4 | 0.02 | -0.1 | 0.00 | -0.1 | 0.00 |
| Class D | -0.4 | 0.02 | -0.1 | 0.00 | 0.0 | 0.00 |
| Average | -0.4 | 0.01 | -0.1 | 0.00 | 0.0 | 0.00 |

In the next table the encoding and decoding running time relative to the anchor are summarised for the same configurations mentioned above.

Table 4. Relative to the anchor encoding and decoding running time

| Time | Enc | Dec |
| --- | --- | --- |
| HE Random access | 102% | 102% |
| HE Low delay | 102% | 103% |

Also, the complexity of ACS in the encoder side can be further reduced. When texture direction of large block size in the reference block is detected, directional information of small block size in the same reference block can be achieved without further calculating the gradients.

# Conclusions

In this contribution, a novel adaptive coefficient scanning method is proposed and its simulation results are provided. From the results, it was observed that the alternative scanning method can provide better performance than the TMuC while the additional complexity in the encoder or decoder side is very small. We suggest the propose ACS can be further evaluated in CE11.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC Meeting, Guangzhou, October 2010

- JCT-VC, “Test Model under Consideration”, 2nd JCT-VC Meeting Document JCTVC-B205, July 2010.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).