| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D313 WG11 Number: m19079 |
| --- | --- |

| Title: | CE13: Intra Smoothing Test Report by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Result Report |  |  |
| Author(s) or Contact(s): | B. Bross D. Marpe H. Schwarz T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of core experiment 13 evaluating intra smoothing tools [1]. Two of the four tested tools perform intra reference sample smoothing based on a mode and block size dependent lookup table. The other two combine the lookup table with signaling. For the best performing tool, encoding times from 101% (intra low complexity) to 97% (low delay low complexity) of the anchor times, decoding times around 98% of the anchor times with bitrate savings from 0.87% (intra low complexity) to -0.04% (low delay low complexity) are reported.

# Tested Proposals

## Mode Dependent Intra Smoothing

In JCTVC-C234 [2], a lookup table (LUT) based intra smoothing technique was proposed. Depending on the prediction mode (intra direction) and block size, the reference samples for intra prediction are either 1-pass filtered or not.

## Mode Dependent Hybrid Intra Smoothing

Additional to the LUT-table smoothing approach in 1.1, the filtering of intra reference samples is signaled for all intra prediction modes except intra DC for blocks sizes greater than 8x8.

## LUT-based Intra Smoothing

In JCTVC-C302 [3], a similar LUT-based intra smoothing technique was proposed. Depending on the prediction mode (intra direction) and block size, the reference samples for intra prediction are either 2-pass filtered or not.

## Fast Adaptive LUT-based Intra Smoothing

In this adaptive approach it is signaled whether the LUT from 1.3 is used or inverted. In this case, inverted means if the table entry is “filter” it is changed to “not filter” and vice versa. For intra DC modes nothing is signaled. For all other modes, the proposed fast estimation is threshold based. Instead of testing only the best mode, several candidates are tested. For this, the RD cost range (rd_range) between best (rd_best) and worst candidate is calculated and only candidates with RD costs lower than 3/8 * rd_range + rd_best are tested.

# Simulation Environment and Results

## Software and Settings

The software used to integrate the intra smoothing tools is HEVC TMuC 0.9 [4] with default HM1.0 tool/macro configuration [5]. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [6]. These default settings, both for configuration file and compile-time, are used in this test as anchor. Changes from this anchor configuration are listed in the next sections.

### JCTVC-C234 Software

The proposal in sections 1.1 and 1.2 has been integrated by Qualcomm and sent to the testers. On top of TMuC 0.9 the following macros have been added:

QC_MFIS		Set to 1 enables Intra Smoothing mode dependent, no signaling).

QC_MDHIS	Set to 1 enables hybrid intra smoothing (signaled)

The following configurations have been tested:

| Tool | Changes relative to TMuC 0.9 default: |  |
| --- | --- | --- |
|  | Cfg Parameters | Macros |
| Mode Dependent Intra Smoothing | AIS : 1 | #define QC_MFIS 1 #define QC_MDHIS 0 |
| Mode Dependent Hybrid Intra Smoothing | AIS : 1 | #define QC_MFIS 1 #define QC_MDHIS 1 |

### JCTVC-C302 Software

The proposal in sections 1.3 and 1.4 has been integrated by Fraunhofer HHI and sent to the testers. On top of TMuC 0.9 the following macros have been added:

HHI_AIS_SPEEDUP		Set to 1 enables the speedup described in C302.

HHI_AIS_ANGULAR_FIX 	Set to 1 enables the new smoothing LUT and the corresponding context derivation described in C302.

The following configurations have been tested:

| Tool | Changes relative to TMuC 0.9 default: |  |
| --- | --- | --- |
|  | Cfg Parameters | Macros |
| LUT Intra Smoothing | - | #define DEFAULT_IS 1 #define HHI_AIS_ANGULAR_FIX 1 |
| Fast Adaptive LUT Intra Smoothing | AIS : 1 | #define HHI_AIS_ANGULAR_FIX 1 #define HHI_AIS_SPEEDUP 1 |

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in Table 2-1.

Table 2-1 Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 4x6 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 10.10, 64 bit |
| 2 | 2x6 | Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB | Ubuntu 10.10, 64 bit |
| 3 | 2x4 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 10.10, 64 bit |

## Results

The coding efficiency results in this section are given in terms of BD-rate [7]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-D313-MDIS.xls, JCTVC-D313-MDHIS.xls, JCTVC-D313-C302IS.xls, JCTVC-D313-C302AIS.xls.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented in two tables: Table 2-2, and Table 2-3. Each table represents a test set for intra, random access, and low delay coding constraint set using the high efficiency and low complexity (LoCo) settings.

Table 2-2	Average BD-rates and relative encoding/decoding runtimes for intra, random access, low delay constraint set for high efficiency and low complexity (LoCo) profiles.

Table 2-3	Average BD-rates and relative encoding/decoding runtimes for intra, random access, low delay constraint set for high efficiency and low complexity (LoCo) profiles.

# Conclusion

With average bitrate savings between 0.48% and 0.85% for intra only and no significant differences in encoding/decoding time, we conclude that the mode dependent hybrid intra smoothing (MDHIS) is the preferred intra smoothing tool.

# References

- Muhammed Coban, Benjamin Bross, Jianle Chen, “Core Experiment 13: Intra Smoothing”, JCTVC-C513, Guangzhou, October, 2010.

- Yunfei Zheng, Muhammed Coban, Marta Karczewicz, “Simplified Intra Smoothing”, JCTVC-C234, Guangzhou, October, 2010.

- Tung Nguyen, Benjamin Bross, Detlev Marpe, Thomas Wiegand, “Improved intra smoothing for UDI and new AIS fast mode”, JCTVC-C302, Guangzhou, October, 2010.

- JCT-VC, “Test Model under Consideration Software 0.9”, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9

- T. K. Tan, G. J. Sullivan, J.-R. Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Guangzhou, October, 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).