| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D314 WG11 Number: m19080 |
| --- | --- |

| Title: | CE9: Motion Vector Coding Test Report by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Result Report |  |  |
| Author(s) or Contact(s): | B. Bross S. Oudin D. Marpe H. Schwarz T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of core experiment 9 on motion vector coding evaluating merge and skip tools [1]. For the best performing tool combination, encoding times from 101% (intra low complexity) to 97% (low delay low complexity) of the anchor times, decoding times around 98% of the anchor times with bitrate savings from 0.5% (low delay high efficiency) to 1.75% (random access low complexity) are reported.

# Block Merging

Additional to the constrained block merging for 2Nx2N inter partitions as specified in HM 1.0, a generalized block merging process, which is applied to all inter-coded partitions [2], has been integrated into the TMuC 0.9 software for this core experiment. For clarification, both concepts are briefly described in the next sections.

Figure 1-1	Possible Coding Unit Partitioning for Prediction Units

## Constrained Block Merging for 2Nx2N Partition (HM 1.0)

For the constrained block merging as specified in HM 1.0, the level on which the block merging is applied is the coding tree leaf, i.e. the Coding Unit. The corresponding Prediction Unit partition is 2Nx2N (see Figure 1-1).

Signalization

The merging mode is signaled by the syntax elements, merge_flag and merge_index. In the following, the signalization is described for the merging using the PU blocks left (L) or above (T) the current 2Nx2N PU (X) as candidates in the block merging list:

Figure 1-2	Block Merging for 2Nx2N partitions

As illustrated in Figure 1-2, if both blocks L and T are not inter-coded, the current block X cannot be merged and merge_flag is not transmitted, but inferred to be equal to 0. If either or both of the blocks L and T are inter-coded, the syntax element merge_flag is coded. If merge_flag is equal to 0, the current block X is not merged with any of the two neighboring blocks and motion parameters for the current block X are signaled explicitly. Otherwise, the current block X is merged with either L or T and the motion parameters are copied from the corresponding block. If blocks L and T have identical prediction parameters, no distinction needs to be made since merging with any of the two blocks will lead to the same result. In this case, the syntax element merge_index is not transmitted. Otherwise, if blocks L and T have different prediction parameters, merge_index is coded. If merge:index is equal to 1, the current block X is merged with the left neighbor L; and if merge_index equal to 0, the current block X is merged with the top neighbor T.

The signalization of block merging is done directly after the skip flag and before the prediction mode signalization, as illustrated in the following:

1.	Split flag

2.	Skip flag

3.	Merge flag

4.	Merge Index

5.	Prediction Mode Signalization

6.	Partition Size Signalization

Estimation

The estimation for block merging is done like the estimation for other prediction modes (intra- or inter-prediction). It means that it is based on the reconstruction of the estimated 2Nx2N coding block and the exact calculation of the rate distortion costs with the Lagrangian functional. It should be noted that in contrast to other inter prediction modes, no motion estimation process is required.

## Generalized Block Merging (all partition sizes)

For the generalized block merging, we removed the restriction that merging information is only transmitted for 2Nx2N partitions. The level on which the generalized block merging is applied is a Prediction Unit. That is why all the Prediction Unit partitions of an inter-predicted Coding Unit partitioning presented in Figure 1-1 can be merged with one candidate of the block merging list. It means that for each inter-predicted Coding Unit, each Prediction Units can be either predicted with the “normal” inter-prediction (motion search and motion vector prediction) or it can be merged with one candidate of the block merging list. The merging for each Prediction Unit of a Coding Unit is basically independent of the merging of other Prediction Units inside the same Coding Unit. However, there are some restrictions for the block merging list in order to avoid redundancies between the Coding Unit partitioning and the virtual partitioning resulting from block merging.

Signalization

The signalization of block merging is done directly after the partition size signalization. The granularity of the block merging signalization is the one of the prediction unit.

- Prediction Mode Signalization

- Partition Size Signalization

- For each prediction unit

  - Merge flag

  - Merge Index

  - Spatial prediction side information

    - Inter directional flag

    - Reference Frame Index

    - Motion vector difference

    - Motion Vector prediction Index

The exceptions for which merge flag and merge index are not signaled are the same as described in 1.1.

Estimation

The estimation is done in two different ways depending of the Coding Unit partitioning. If the partitioning is 2Nx2N, the estimation is based on the reconstruction of the current coding block and the exact calculation of the rate distortion (RD) costs with the Lagrangian functional.

If the Coding Unit partitioning is not 2Nx2N, the estimation is not using the complete reconstruction for each Prediction Unit. For every Prediction Unit, the RD costs are estimated with the Lagrangian functional based on the sum of absolute differences between the original block and the predicted block and a bitrate estimation. The predicted block is the result of a motion compensation based on the inter-prediction information taken from one of the candidates in the block merging list. The resulting RD costs for each candidate in the block merging list have to be compared with the RD costs calculated for the “normal” inter-prediction resulting from motion estimation. When this is done for all Prediction Units of one Coding Unit partitioning, the exact RD Costs calculated based on the reconstruction of this Coding Unit.

## Difference to Skip/Direct Modes

The main difference to the Skip/Direct Modes is that block merging copies all inter prediction parameters from the chosen candidate block. If the candidate block uses one motion vector, this motion vector and the corresponding reference index is copied. If the candidate block uses two motion vectors, the two motion vectors with reference indices are copied.

# Tested Proposals

The following proposals have been integrated into TMuC 0.9. All are applicable for the constrained block merging with 2Nx2N partition (or CU merging) as well as for the generalized partition merging (or PU merging).

## Combined Merge CBF Signaling

An additional syntax element mrg_cbf was added to the merging syntax for 2Nx2N inter blocks. A mrg_cbf syntax element equal to 1 signals that the current block is merged (merge_flag equal to 1) and that no residual data is coded for the current block (cbf equal to 0 for luma and chroma). There is no need to code the merge_flag or the cbf flags because the merge_flag can be inferred to be 1 and the cbf flags can be inferred to be 0. A mrg_cbf syntax element equal to 0 signalizes that the merge_flag and the cbf flags have to be coded after mrg_cbf.

## Modified Merge Candidates

The list of candidate blocks for merging has been modified. The default configuration uses the motion information of the blocks left and above of the top left corner sample of the current block. In the modified version, the collocated block and two corner blocks – the block above right and the block below left – are added to the list of candidate blocks. In this version, the range of values for the merge_index has been increased correspondingly. The tested candidate sets are listed in Table 2-1.

## Modified AMVP Candidates for Skip/Direct

The set of AMVP candidates used in skip and direct modes has been modified too. The tested sets are listed in Table 2-2.

Table 2-1	Candidate blocks for merging where corner includes the blocks above right and below left.

|  | left | above | corner | collocated |
| --- | --- | --- | --- | --- |
| merge set 0 (default) | 1 | 1 | 0 | 0 |
| merge set 1 | 1 | 1 | 1 | 0 |
| merge set 2 | 1 | 1 | 1 | 1 |

Table 2-2	Candidate motion vectors used in skip in direct modes where corner signifies the motion vector above right and if above right is not available, below left is taken.

|  | (0,0) | left | above | median | corner | collocated |
| --- | --- | --- | --- | --- | --- | --- |
| AMVP set 0 (default) | 1 | 1 | 1 | 1 | 1 | 1 |
| AMVP set 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| AMVP set 2 | 0 | 0 | 0 | 0 | 1 | 1 |
| AMVP set 3 | 0 | 0 | 0 | 0 | 0 | 1 |

## Disabling Direct Mode

In this core experiment, the possibility of disabling the direct mode has been integrated and tested. The direct mode is not signaled when it is disabled

# Tested Combinations

## 2Nx2N Partition Merging

Table 3-3	Tested combinations of proposed tools for 2Nx2N partition merging.

|  | JCTVC-C509 Test | AMVP | Skip | Direct | Merge | mrg_cbf |
| --- | --- | --- | --- | --- | --- | --- |
| a | 3.1.k | n.a. | 0 | 0 | Set 2 | 1 |
| b | 3.1.l | n.a. | 0 | 0 | Set 2 | 0 |
| c | 3.1.q | Set 2 | 1 | 0 | Set 0 | n.a. |
| d | 3.1.m | Set 0 | 1 | 0 | Set 0 | n.a. |
| e | 3.1.j | Set 3 | 1 | 1 | Set 1 | 1 |
| f | 3.1.i | Set 2 | 1 | 1 | Set 0 | 1 |

## All Partition Merging

Table 3-4	Tested combinations of proposed tools for all partition merging.

|  | JCTVC-C509 Test | AMVP | Skip | Direct | Merge | mrg_cbf |
| --- | --- | --- | --- | --- | --- | --- |
| a | 3.1.e | n.a. | 0 | 0 | Set 2 | 1 |
| b | 3.1.f | n.a. | 0 | 0 | Set 2 | 0 |
| c | 3.1.r | Set 2 | 1 | 0 | Set 0 | 1 |
| d | 3.1.d | Set 3 | 1 | 1 | Set 1 | 1 |
| e | 3.1.c | Set 2 | 1 | 1 | Set 0 | 1 |
| f | 3.1.b | Set 1 | 1 | 1 | Set 0 | 1 |

# Simulation Environment and Results

## Software and Settings

The software used to integrate the merge tools is HEVC TMuC 0.9 [3] with default HM1.0 tool/macro configuration [4]. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [5]. These default settings, both for configuration file and compile-time, are used in all tests as anchor. The configuration of the TMuC 0.9 software modified by Fraunhofer HHI and NTT DOCOMO for all tests listed in Table 3-3 and Table 3-4 can be found in [1].

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in Table 4-5.

Table 4-5	Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 4x6 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 10.10, 64 bit |
| 2 | 2x6 | Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB | Ubuntu 10.10, 64 bit |
| 3 | 2x4 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 10.10, 64 bit |

## Results

The coding efficiency results in this section are given in terms of BD-rate [6]. The values are averaged over all sequences. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-D314-3.1.X.xls, with X in {b,c,d,e,f,i,j,k,l,m,q,r}.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented for 2Nx2N partition merging in Figure 4-3, Figure 4-4, Figure 4-5 and for all partition merging in Figure 4-6, Figure 4-7, Figure 4-8. Each figure represents a test set for random access (RA) and low delay (LD) coding constraint set using the high efficiency (HE) and low complexity (LC) settings.

Figure 4-3	Average BD rates for 2Nx2N partition merging tests.

Figure 4-4 	Encoding time in percent of the anchor encoding time for 2Nx2N partition merging tests.

Figure 4-5 	Decoding time in percent of the anchor decoding time for 2Nx2N partition merging tests.

Figure 4-6	Average BD rates for all partition merging tests.

Figure 4-7	Encoding time in percent of the anchor encoding time for all partition merging tests.

Figure 4-8 	Decoding time in percent of the anchor decoding time for all partition merging tests.

# Conclusion

With average bitrate savings between 1.75% and 0.5% and between 93% and 75% of the reference encoding time, we conclude that the generalized block merging using left, above, above right, below left and collocated blocks with combined merge_cbf signaling and no skip/direct modes is the preferred merging configuration (3.2 case a).

# References

- J. Jung, B. Bross, “Core Experiment 9: Motion Vector Coding”, JCTVC-C509, Guangzhou, October, 2010.

- B. Bross, P. Helle, S. Oudin, H. Schwarz, D. Marpe, T. Wiegand, “TE12.1: Results for PU-based merging by Fraunhofer HHI”, JCTVC-C109, Guangzhou, October, 2010.

- JCT-VC, “Test Model under Consideration Software 0.9”, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9

- T. K. Tan, G. J. Sullivan, J.-R. Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Guangzhou, October, 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).