| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D318 |
| --- | --- |

| Title: | CE4: Cross verification of high precision chroma interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Email: | kemal.ugur@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports Core Experiment 4 cross verification results by Nokia. The results indicate that High Precision Chroma interpolation filter bring 0.35% and 0.55% gain on average for high efficiency and low complexity configurations, respectively.

# Results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux environment (on 24 core Intel Xeon X5650 @ 2.67GHz nodes). The BD-YUVPSNR numbers are calculated using the source code distributed among the CE participants.

|  |  | Random access |  |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -1.7 | -2.4 | -0.7 | -0.1 | -2.7 | -3.2 | -1.1 |
| Class B | -0.1 | -3.9 | -5.6 | -1.6 | -0.1 | -7.2 | -7.0 | -2.3 |
| Class C | -0.2 | -4.3 | -4.3 | -1.7 | -0.2 | -8.2 | -8.0 | -3.0 |
| Class D | -0.3 | -5.5 | -6.6 | -2.2 | -0.2 | -9.5 | -11.5 | -3.2 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -4.2 | -5.1 | -1.7 | -0.2 | -7.5 | -8.0 | -2.6 |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 104% |  |  |  | 108% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -2.4 | -3.3 | -1.3 | -0.9 | -9.7 | -11.5 | -3.9 |
| Class C | -0.5 | -3.7 | -4.4 | -2.0 | -0.9 | -10.2 | -9.5 | -4.7 |
| Class D | -0.7 | -5.6 | -5.9 | -2.3 | -1.3 | -9.7 | -11.6 | -4.3 |
| Class E | -0.2 | -5.1 | -5.6 | -1.8 | -0.2 | -0.5 | -4.2 | -0.8 |
| All | -0.5 | -4.0 | -4.6 | -1.8 | -0.9 | -8.1 | -9.7 | -3.6 |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 113% |  |  |  |

# Findings and recommendations

Results indicate that high precision chroma filter is bringing around 0.45% gain on average for all the tested configurations. The encoding time increase is negligible and decoding time increase is on average 7%.

As the current HM does not represent an optimal decoder implementation, the decoding complexity increase is probably more than 7%. This is because, in many implementations, the bilinear chroma interpolation complexity represents a much smaller complexity compared to luma interpolation and using a chroma interpolation as complex as luma interpolation would bring a sizable increase in total complexity (this comment is valid for all the CE4 proposals that use longer tap chroma filters).

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).