| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D319 |
| --- | --- |

| Title: | CE5: Cross verification of Qualcomm’s counter adaptation for LCEC tool |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Email: | kemal.ugur@nokia.com jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution is the cross check report for the Qualcomm proposal to improve the performance of LCEC.

# Results

Within CE5, several LCEC related improvements are being tested. Qualcomm proposed the idea of using counters to be used in VLC adaptation for shorter VLC tables. It should be noted that the encoding times are not very reliable due to varying loads on the cluster nodes. By examining the algorithm, we don’t expect much change to encoding times.

|  |  | Random access LoCo |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.1 |
| Class B | 0.0 | 0.1 | 0.1 |
| Class C | -0.2 | 0.1 | -0.1 |
| Class D | -0.2 | -0.2 | 0.0 |
| Class E |  |  |  |
| All | -0.1 | 0.0 | 0.1 |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 103% |  |  |

|  | Low delay LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.4 | 0.7 | 0.5 |
| Class C | -0.4 | 0.0 | 0.2 |
| Class D | -0.5 | 0.1 | 0.2 |
| Class E | -0.5 | 0.2 | -0.1 |
| All | -0.4 | 0.3 | 0.3 |
| Enc Time[%] | 79% |  |  |
| Dec Time[%] | 109% |  |  |

* The intra results are to be added soon.

# Findings and recommendations

It is found that modifying the table adaptation for shorter tables improve the performance of low complexity setting by 0.1% and 0.4% respectively for random access and low delay settings.

It should be noted that similar minor improvements could be achieved using simpler methods, without the need to keep track of counters for VLC tables.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).