| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D320 |
| --- | --- |

| Title: | CE11: Cross verification of Samsung’s proposal JCTVC-C205 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur | Email: | kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution is the cross check report for the Samsung’s proposal JCTVC-C205

# Results

Since the proposal was not integrated in LCEC, only the high efficiency cases were tested. The number of significant_coeff_flag elements is also counted using the method described by the CE coordinator. The experimental results are found as:

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | -0.7 | -1.3 | -1.1 | 2.38% |
| Class B | -0.9 | -1.8 | -1.8 | -10.49% |
| Class C | -1.3 | -1.6 | -1.5 | -6.10% |
| Class D | -1.2 | -1.5 | -1.5 | -8.26% |
| Class E | -1.8 | -1.8 | -2.2 | -8.53% |
| All | -1.2 | -1.6 | -1.6 | -6.20% |
| Enc Time[%] | 123% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Random access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | -0.3 | -0.5 | -0.6 | -0.25% |
| Class B | -0.6 | -1.0 | -1.1 | -11.29% |
| Class C | -0.5 | -0.9 | -0.9 | -11.09% |
| Class D | -0.5 | -0.9 | -0.5 | -10.76% |
| Class E |  |  |  |  |
| All | -0.5 | -0.9 | -0.8 | -8.35% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 81% |  |  |  |

|  | Low delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.8 | -11.89% |
| Class C | -0.5 | -0.7 | -0.7 | -13.21% |
| Class D | -0.5 | -1.5 | -0.3 | -11.68% |
| Class E | -0.4 | -0.6 | -2.1 | -19.40% |
| All | -0.6 | -0.9 | -0.9 | -14.04% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# Findings and recommendations

It is seen from the experimental results that the proposal is able to achieve gains on the magnitude of 0.5 to 1.2% It is also noted that the proposal is able to reduce the number of significant_coeff_flag elements by 6.2% to 14.04%.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).