| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D322 |
| --- | --- |

| Title: | CE3: On complexity calculation of interpolation filter for low complexity anchor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kemal Ugur | Email: | Kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Core Experiment 3 on Luma Interpolation is using a theoretical complexity analysis developed by TI and Samsung, by counting multiply/adds or add/shifts. One week before the meeting a mistake was identified on the complexity estimate for low complexity anchor. This contribution describes the mistake and its correction, and recommends using correct complexity numbers, not the wrong ones, for anchor data to evaluate various interpolation proposals.

# Introduction

The complexity analysis used in CE3 tries to estimate the complexities of various filters by counting multiply/adds or adds/shifts. For the high efficiency DCT-IF 12-tap anchor, when the filter coefficients are symmetric the number of multiply and adds (or adds/shifts) are reduced accordingly. However this is not done for the case of DIF, even though DIF also uses symmetrical filters. This makes the complexity estimates unfair for the DIF case.

More specifically, the complexity estimate for DIF assumes a symmetrical filter is implemented as (for the sake of simplicity, single 6-tap case is given here as an example):

filt_coef[0].A + filt_coef[1].B + filt_coef[2].C + filt_coef[2].D + filt_coef[2].E + filt_coef[1].F + filt_coef[0].G 		(1)

Whereas for other symmetrical filters complexity estimate assumes filter is implemented as:

filt_coef[0].(A+F) + filt_coef[1].(B+E) + filt_coef[2].(C+D) 						(2)

It is proposed to utilize the symmetry assumption for both of the anchors when counting operations, to make the comparisons fair. (It should be noted the implementation of DIF in HMv0.9 is using the function (1) but since the filters are symmetric there is no reason why it can’t be implemented as in (2))

The attached excel sheet is the modified version from the one used in CE3 and it assumes DIF is also implemented with symmetry. The modified numbers were calculated by Nokia and independently verified by Samsung. In addition, source code is attached that implements DIF using the symmetry which produces identical results as the anchor.

# Conclusions

Since the fix affects the complexity estimate significantly (around 20% for low complexity anchor), it is proposed to use the correct complexity numbers for the anchor within the CE-3 complexity analysis, not the wrong ones.