| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D323 |
| --- | --- |

| Title: | CE3: Results for luma interpolation filter tests by Nokia and Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur1 Jani Lainema1 Antti Hallapuro1 Marta Karczewicz2 In Suk Chong2 Liwei Guo2 | Email: | Kemal.ugur@nokia.com martak@qualcomm.com |
| Source: | Nokia1, Qualcomm2 |  |  |

_____________________________

# Abstract

This document presents the details of Nokia’s and Qualcomm’s proposal to CE3 – Luma Interpolation Filter. The proposal argues that 1D directional filter represents a low complexity design which is good for majority of the sequences. For the sequences where there is significant high frequency content in the input video, proposal switches to use a separable interpolation filter. Results for both 6-tap and 8-tap switching between separable and directional filters are provided. Various filter coefficients that have been proposed within the CE have been tested.

# Introduction

The current HM utilizes 1D 6-tap directional filters for low complexity and 2D separable filters for high efficiency settings. The 1D directional filter is chosen as low complexity anchor due to the complexity benefit it provides. The worst case and average complexity in terms of instructions for directional over separable is given as (this assumes DIF and DCT-IF are implemented using the same symmetric equations):

DIF6tap	Separable6tap with DCT-IF

Average		100%		165%

Worst Case	100%		226%

DIF8tap	Separable8tap with DCT-IF

Average		100%		177%

Worst Case	100%		261%

It was reported in TE12 experiments at Guangzhou meeting that 1D directional filters suffer a coding efficiency loss over separable filter for certain sequences, like BQSquare, where there is significant high frequency content.

This proposal uses frame level switching between directional and separable filter. This way, the main complexity benefits of directional filter is preserved, whereas for sequences with significant high frequency content, separable filter is used to improve the coding efficiency.

It should be noted that Nokia and Qualcomm have independently arrived to a very similar design except small differences in the encoder algorithm. For the sake of unification, Nokia and Qualcomm decided to present their proposals jointly.

Nokia’s encoder algorithm first determines if the picture includes significant high frequency content using sum of absolute gradients and if the sum is larger than a threshold, than separable filter is utilized. Otherwise, 1D directional filter is utilized. The tested filter coefficients are described in the next section.

Qualcomm’s design utilizes the motion estimation results from the previous frame to decide on the filter for the current frame.

# Results

The subsections below summarize the objective performance of the proposed algorithm with various coefficients. All the data could be found in attached excel sheets.  It should be noted that for DIF, position j is always interpolated using a strong filter, whose coefficients are given as:

0 5  5  0

5 22 22 5

5 22 22 5

0 5  5  0

## 8-tap switching with original DCT-IF coefficients

In this test the DCT-IF coefficients in the HM v0.9 are used and they are given as:

½ position: 			{-1, 5,-12, 40, 40,-12, 5, -1}

¼ and ¾ positions: 	{-1, 4,-10, 57, 19, -7, 3, -1}

Results are given for both Nokia’s and Qualcomm’s proposal as:

Nokia’s Proposal		Qualcomm’s Proposal

vs. LoCo(Avg)	-4.2%				-4.25

vs. HE(Avg)		+0.7%				+0.4%

## 8-tap switching with new DCT-IF coefficients with 6-bit accuracy

In this test the new DCT-IF coefficient set that was developed by Samsung with 6-bit accuracy are used. It should be noted that Samsung proposed another set of new DCT-IF coefficients with 8-bit accuracy, but they are not utilized here due to the problems in storing high precision intermediate values. The filter values are given as:

½ position: 			{-1, 4,-11, 40, 40,-11, 4, -1}

¼ and ¾ positions: 	{-1, 4,-10, 57, 19, -7, 3, -1}

Results are given for both Nokia’s and Qualcomm’s proposal as:

Nokia’s Proposal		Qualcomm’s Proposal

vs. LoCo(Avg)	XXX				XXX

vs. HE(Avg)		XXX				XXX

## 6-tap switching

This test was proposed for low complexity operating point and 6-tap filters are used for switching. In this test, 1D-directional filter uses the DIF coefficients in HM low complexity anchor and it is given as:

½ position: 			{3, -17, 78, 78,-17, 3}

¼ and ¾ positions: 	{3, -15,111, 37,-10, 2}

Separable filter uses the original DCT-IF 6-tap coefficients given as:

½ position: 			{1, -5, 20,20, -5, 1}

¼ and ¾ positions: 	{1, -4, 28, 9, -3, 1}

Results are given for Nokia’s and Qualcomm’s proposal as:

Nokia’s Proposal		Qualcomm’s Proposal

vs. LoCo(Avg)	-2.15%				-2.3%

To consider the effect of different filters, Qualcomm provided supplemental results using two other high precision DCT-IF coefficients that was developed by Samsung.  In this test, separable filter is using the new DCT-IF coefficients and it is given as:

DCT-IF coefficients in cfp of  Samsung (Case 1)

½ position: 			{ 5,-33,156,156,-33, 5}

¼ and ¾ positions: 	{ 8,-35,227, 73,-23, 6}

New DCT-IF coefficient proposed to CE3 in this meeting (Case 2)

½ position:                     { 11,-43,160,160,-43,11}

¼ and ¾ positions:        {  9,-37,227, 75,-25, 7}

The results are given as:

(Case 1)

Nokia’s Proposal		Qualcomm’s Proposal

vs. LoCo(Avg)	N/A				-3.1%

(Case 2)

Nokia’s Proposal		Qualcomm’s Proposal

vs. LoCo(Avg)	N/A				-2.7%

# Complexity Analysis

For the Nokia’s encoder algorithm, the frequency of directional and separable filters for each sequence is given below. As can be seen, for HD sequences, it is almost directional filters are chosen (~92.2%) and over all sequences directional is chosen on average 69.5%.

|  | Directional percentage | Separable percentage |
| --- | --- | --- |
| Traffic | 100.00% | 0.00% |
| PeopleOnStreet | 100.00% | 0.00% |
| Kimono | 100.00% | 0.00% |
| ParkScene | 100.00% | 0.00% |
| Cactus | 100.00% | 0.00% |
| BasketballDrive | 100.00% | 0.00% |
| BQTerrace | 22.17% | 77.83% |
| BasketballDrill | 100.00% | 0.00% |
| BQMall | 51.17% | 48.83% |
| PartyScene | 0.00% | 100.00% |
| RaceHorses | 57.67% | 42.33% |
| BasketballPass | 83.20% | 16.80% |
| BQSquare | 0.00% | 100.00% |
| BlowingBubbles | 0.00% | 100.00% |
| RaceHorses | 36.67% | 63.33% |
| Vidyo1 | 100.00% | 0.00% |
| Vidyo3 | 100.00% | 0.00% |
| Vidyo4 | 100.00% | 0.00% |
| Average_HD | 92.22% | 7.78% |
| Average_all | 69.49% | 30.51% |

Since this proposal is hybrid between separable and directional, the complexity also lies in between. The worst-case complexity happens if separable filter is chosen and all the blocks are 4x4, and MV points to the most complex position (e.g. position f). For the average complexity, actual statistics as given above are calculated .

It should be noted that when calculating the numbers, it is assumed that DIF could be implemented with symmetry for the proposal and not for the anchor, as no agreement was reached to change the anchor complexity numbers within the CE3.  Therefore, complexity numbers for the proposal are looking slightly better than it really is:

## 8-tap switching with original DCT-IF coefficients

vs. LC Anchor		vs. HE Anchor

Average			139%				41%

Worst Case		261%				63%

## 8-tap switching with new 6-bit DCT-IF coefficients

vs. LC Anchor		vs. HE Anchor

Average			XXX%				XXX%

Worst Case		XXX%				XXX%

## 6-tap switching

vs. LC Anchor		vs. HE Anchor

Average			102%				36%

Worst Case		170%				39%

# Conclusions

This proposal presents a simple frame-level switching between directional and separable filters, using various coefficient sets. We propose two possibilities using these data. First we propose to use 8tap switching filters for high complexity and 6 tap switching filters for low complexity. Another possibility is a unified solution for both high efficiency and low complexity configurations.

At the Guangzhou meeting there was a strong desire to unify the interpolation filters for both the low complexity and high efficiency cases. It is believed that the proposed 8-tap design with 6-bit accuracy DCT-IF coefficients from Samsung could be a candidate for this unified design, because of the following benefits:

- It could be run in low complexity mode using 1D directional filters for majority of the sequences.

- The coding efficiency comes very close to the 12-tap separable DCT-IF filter. (with 0.4% penalty), with significant complexity benefit.

- Filter coefficients use low accuracy, making the 16-bit software implementation significantly easier

- Filters for both 1D-directional and separable use the same coefficients, making the hardware implementation easier compared to switching between different coefficients.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The selection statistics for Qualcomm’s encoder algorithm is not available.