| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D324 |
| --- | --- |

| Title: | CE4: Results for chroma interpolation filter tests by Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Email: | Kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents the details of Nokia’s proposal to CE4 – Chroma Interpolation Filter. The proposal argues that 1D directional filter represents a low complexity design and utilizes it for interpolation chroma samples. Two variants of the proposal is tested and described below.

# Introduction

The current HM utilizes bilinear interpolation for obtaining fractional chroma samples. The low complexity setting of HM uses 1D directional filters for luma interpolation. In this proposal, the performance of directional interpolation for fractional chroma samples is evaluated. Two variants of the proposal is evaluated:

Variant-1

Assume the following 1/8 pel grid where black squares indicate integer pixels. The white positions (e.g. 1/8, 1/8) are obtained using regular bilinear prediction and green positions (e.g. 2/8, 2/8) are obtained using directional filter using same coefficients of luma interpolation.

The main benefit of this variant is that, there is no need to support interpolation with different coefficients for chroma, which is beneficial for hardware implementations.

Variant-2

In this variant, we also add 1/8 positions that can be implemented using a single 1D directional filter as seen below.

# Results

Variant – 1 (no additional coefficients are needed)

|  |  |  |  | vs | TMuC 0.9 HM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  |  | Random access LoCo |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.0 | -0.6 | -0.7 | -0.2 | 0.0 | -0.5 | -0.6 | -0.2 |
| Class B | 0.0 | -1.2 | -1.8 | -0.5 | 0.0 | -1.6 | -1.8 | -0.5 |
| Class C | 0.0 | -1.2 | -1.1 | -0.4 | 0.0 | -2.7 | -2.5 | -0.9 |
| Class D | 0.0 | -1.0 | -1.2 | -0.4 | 0.0 | -2.9 | -3.6 | -0.9 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | -1.1 | -1.3 | -0.4 | 0.0 | -2.1 | -2.3 | -0.7 |
| Enc Time[%] | 94% |  |  |  | 89% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 101% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.8 | -1.3 | -0.4 | -0.2 | -2.0 | -3.0 | -0.8 |
| Class C | -0.1 | -1.0 | -1.1 | -0.5 | -0.3 | -2.2 | -2.0 | -1.0 |
| Class D | -0.1 | -1.3 | -0.6 | -0.4 | -0.4 | -2.3 | -2.8 | -1.0 |
| Class E | -0.1 | -1.1 | -1.2 | -0.4 | -0.1 | -0.3 | -0.8 | -0.2 |
| All | -0.1 | -1.0 | -1.1 | -0.4 | -0.3 | -1.8 | -2.3 | -0.8 |
| Enc Time[%] | 91% |  |  |  | 80% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 103% |  |  |  |

Variant – 2

| Tool 1 |  |  |  | vs | TMuC 0.9 HM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  |  | Random access LoCo |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0.0 | -0.8 | -1.2 | -0.4 | -0.1 | -1.6 | -1.7 | -0.7 |
| Class B | 0.0 | -1.8 | -2.9 | -0.8 | 0.0 | -2.8 | -3.2 | -0.9 |
| Class C | 0.0 | -2.5 | -2.3 | -0.9 | -0.1 | -5.3 | -5.0 | -1.9 |
| Class D | -0.1 | -2.4 | -2.4 | -0.9 | 0.0 | -4.4 | -5.1 | -1.5 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | -2.0 | -2.4 | -0.8 | 0.0 | -3.7 | -4.0 | -1.3 |
| Enc Time[%] | 95% |  |  |  | 91% |  |  |  |
| Dec Time[%] | 82% |  |  |  | 102% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2 | -1.6 | -2.3 | -0.8 | -0.3 | -3.9 | -4.9 | -1.6 |
| Class C | -0.2 | -2.3 | -2.4 | -1.1 | -0.6 | -7.6 | -7.1 | -3.4 |
| Class D | -0.3 | -3.1 | -2.6 | -1.2 | -0.8 | -6.0 | -7.0 | -2.5 |
| Class E | -0.1 | -4.5 | -5.2 | -1.6 | -0.2 | 0.8 | -1.7 | -0.3 |
| All | -0.2 | -2.7 | -2.9 | -1.1 | -0.5 | -4.5 | -5.4 | -2.0 |
| Enc Time[%] | 91% |  |  |  | 80% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 104% |  |  |  |

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).