| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D325 |
| --- | --- |

| Title: | CE 12: Nokia proposal on adaptive MV resolution with directional filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema | Email: | Kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents Nokia’s results for Core Experiment 12. This proposal utilizes 1D directional filters for achieving 1/8 pixel resolution. Since all the utilized 1/8 positions can be interpolated using a single 1D interpolation filter, the proposal improves coding efficiency and reduces average decoding complexity at the same time. Due to the need to search for additional positions, encoding complexity is increased.

# Introduction

Due to its low complexity, directional filters were included in the low complexity profile of HM. JCTVC-C215 extended directional filters and proposed to use adaptive MV resolution using 1D directional filters.

This contribution proposes adaptive MV resolution with the directional interpolation that improves the coding efficiency without increase in decoding complexity (actually the decoding complexity is reduced further as described below).

The design is very similar to AMVRES concept that was in TMuC with the following differences:

1. The 1/8 motion vectors are restricted so that they are allowed to point only to positions that are located where they could be interpolated using 1D directional filters (see small dots Figure 1).

2. The motion vector prediction is always done in quarter pixel. Encoder signals MVD and “one eight pixel refinement bit” to the decoder. If the refinement bit is 0, then MV with quarter-pixel accuracy is used. Otherwise decoder maps the quarter pixel position to one of the one eight pixel positions.

Figure 1. Utilized one eight pixel positions (small filled circles). Squares donate integer pixels, circles donate half-pixel and triangles donate quarter-pixel. All one-eight positions are diagonally aligned with integer samples, and could be obtained using a single 1D filter.

Since all one eight positions are diagonally aligned with integer pixel samples, they are calculated using single 1D 6-tap filters. This means that the decoding complexity is even further reduced compared to directional filters when this proposal is used.

# Experimental Results

|  |  | Random access LoCo |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 1.3 | 1.2 |
| Class B | -0.3 | 1.1 | 1.0 |
| Class C | -0.7 | 1.4 | 1.1 |
| Class D | -1.2 | 0.8 | 0.9 |
| Class E |  |  |  |
| All | -0.6 | 1.1 | 1.0 |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.4 | 0.0 | -0.2 |
| Class C | -0.6 | 0.0 | -0.2 |
| Class D | -0.9 | -1.1 | -1.3 |
| Class E | -0.2 | 0.2 | 2.6 |
| All | -0.6 | -0.2 | 0.1 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 106% |  |  |

* The decoding time is unexpectedly increased and after some investigation its reason is found as: The filter coefficients for 1/8 pixel positions are stored in an integer array. This means the compiler cannot perform machine code optimization as it treats those coefficients as a variable. Since the interpolation filter for the anchor does not store the filter coefficients in an array, it does not suffer from this phenomenon.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).