| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D326 |
| --- | --- |

| Title: | Planar intra coding for improved subjective video quality |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema, Kemal Ugur, Oguz Bici Nokia Research Center Visiokatu 1 Tampere, Finland | Tel: Email: | jani.lainema@nokia.com kemal.ugur@nokia.com ext-oguz.bici@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document describes encoder modifications for planar coding with an aim to improve subjective quality when the planar intra coding is enabled in the HM 0.9 environment. Planar coding was initially included in TMuC, but it was decided in Guangzhou meeting to further study it and consider encoder decisions based on subjective quality. This contribution proposes the planar coding, but with an encoder algorithm to improve the subjective performance. Since planar coding is a tool to improve subjective quality, rather than objective quality, visual examples demonstrating the claimed benefit of planar coding are also included. Finally, restricting the block size of planar coded blocks is proposed to further improve objective results without affecting subjective quality.

# Introduction

Section 2 provides details of encoder modifications introduced to improve the subjective quality of planar mode. Section 3 discusses the indication of the planar mode in the low complexity entropy coder environment and proposes to apply joint coding of the indication of planar mode and PU size indicator to better match the probabilities of different combinations of these two data elements. Section 4 provides proposal for restricting planar CU sizes. Finally, Section 5 presents both the objective compression results and subjective results by providing several snapshots where planar coding helps improving the subjective quality.

# Encoder modifications to improve subjective quality

Since planar coding is a tool to improve subjective quality, rather than objective quality, selection of planar mode by the encoder based on objective RD decisions does not fully demonstrate the benefits of the mode. For this reason, modifications to planar mode selection process are proposed.

The modifications are as follows. For each CU, the encoder checks whether the original image block is smooth. The smoothness checking algorithm is presented in the following subsection. If the block is identified as smooth, rate cost for planar coding of the CU is set to 0 so that it may be chosen to be planar coded more frequently in these areas. Also in this way, a weighted favoring is provided when this CU is subpart of a larger CU. The distortion cost is not modified so that in case of a false alarm, the CU is not planar coded due to higher RD cost.

## Smoothness check

The aim of the smoothness check is to determine whether a given picture block is smooth so that planar coding of this area is likely to produce a reconstruction with high subjective quality. The input is original image block corresponding to the CU region. The first step is calculating the low frequency energy of the original image block as the sum of energies of the 2x2 top-left DCT coefficients and then comparing the ratio of the low frequency energy to the total energy of the block. If the ratio is higher than empirically determined threshold, the block is considered as a candidate smooth block for the next step. In the second step we are discarding blocks with texture that may be better represented with directional intra prediction. This is also done by analyzing the same 2D DCT of original picture block. For each topmost row and leftmost column DCT coefficient excluding the 2x2 top-left coefficients, the value of (energy of the DCT coefficient)/(Total energy – DC energy) is compared against empirically determined threshold. If any of them is above the threshold for a given block, the block is not considered smooth.

# Indication of planar coding in low complexity entropy coding

When planar coding is enabled in HM 0.9, there is roughly a 1.3% objective penalty in the Intra Low Complexity configuration. As mentioned in [1], this is a result of the rather simplistic implementation of the indication of Planar mode in the earlier TMuC low complexity entropy coder configurations where one bit is allocated for each Intra coded CU to indicate whether the Planar mode is used or not. As the probability of the planar mode is often significantly less than 50% in typical video sequences this leads into suboptimal objective results. As proposed earlier in [1], joint coding of the planar_flag and intra_split_flag syntax elements is also proposed in this document with the following variable length code:

| codeword | planar_flag | intra_split_flag |
| --- | --- | --- |
| 1 | 0 | 0 |
| 01 | 0 | 1 |
| 00 | 1 | 0 |

# Restricting planar coded block sizes

Initial experiments showed that planar coded block sizes smaller than 16 did not occur frequently and did not help subjective visual quality. Therefore, planar CU sizes are proposed to be restricted to 16x16, 32x32 and 64x64. In this way, planar mode is not signaled for smaller block sizes, which brings additional objective improvement.

# Results

## Objective results

The tables below summarize the objective performance of proposed planar coding with the encoder modifications described in Section 2, the planar mode indication described in Section 3 and the block size restriction described in Section 4. The anchor is HM 0.9. Full results are included in the JCTVC-D326-PlanarSubjective.xls.

The table shows that planar mode has minimal impact on the objective performance of the codec when applying the proposed joint coding and block size restrictions.

For the results reported above, DCT coefficients required during the smoothness check are calculated with double precision and without optimization. In order to provide realistic encoding times of Intra configurations, it was also investigated to use integer based DCT calculations available in HM software for the smoothness check procedure. The results for the intra configurations are shown in the table below. The results indicate that BD-rate change is very small and there are also no visual effects due to slight differences in DCT calculations. However, it is observed that encoding time is reduced significantly.

## Planar percentages

In this part, resultant percentage ratios of planar coded image area to whole picture area for intra high efficiency and intra low complexity are presented. Planar percentage is calculated for each sequence and for each QP by averaging the percentages of all frames. All results are available in JCTVC-D326-PlanarSubjective_PlanarPercentages.xls.

Average planar percentage of all sequences for intra high efficiency is as below:

| QP22 | QP27 | QP32 | QP37 |
| --- | --- | --- | --- |
| 2.09 | 4.23 | 6.34 | 7.84 |

Average planar percentage of all sequences for intra low complexity is as below:

| QP22 | QP27 | QP32 | QP37 |
| --- | --- | --- | --- |
| 2.19 | 4.04 | 6.06 | 7.56 |

## Subjective results

Several snapshots are presented to compare subjective quality of HM 0.9 as anchor and the planar coding proposed in this document. In the snapshots, the left/top image is from the anchor and the right/bottom image is from the proposed planar enabled codec.

Figure 1 Kimono face region, frame 1, Intra low complexity, QP=37

Figure 2 Kimono sky region, frame 147, Intra low complexity, QP=37

Figure 3 Kimono face region, frame 3, Intra low complexity, QP=32

Figure 4 Kimono sky region, frame 147, Intra low complexity, QP=32

Figure 5 Kimono face region, frame 15, Intra high efficiency, QP=32

Figure 6 Kimono sky region, frame 142, Intra high efficiency, QP=32

Figure 7 Kimono face region, frame 90, Intra low complexity, QP=27

Figure 8 Kimono sky region, frame 154, Intra low complexity, QP=27

Figure 9 Kimono face region, frame 28, Random access low complexity, QP=32

Figure 10 Kimono sky region, frame 151, Random access low complexity, QP=32

Figure 11 Vidyo1, frame 128, Intra high efficiency, QP=32

Figure 12 Vidyo 4, frame 3, Intra high efficiency, QP=32

Figure 13 Vidyo 4, frame 32, Intra high efficiency, QP=32

Figure 14 Vidyo4, frame 11, Intra low complexity, QP=37

Figure 15 Vidyo3, frame 51, Intra low complexity, QP=37

# References

[1] J. Lainema, K. Ugur and O. Bici “TE12.3: Results for planar prediction tests by Nokia,” JCT-VC Doc. JCTVC-C188, Guangzhou, Oct 2010.

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).