| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4rd Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D328 WG11 Number: m19094 |
| --- | --- |

| Title: | CE2: cross-checking of Asymmetric motion partitioning with OBMC proposal from Samsung and Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard Francois, Philippe Bordes | Tel: Email: | +33-2-99-27-32-62 edouard.francois@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment 2 (CE2)-subset 1 cross-checking. CE2-subset 1 relates to the use of OBMC on top of Asymmetric motion Partitioning (AMP) tool (initial proposals JCTVC-A124 and JCTVC-C251), in order to reduce boundary effects at the partitioning line. The implementation from the proponent has been verified and the results have been exactly reproduced.

- Code verification

The source code has been delivered by Qualcomm to the cross-checker Technicolor. The implementation is made on top of the AMP implementation into TMuC0.9 from Samsung.

The OBMC tool implementation has been analyzed.

The source code has been compiled successfully using Linux (Red Hat 4.1.2-42) with gcc4.1.2.

# Results

The encoder and decoder compiling and executions have been verified. Results have been generated for the Random access and Low delay High efficiency cases. The comparison is made with the TMuC0.9 as anchor.

The simulations have been run with a non-homogeneous computers cluster and therefore the encoding computing times are not really reliable. S For decoding time, all the simulations have been made on the same type of computers so the numbers can be trusted.

The table below summarizes the obtained results:

|  | Random access HE |  |  | Low Delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -1.6 | -1.5 |  |  |  |
| Class B | -0.8 | -1.0 | -1.0 | -0.9 | -1.4 | -1.4 |
| Class C | -1.1 | -1.4 | -1.4 | -1.1 | -1.4 | -1.6 |
| Class D | -1.1 | -1.1 | -1.1 | -1.1 | -2.1 | -1.4 |
| Class E |  |  |  | -2.1 | -2.7 | -2.5 |
| All | -0.9 | -1.2 | -1.1 | -1.2 | -1.8 | -1.6 |
| Enc. time | 152% |  |  | 153% |  |  |
| Dec. time | 100% |  |  | 99% |  |  |

The results obtained by Technicolor perfectly match the results provided by Qualcomm.

# Conclusion

The source code version implementing AMP with OBMC has been provided. The cross-checking of results have been done by Technicolor, and perfectly match the results provided by Qualcomm.