| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D331 |
| --- | --- |

| Title: | Cross verification on scalable motion vector competition and simplified MVP calculation (JCTVC-D055) from TI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of JCTVC-D055 [1] on scalable motion vector competition and simplified MVP calculation submitted by TI. The cross-check results match those proposed in JCTVC-D055. Attached spreadsheet contains detailed data of the results.

# Simplified MVP calculation

The proposed technique from TI simplifies MVP candidate selection for the above and the left spatial neighbors. Instead of trying to search all the adjacent spatial neighbors until the MVP candidate having the same reference index of the current PU is found, it tries to search the first and the last adjacent one on each of the left and the above side.

# Result

When compared with the anchor defined in JCTVC-C500, simplified MVP calculation has no loss in average for all configurations except small U BD-rate loss 0.6% in class E for LC-LD.

For high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), the average encoding times are 99%, 103%, 98%, and 100%, and the average decoding times are 100%, 104%, 98%, and 100%, respectively. Encoding/decoding time may not be accurate since we simulate the result on clustered system.

Table 1. Summary of BD-rate and encoding/decoding time for 4 configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.1 | -0.4 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | -0.2 | 0.0 | 0.6 | 0.0 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Enc Time[%] | 98% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusions

The existing Advanced Motion Vector Prediction scheme in HM is the tradeoff between complexity and coding efficiency. This contribution shows the good possibility to reduce complexity by simplification. To find the best scheme to reduce the complexity of AMVP schemes, we would like to create CE on AMVP simplification including this approach.

# References

- Minhua Zhou, Scalable motion vector competition and simplified MVP calculation, in 4th JCT-VC meeting , no. JCTVC-D055, Jan. 2011.