| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D333 |
| --- | --- |

| Title: | Cross verification on Improved AMVP (JCTVC-D125) from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of JCTVC-D125 [1] on Improved Advanced Motion Vector Prediction submitted by MediaTek. The cross-check results match those proposed in JCTVC-D125. Attached spreadsheet contains detailed data of the results.

# Improved Advanced Motion Vector Prediction

Improved Advanced Motion Vector Prediction introduces maximum 5 more motion vector prediction candidates, in which 2 more corner candidates and 3 more temporal candidates are used to compared to existing Advanced Motion Vector Prediction. To increase the possibility of usage of motion vector prediction candidates, scaled motion vector prediction scheme is introduced.

By the fast estimation algorithm for Direct mode, encoder complexity is reduced compared to existing Direct mode. In case of low complexity profile, the total number of candidates of motion vector prediction is limited to 2 if the picture of collocated block is adjacent to the current picture, which may further reduce the complexity.

# Result

When compared with the anchor in JCTVC-C500, IAMVP achieve -2.1%, -2.0%, -1.5%, and -1.0% BD-rate reductions for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively. For HE-RA, LC-RA, HE-LD, and LC-LD, the average encoding times are 94%, 99%, 93%, and 95%, and the average decoding times are 97%, 108%, 102%, and 104%, respectively. Summarized results are shown in Table 1.

Table 1. Summary of BD-rate reduction and encoding/decoding time for 4 configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.1 | -2.9 | -2.7 | -2.9 | -2.8 | -2.8 |
| Class B | -1.8 | -1.6 | -1.4 | -1.7 | -1.5 | -1.5 |
| Class C | -2.0 | -2.2 | -2.3 | -1.9 | -1.8 | -2.0 |
| Class D | -2.1 | -1.7 | -1.8 | -1.8 | -1.7 | -1.7 |
| Class E |  |  |  |  |  |  |
| All | -2.1 | -2.0 | -1.9 | -2.0 | -1.8 | -1.9 |
| Enc Time[%] | 94% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -1.0 | -0.8 | -0.9 | -1.0 | -0.8 |
| Class C | -1.6 | -1.6 | -1.5 | -1.1 | -1.2 | -1.1 |
| Class D | -1.3 | -1.9 | -0.7 | -1.4 | -1.1 | -0.9 |
| Class E | -1.6 | 1.3 | -0.4 | -0.6 | -0.3 | -0.5 |
| All | -1.5 | -1.0 | -0.9 | -1.0 | -0.9 | -0.8 |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

# Conclusions

The existing Advanced Motion Vector Prediction scheme in HM is the tradeoff between complexity and coding efficiency. This contribution shows that its coding efficiency can increase up to 2% by using additional motion vector prediction candidates without noticeable increases of encoder and decoder running times. Thus we would like to recommend considering this improved design of AMVP in the next release of test model.

# References

- Jian-Liang Lin, Yu-Pao Tsai, Yu-Wen Huang, and Shawmin Lei, Improved Advanced Motion Vector Prediction,  in 4th JCT-VC meeting , no. JCTVC-D125, Jan. 2011.