| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D334 |
| --- | --- |

| Title: | CE8 subset1: Results of intra deblocking filter testing by SKKU/SKT (JCTVC-C130) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungyoup Yang Kwanghyun Won Heechul Yang Byeungwoo Jeon Sungkyunkwan University 300 Chunchun-dong, Jangan-gu, Suwon, Korea Jeongyeon Lim SK Telecom 9-1 Sunae-dong, Bundang-gu, Sungnam, Korea | Tel: Email: | +82-31-290-7186 binbak1@ece.skku.ac.kr +82-31-290-7186 wkh12345@skku.edu +82-31-290-7144 heechul2@skku.edu +82-31-290-7144 bjeon@skku.edu +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sungkyunkwan University and SK Telecom |  |  |

_____________________________

# Abstract

This document presents an intra deblocking filter (DF) which was proposed in JCTVC-C130 [1]. The proposed method employs exactly the same DF scheme as that in the TMuC0.9 except filter strength control for intra blocks. Even the small change of assigning different strength of DF for intra blocks have generated both positive coding gain and improved visual quality by preserving vertical and horizontal edges.

- Proposed intra deblocking filter

## Modified filtering decision

In TMuC DF, if blocks P and Q are intra coded, the boundary between them is always filtered. However, in the proposed method, although P and Q are intra coded, if intra prediction modes of blocks P and Q are the same and if they don’t contain non-zero transform coefficients, the boundary between them is not filtered. It is based on the understanding that in this case, the blocks are expected to be very similar in their contents and the blocking artifact may not be conspicuous. Other filtering decision conditions are the same as the TMuC DF.

## Control of the filtering strength for intra blocks

As shown in Table 1, if the boundary between blocks P and B is decided to be filtered by the filtering decision process, the proposed method controls the strength of DF for intra coded blocks as follows.

Table 1. Control of the filtering strength

| Conditions | Filtering |
| --- | --- |
| P and/or Q are intra coded and P and Q have different PredMode | Strong and weak filtering with tC+2 |
| P and Q are intra coded and P and Q have different IntraPredMode | Strong and weak filtering with tC+1 |
| P and Q are intra coded and P and Q have the same IntraPredMode and boundary direction & prediction direction are different | Weak filtering with tC |
| Otherwise | Weak smoothing filtering with tC |

If block P and/or block Q are intra-coded and their prediction modes are different, the same filtering process of TMuC is performed with tC plus 2 since the boundary between P and Q may have strong blocking artifact. It is to note that one of two types of filtering (weak and strong filtering) is applied by using the same decision criterion of TMuC DF. Otherwise, intra prediction modes of P and Q are considered to jointly control the strength of filtering. Different intra prediction modes of P and Q indicate that block P and block Q are predicted in different directions, thus the effects of intra prediction and quantization of blocks P and Q are expected to be non-identical. In this case, the same filtering process of TMuC is performed with tC plus 1. Otherwise, if intra prediction modes of P and Q are the same, P and Q are most likely to be in the same region. Therefore, in this case, if block boundary direction and prediction are the same, the weak filtering with tC is only applied. Otherwise, a weak smoothing filter with tC is only applied as follows:

Note that strong filtering is not applied to avoid an unintended distortion for these two cases. When directions of block boundary and prediction are the same, for example, as shown in Fig 1 (a), the two blocks P and Q are most likely to belong to different objects, and the boundary is caused by texture. Thus, weak filtering instead of the strong filtering should be helpful to avoid an unintended smoothing over the block boundary between P and Q. On the other hand, if intra prediction is performed across the given block boundary, P and Q blocks are most likely to belong to the same object, so weak smoothing might help to remove the quantization error which otherwise may be visible across the boundary.

(a) Same direction                            (b) Different direction

Fig 1. Example of block boundary direction and prediction direction

For this reason, in the proposed method, vertical and horizontal intra prediction modes are considered to check the difference between block boundary direction and prediction direction. Since DF is applied to vertical and horizontal block boundaries, in this proposal, vertical and horizontal intra prediction modes are considered to check the difference between boundary direction and prediction direction. Other filtering operations are the same as the TMuC DF.

# Experimental results

The proposed method is implemented on TMuC0.9 software and simulated under CE10 Subset1 conditions [2]. Table 2 shows the summarized coding performance of the proposed method compared to TMuC0.9 anchors. More detailed results are included in JCTVC-D334_CE8Subset1_results_SKKU.xls and JCTVC-D334_CE8Subset1_add_results_SKKU.xls.

Table 2. Experimental results

| BDBR | HE |  |  | LC |  |  | HE w/o ALF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD | IO | RA | LD |
| Class A | -0.9 | -0.5 |  | -0.5 | -0.3 |  | -0.5 | -0.3 |  |
| Class B | -0.6 | -0.3 | -0.1 | -0.5 | -0.3 | 0.0 | -0.4 | -0.2 | 0.0 |
| Class C | -0.5 | -0.3 | -0.1 | -0.5 | -0.2 | 0.0 | -0.5 | -0.2 | -0.1 |
| Class D | -0.5 | -0.2 | -0.1 | -0.4 | -0.2 | -0.1 | -0.4 | -0.1 | 0.0 |
| Class E | -0.6 |  | -0.2 | -0.3 |  | -0.1 | -0.3 |  | -0.1 |
| All | -0.6 | -0.3 | -0.1 | -0.4 | -0.3 | -0.1 | -0.4 | -0.2 | -0.1 |
| Enc. Time | 102 | 99 | 101 | 104 | 100 | 100 | 100 | 100 | 101 |
| Dec. Time | 101 | 100 | 100 | 102 | 102 | 100 | 101 | 100 | 100 |

Fig. 2 and 3 shows the subjective quality comparison between TMuC0.9 anchor and the proposed. As shown in figures, over-smoothing effect can be removed by the proposed method.

(a) TMuC0.9                                (b) Proposed

Fig. 2. Subjective quality comparison (IO(LC)_BQMall_832x480_QP37)

(a) TMuC0.9                                (b) Proposed

Fig. 3. Subjective quality comparison (IO(LC)_PartyScene_832x480_QP32)

# Concluding remarks

In this document, the results of the proposed intra deblocking filter [1] are presented. The proposed method employs exactly the same DF scheme as that in the TMuC except filter strength control for intra blocks. However, even the small change of assigning different strength of DF for intra-coded blocks have generated positive coding gain and improved subjective quality.

# References

[1] J. Yang, K. Won, B. Jeon, J. Lim and J. Song, “In-loop deblocking filtering for intra blocks,” JCTVC-C130, Oct, 2010.

[2] T. Yamakage, T. Chujoh, Y.W. Huang, K. Chono, I.S. Chong, T. Yamamoto, J. Lim, B. Jeon, J. Xu and M. Narroschke, “Description of CE8: In-loop filtering,” JCTVC-C508r1, Oct. 2010.

SKKU and SK Telecom may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).