| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D337 |
| --- | --- |

| Title: | Improved motion vector predictor selection in AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Tammy Lee | Email: | ilkoo.kim@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes encoder-side improvements of the motion vector predictor selection in AMVP implementation. The proposed method compares RD-cost with all motion vector predictors combination for all reference indices with zero mvd rather than simply using the template matching. Furthermore, this contribution uses the same interpolation filter and distortion method of motion estimation when setting the initial motion vector predictor to be used as motion search starting point. The experimental result shows -0.3%, -0.4%, -0.6%, and -0.4% BD-rate reduction for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively.

# Algorithm description

The existing Advanced Motion Vector Prediction in HM employs multiple motion vector candidates, in which mvd coding bits can be reduced and provides the better motion search result by selecting the appropriate motion vector predictor. Although the lowest motion cost can be obtained from efficient motion vector predictor selection and zero mvd, the current fast motion vector selection scheme excludes such tests, especially in Bi-directional prediction. On the contrary, full RD comparison is used for specific modes such as skip, direct and merging modes assuming the zero mvd. In this proposal, full RD comparison scheme in normal mode using zero mvd has been tested to remove the disadvantages of those modes due to the fast encoding scheme.

For uni-directional prediction, RD cost comparison for motion vector predictors with zero mvd can be merged into initial motion vector predictor selection to be used as motion search starting point. Currently, one of motion vector predictors is selected by template matching in which interpolation filter is DCTIF and distortion measurement is SAD and from it, motion search starts. By using the same interpolation filter and distortion measurement of motion estimation, its distortion can be reused when it compares RD-costs of uni-directional multiple motion vector predictors having zero mvd.

For bi-directional prediction, the current motion search starts from the latest motion vector which may not be motion vector predictor. Due to fast motion search, motion vector predictors for both directions having zero mvds are not tested sometimes. In this proposed technique, for every combination of possible motion vector predictors in both directions, RD-costs are calculated with zero mvd and compared with others.

# Result

When compared with the anchor defined in JCTVC-C500, improved motion vector estimation for all cases has -0.3%, -0.4%, -0.6%, and -0.4% BD-rate reduction for high efficiency random access (HE-RA), low complexity random access (LC-RA), high efficiency low delay (HE-LD), and low complexity low delay (LC-LD), respectively. For HE-RA, LC-RA, HE-LD, and LC-LD, the average encoding times are 105%, 106%, 108%, and 113%, and the average decoding times are 100%, 101%, 96%, and 99%, respectively. Summarized results are shown in Table 1.

Table 1. Summary of BD-rate and encoding/decoding time for 4 configurations

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.2 | -0.2 | -0.2 | -0.2 |
| Class B | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 | -0.1 |
| Class C | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class D | -0.4 | -0.2 | -0.3 | -0.5 | -0.5 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.2 | -0.2 | -0.4 | -0.3 | -0.2 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.5 | -0.8 | -0.5 | -0.6 | -0.6 |
| Class C | -0.5 | -0.5 | -0.6 | -0.4 | -0.5 | -0.4 |
| Class D | -0.8 | -1.1 | -0.7 | -0.6 | -0.6 | -0.7 |
| Class E | -0.6 | -1.2 | -1.0 | -0.3 | -0.4 | -0.4 |
| All | -0.6 | -0.8 | -0.8 | -0.4 | -0.6 | -0.6 |
| Enc Time[%] | 108% |  |  | 113% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

# Conclusions

In this proposal, the motion vector estimation part of AMVP in the HM software is improved. This encoder-only change shows -0.4% performance improvement in average. Since it removes the unnecessary penalty of the normal mode which encodes the mvd compared to the modes which does not encode the mvd, we recommend considering it as the encoder improvements of the next version of the HM software.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).