| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D340 |
| --- | --- |

| Title: | CE1: Cross-verification of Kenwood’s experimental results of refinement motion compensation using DMVD (JCTVC-D120) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Elena Alshina Woo-Jin Han Alexander Alshin | Email: | elena_a.alshina@samsung.com wjhan.han@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

# Abstract

This contribution provides the cross-verification for Kenwood’s test contributed in CE1. Both performance and encoding/decoding time measurement are verified.

- Algorithm description

The proposed tool uses in decoder side motion vector for uni-predictive to obtain other motion vectors for mixed prediction using DMVD approach. To validate the performance, the proposed scheme has been implemented on top of TMuC 0.9.

- Test results

Proponent provided s/w for verification test in time.

After code investigation we conclude that test totally matches with algorithm description in JCTVC-C138 and satisfies requirements of CE1 [2].

We run full-frame test for 4 test cases: RA-HE, RA-LC, LD-HE and LD-LC using configuration specified in [1].

Declared by proponent performance is totally matched only decoder/encoder time ratio has slight difference. It can be explained by OS and compiler distinction analysis.

Decoder time measurement was done with/without YUV writing.

For the performance test we use cluster of x86_64 machines 3.2-3.8GHz (EM64T) running Red Hat Enterprise Linux with kernel 2.6.18 and GCC 4.1.2.

The verified tool improves TMuC performance in Random access by 1.57%(HE) / 1.16%(LC) ,

In Low delay there is small loss 0.04%(HE)/ 0.22%(LC).

Small drop with decoding time increment for Low Delay test case indicates it is better to disable this DMVD technique if both predictions come from the past. This should improve this tool performance even in RA test case.  With GPB on formally bi-directional prediction is used even both prediction come from the past (not from different time directions). But assumption embedded into this DMVD tool seams to be not valid for this not-truly bi-directional prediction.

Test results are summarized in Tab. 1

Table 1 Test results: TMuC 0.9 with Kenwood’s low complexity MDDT vs TMuC 0.9.

|  | TMuC 0.9 HM + JVC CE1 tool |  |  |  |  |  | TMuC 0.9 HM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V | Y |  |  | U | V |  |  |  |  |
|  | Class A | -1.88 |  | -2.08 |  | -2.01 | -1.56 |  |  | -1.67 | -1.57 |  |  |  |  |
|  | Class B | -1.43 |  | -1.36 |  | -1.43 | -1.09 |  |  | -0.77 | -0.70 |  |  |  |  |
|  | Class C | -1.67 |  | -1.76 |  | -1.74 | -1.19 |  |  | -1.04 | -1.17 |  |  |  |  |
|  | Class D | -1.49 |  | -1.46 |  | -1.24 | -1.01 |  |  | -0.86 | -0.64 |  |  |  |  |
|  | Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All | -1.57 |  | -1.59 |  | -1.54 | -1.16 |  |  | -0.99 | -0.93 |  |  |  |  |
|  | Enc Time[%] | 145% |  |  |  |  | 137% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 132% |  |  |  |  | 131% |  |  |  |  |  | "-"o" |  |  |
|  | Dec Time[%] | 159% |  |  |  |  | 167% |  |  |  |  |  |  |  |  |
|  |  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V | Y |  |  | U | V |  |  |  |  |
|  | Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Class B | -0.06 |  | 0.07 |  | -0.09 | 0.14 |  |  | 0.14 | 0.23 |  |  |  |  |
|  | Class C | -0.03 |  | 0.01 |  | 0.22 | 0.20 |  |  | 0.05 | 0.21 |  |  |  |  |
|  | Class D | 0.15 |  | -0.40 |  | 0.38 | 0.24 |  |  | 0.57 | 0.32 |  |  |  |  |
|  | Class E | 0.13 |  | 0.77 |  | 1.21 | 0.35 |  |  | 0.54 | 0.45 |  |  |  |  |
|  | All | 0.04 |  | 0.07 |  | 0.35 | 0.22 |  |  | 0.30 | 0.29 |  |  |  |  |
|  | Enc Time[%] | 100% |  |  |  |  | 103% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |  | 112% |  |  |  |  |  | "-"o" |  |  |
|  | Dec Time[%] | 123% |  |  |  |  | 121% |  |  |  |  |  |  |  |  |

# Conclusions

Kenwood refinement motion compensation using DMVD shows noticeable performance improvement for TMuC. From our understanding this algorithm is more efficient for B-slice. We think that this approach should be considered for further investigation. Because of performance drop with decoding time increment in Low Delay test case we recommend to proponent to disable this tool for not truly bi-directional prediction.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

[2] Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, Shun-ichi Sekiguchi, “CE1: Decoder-side motion vector derivation,” JCT-VC Document, JCTVC-C501_r1, Guangzhou, October 2010.