| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D341 |
| --- | --- |

| Title: | CE3: Cross-verification of Qualcomm’s experimental results for Luma interpolation using SIFO6 by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Elena Alshina Woo-Jin Han Jianle Chen | Email: | elena_a.alshina@samsung.com wjhan.han@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

# Abstract

This contribution provides the cross-verification for Qualcomm’s test contributed in CE3 Both performance and encoding/decoding time measurement are verified by Samsung.

- Algorithm description

The proposed tool modifies interpolation part of MC for Luma component. The motion vector accuracy is 1/4 pel so fractional part of MV may have 15 different values (refer to Fig. 1).

Fig. 1 Farctional positions for 1/4 pel MC.

For all fractional position either DCTIF6 or DIF is applied. Decision about filter type is made on encoder side by analyzing prediction error and signaled to decoder explicitly.

For DIF proponent re-uses the code of default HM interpolation filter for LC.

This is combination of

- 1D horizontal  filter for “a”, “b”,”c”: positions

- 1D vertical filter for “d”, “h”,”l”: positions

- 2D non-separable filter for central position “j”

- 1D diagonal filter for “e”, “g”,”m” and “o” positions

- Semi-sum of 2 1D diagonal filters for remaining fractional positions

DCTIF6 is 2D separable interpolation filter for all fractional positions.

DCTIF6 was tested in two different modifications.

In Test 1 proponent uses following filter coefficients :

1/4: {     9,  -37,  227,   75,  -25,    7 },

1/2: {    11,  -43,  160,  160,  -43,   11 }.

In Test 2 proponent uses following filter coefficients :

1/4:{      8,  -35,  227,   73,  -23,    6 },

1/8:{      5,  -33,  156,  156,  -33,    5 }.

In both cases filetr coefficents are represenetd using 8 bits depth.

- Test results

Proponent provided s/w for verification test at time enough for running verification test.

After code investigation we conclude that test satisfies CE3 requirements.

We tested provided s/w in 2 test configurations: Low Delay –LC, Random access - LC (according to proponent recommendation) using test conditions specified in JCTVC-C500.

Performance totally matches with proponent data, encoding/decoding time measurement difference is under noise level.

Our verification test results are summarized in Tab. 1 (Test 1) and Table (Test 2).

Table 1 Test1 results: TMuC 0.9 with Qualcomm’s SIFO6 as Luma interpolation filter’s vs TMuC 0.9.

|  | Random access LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.3 | 0.3 |
| Class B | -0.6 | 0.6 | 0.5 |
| Class C | -3.0 | -0.4 | -0.6 |
| Class D | -7.7 | -3.3 | -3.3 |
| Class E |  |  |  |
| All | -3.0 | -0.8 | -0.8 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -1.0 | -1.3 | -1.5 |
| Class C | -2.0 | -3.0 | -3.2 |
| Class D | -5.9 | -7.0 | -7.1 |
| Class E | -0.4 | -0.8 | -0.8 |
| All | -2.4 | -3.1 | -3.2 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% |  |  |

Table 2 Test2 results: TMuC 0.9 with Qualcomm’s SIFO6 as Luma interpolation filter’s vs TMuC 0.9

|  | Random access LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.4 | 0.4 |
| Class B | -1.2 | 0.5 | 0.7 |
| Class C | -2.7 | -0.3 | -0.4 |
| Class D | -6.4 | -2.7 | -2.6 |
| Class E |  |  |  |
| All | -2.9 | -0.5 | -0.5 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -2.1 | -1.2 | -1.4 |
| Class C | -2.7 | -2.5 | -2.7 |
| Class D | -6.3 | -6.1 | -6.3 |
| Class E | -1.9 | -1.5 | -1.4 |
| All | -3.3 | -2.8 | -3.0 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 103% |  |  |

# Complexity measurement

We also checked the number of operation per pixel measurement provided by proponent. As it mentioned above this algorithm is content adaptive switchable interpolation filter. Switching is perform between 2 filter types: DCTIF and DIF. For each filter separately reported number of operations totally matches with implementation in s/w.

During calculation an average number of operations the proponent assumes that each filter is used with equal probability. But as we found the hit-ration for one of used filters can reach 100% . This makes complexity estimation for this and other content-adaptive interpolation filters questionable.

# Conclusions

From these test results we confirm that SIFO6 improves HM performance in LC case by 2.8% in Test 1 and by 3.1% in Test 2. Encoding time increment is no more than 6%, decoding time increment is no more than 9%.