| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D342 |
| --- | --- |

| Title: | More improvements and results of the arithmetic coding based on probability aggregation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Tel: Email: | hurricanezhb@hotmail.com |
| Source: | Hongbo Zhu |  |  |

_____________________________

# Introduction

This arithmetic coding based on the probability aggregation (PA-Coder) is introduced in JCTVC-A027 [0] and improved in [2][3]. The basic idea is to represent the current range and the MPS and LPS probability associated to symbols in log domain. The log representation of the range is logprob, and logprobMPS and logprobLPS represent the MPS probability and LPS probability respectively.

# Three bytes-based input and output

In the PA-Coder, the condition that triggers the logprob renormalization and output/input bytes is that the logprob is bigger than or equal to a renormalization number (0x8000 for one byte based output/input, 0x10000 for word based output/input, 0x18000 for three bytes based output/input). In the renormalization, the logprob subtract the number and the logprob is renormalized to guarantee the range can’t exceed the original one [0]. But the logprob renormalization is complex and it stop the pipeline of the multi-symbol parallel decoding. To facilitate the 4-bits parallel decoding while triggering only one logprob renormalization in worst case, the renormalization number must bigger than the four times of the biggest logprobLPS or logprobMPS associated to all probabilities. As the maximum is about 22757 and 4*22757 = 91028 < 0x18000, so three bytes-based input and output is enough.

# New improved logprobMPS and logprobLPS table

The logprobMPS and logprobLPS in the old design are shifted to the left by 5 to guarantee the accuracy, but now it is found that it is not necessary. Assuming that the distribution of the range is uniform, define the LPS probability associated to each state in TMuC as follows:

probi=1256j=0255TComCABACTables::sm_aucLPSTable[i][j>>6](512+j)

According to these probabilities and considering the three bytes based input and output, the new logprobMPS and logprobLPS are as follows:

const UShort TComPACoderTables::lgprobMLps[64][2] = {

{   128,   128}, {   141,   116}, {   148,   110}, {   153,   106},

{   164,    98}, {   176,    90}, {   186,    84}, {   195,    79},

{   211,    71}, {   211,    71}, {   227,    64}, {   237,    60},

{   245,    57}, {   245,    57}, {   269,    49}, {   269,    49},

{   283,    45}, {   283,    45}, {   311,    38}, {   311,    38},

{   320,    36}, {   320,    36}, {   345,    31}, {   345,    31},

{   345,    31}, {   375,    26}, {   375,    26}, {   389,    24},

{   404,    22}, {   404,    22}, {   412,    21}, {   412,    21},

{   439,    18}, {   439,    18}, {   460,    16}, {   460,    16},

{   460,    16}, {   484,    14}, {   497,    13}, {   511,    12},

{   511,    12}, {   511,    12}, {   527,    11}, {   544,    10},

{   544,    10}, {   563,     9}, {   563,     9}, {   584,     8},

{   584,     8}, {   584,     8}, {   609,     7}, {   609,     7},

{   637,     6}, {   637,     6}, {   637,     6}, {   670,     5},

{   670,     5}, {   670,     5}, {   670,     5}, {   711,     4},

{   711,     4}, {   711,     4}, {   711,     4}, {   711,     4}};

The loss from the entropy is as follows:

p×log2p+1−p×log21−p−p×log2baselgPrbLPS−1−p×log2baselgPrbMPS, in which p is the probability of the LPS, base=0.51128. The probabilities and the loss are shown in table 1.

| Idx | Prob. | loss | Idx | Prob. | loss | Idx | Prob. | loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.48801 | 0.000415 | 21 | 0.16736 | 0.001458 | 42 | 0.0562 | 0.001031 |
| 1 | 0.46817 | 0.000617 | 22 | 0.15958 | 0.000558 | 43 | 0.05297 | 0.000761 |
| 2 | 0.44998 | 0.000196 | 23 | 0.15092 | 0.00045 | 44 | 0.05025 | 0.001055 |
| 3 | 0.42874 | 0.000259 | 24 | 0.14306 | 0.001578 | 45 | 0.04842 | 0.000834 |
| 4 | 0.40627 | 0.000623 | 25 | 0.13599 | 0.000471 | 46 | 0.0457 | 0.000991 |
| 5 | 0.38627 | 0.000312 | 26 | 0.12933 | 0.000276 | 47 | 0.04299 | 0.000461 |
| 6 | 0.36599 | 0.000375 | 27 | 0.12278 | 0.000587 | 48 | 0.04116 | 0.000543 |
| 7 | 0.34782 | 0.000315 | 28 | 0.11624 | 0.000623 | 49 | 0.03916 | 0.001228 |
| 8 | 0.33078 | 0.000826 | 29 | 0.11116 | 0.000423 | 50 | 0.03732 | 0.001486 |
| 9 | 0.31374 | 0.000443 | 30 | 0.10462 | 0.000356 | 51 | 0.03521 | 0.001832 |
| 10 | 0.29669 | 0.000694 | 31 | 0.10007 | 0.001125 | 52 | 0.03409 | 0.001968 |
| 11 | 0.28236 | 0.000646 | 32 | 0.09412 | 0.00026 | 53 | 0.03137 | 0.001504 |
| 12 | 0.26803 | 0.000427 | 33 | 0.08957 | 0.000444 | 54 | 0.03025 | 0.00181 |
| 13 | 0.25422 | 0.000977 | 34 | 0.08486 | 0.000672 | 55 | 0.02866 | 0.001779 |
| 14 | 0.24173 | 0.000477 | 35 | 0.08031 | 0.000755 | 56 | 0.02682 | 0.001214 |
| 15 | 0.2294 | 0.000167 | 36 | 0.07708 | 0.001443 | 57 | 0.02543 | 0.00147 |
| 16 | 0.2169 | 0.000523 | 37 | 0.07324 | 0.001067 | 58 | 0.02483 | 0.001794 |
| 17 | 0.20641 | 0.001078 | 38 | 0.0693 | 0.000818 | 59 | 0.02359 | 0.002478 |
| 18 | 0.19603 | 0.001475 | 39 | 0.06546 | 0.000555 | 60 | 0.02211 | 0.001596 |
| 19 | 0.18553 | 0.000794 | 40 | 0.06275 | 0.000328 | 61 | 0.02088 | 0.001544 |
| 20 | 0.17714 | 0.000741 | 41 | 0.05951 | 0.000765 | 62 | 0.01975 | 0.002163 |

Table 1

The arithmetic average loss relative to the entropy is 0.000923, it is much smaller than the original design and it is smaller than the CABAC and PIPE according to the document in the last JCT-VC meeting.

As the logprob is not shifted to left by 5, the mapping from the logprob to value is changed to follows:

(((UInt)(subinterval_division[127&logprob]<<16))>>(logprob>>7)).

# Results of the 3bytes-IO and improved logprobMLPS

Table 2 shows BD-rate results for the random access and high efficiency test case. The PA-Coder integrated in the HM 0.9 r396 and the PA-Coder is used for the entropy coding and the bit estimation for RDO. The common coding condition [4] is followed. The encoding time is similar to the anchor, but it is found that the decoding time is increased by 10%.

Table 2. BD-rate for random access and high efficiency test cases.

| Random Access high efficiency |  |  |  |
| --- | --- | --- | --- |
| Sequence | BDR-Y | BDR-U | BDR-V |
| class A average | 0.0 | 0.0 | 0.1 |
| class B average | 0.0 | -0.1 | 0.0 |
| class C average | 0.0 | 0.0 | -0.1 |
| class D average | 0.0 | 0.0 | 0.1 |
| Average | 0.0 | 0.0 | 0.0 |

# Eliminate the logprob renormalization

If the following equation is true, the logprob renormalization is not needed. When the logprob renormalization is eliminated, the multi-symbols parellel decoding will be never stopped.

Ix<<renormbits==Ix−renormnumberrenormbits=8byte−basedIO16word−basedIO

Firstly, decrease the precision of the subinterval_division from full 16-bits to full 11-bits and define the mapping from the logprob to value is as follows:

(((UInt)( subinterval_division[127&(x)]<<21))>>((x)>>7)).

Secondly, disable the three bytes-based input and output and use the word based input and output, the above equation is guaranteed to be true.

The current subinterval_division is as follows:

const UShort TComPACoderTables::subinterval_division[128] = {

2047, 2036, 2026, 2015, 2004, 1993, 1982, 1972,

1961, 1950, 1940, 1929, 1919, 1908, 1898, 1888,

1878, 1868, 1858, 1848, 1838, 1828, 1818, 1808,

1798, 1788, 1779, 1769, 1760, 1750, 1741, 1731,

1722, 1713, 1703, 1694, 1685, 1676, 1667, 1658,

1649, 1640, 1631, 1623, 1614, 1605, 1596, 1588,

1579, 1571, 1562, 1554, 1545, 1537, 1529, 1520,

1512, 1504, 1496, 1488, 1480, 1472, 1464, 1456,

1448, 1440, 1432, 1425, 1417, 1409, 1402, 1394,

1387, 1379, 1372, 1364, 1357, 1350, 1342, 1335,

1328, 1321, 1314, 1307, 1300, 1292, 1285, 1279,

1272, 1265, 1258, 1251, 1244, 1238, 1231, 1224,

1218, 1211, 1205, 1198, 1192, 1185, 1179, 1172,

1166, 1160, 1154, 1147, 1141, 1135, 1129, 1123,

1117, 1111, 1105, 1099, 1093, 1087, 1081, 1075,

1069, 1064, 1058, 1052, 1046, 1041, 1035, 1029};

The mapping from the logprob to interval is as follows:

(((UInt)(TComPACoderTables::subinterval_division[127&logprob]<<21))>>(logprob>>7))

The new logprobLPS and logprobMPS are as follows:

const UShort TComPACoderTables::lgprobMLpsNoRenorm[64][2] = {

{ 128,  128}, { 140,  117}, { 147,  111}, { 152,  107},

{ 167,   96}, { 173,   92}, { 188,   83}, { 199,   77},

{ 207,   73}, { 220,   67}, { 220,   67}, { 232,   62},

{ 248,   56}, { 248,   56}, { 263,   51}, { 273,   48},

{ 280,   46}, { 291,   43}, { 307,   39}, { 307,   39},

{ 316,   37}, { 330,   34}, { 335,   33}, { 357,   29},

{ 357,   29}, { 363,   28}, { 376,   26}, { 397,   23},

{ 397,   23}, { 397,   23}, { 422,   20}, { 422,   20},

{ 431,   19}, { 461,   16}, { 461,   16}, { 461,   16},

{ 461,   16}, { 485,   14}, { 485,   14}, { 513,   12},

{ 513,   12}, { 513,   12}, { 529,   11}, { 546,   10},

{ 546,   10}, { 565,    9}, { 565,    9}, { 587,    8},

{ 587,    8}, { 611,    7}, { 611,    7}, { 611,    7},

{ 611,    7}, { 640,    6}, { 640,    6}, { 640,    6},

{ 674,    5}, { 674,    5}, { 674,    5}, { 715,    4},

{ 715,    4}, { 715,    4}, { 715,    4}, { 715,    4}};

The corresponding loss relative to the entropy is shown in table 3. The arithmetic average loss is 0.2968%.

| Idx | Prob. | loss | Idx | Prob. | loss | Idx | Prob. | loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.48801 | 1.000415 | 21 | 0.16736 | 1.001558 | 42 | 0.0562 | 1.003622 |
| 1 | 0.46817 | 1.001115 | 22 | 0.15958 | 1.001607 | 43 | 0.05297 | 1.003844 |
| 2 | 0.44998 | 1.000983 | 23 | 0.15092 | 1.001891 | 44 | 0.05025 | 1.00353 |
| 3 | 0.42874 | 1.001389 | 24 | 0.14306 | 1.001616 | 45 | 0.04842 | 1.003787 |
| 4 | 0.40627 | 1.000875 | 25 | 0.13599 | 1.001781 | 46 | 0.0457 | 1.00354 |
| 5 | 0.38627 | 1.000869 | 26 | 0.12933 | 1.002095 | 47 | 0.04299 | 1.003657 |
| 6 | 0.36599 | 1.001182 | 27 | 0.12278 | 1.002115 | 48 | 0.04116 | 1.004399 |
| 7 | 0.34782 | 1.001043 | 28 | 0.11624 | 1.00168 | 49 | 0.03916 | 1.00444 |
| 8 | 0.33078 | 1.000957 | 29 | 0.11116 | 1.002141 | 50 | 0.03732 | 1.004387 |
| 9 | 0.31374 | 1.001127 | 30 | 0.10462 | 1.002793 | 51 | 0.03521 | 1.004023 |
| 10 | 0.29669 | 1.000989 | 31 | 0.10007 | 1.002803 | 52 | 0.03409 | 1.004334 |
| 11 | 0.28236 | 1.000859 | 32 | 0.09412 | 1.002915 | 53 | 0.03137 | 1.004871 |
| 12 | 0.26803 | 1.001099 | 33 | 0.08957 | 1.003133 | 54 | 0.03025 | 1.005158 |
| 13 | 0.25422 | 1.001139 | 34 | 0.08486 | 1.002254 | 55 | 0.02866 | 1.005434 |
| 14 | 0.24173 | 1.001125 | 35 | 0.08031 | 1.002311 | 56 | 0.02682 | 1.00642 |
| 15 | 0.2294 | 1.001645 | 36 | 0.07708 | 1.00298 | 57 | 0.02543 | 1.005917 |
| 16 | 0.2169 | 1.001894 | 37 | 0.07324 | 1.002581 | 58 | 0.02483 | 1.006119 |
| 17 | 0.20641 | 1.00176 | 38 | 0.0693 | 1.00295 | 59 | 0.02359 | 1.006419 |
| 18 | 0.19603 | 1.001693 | 39 | 0.06546 | 1.003488 | 60 | 0.02211 | 1.007053 |
| 19 | 0.18553 | 1.001611 | 40 | 0.06275 | 1.003226 | 61 | 0.02088 | 1.006108 |
| 20 | 0.17714 | 1.002066 | 41 | 0.05951 | 1.001558 | 62 | 0.01975 | 1.006003 |

Table 3

# Conclusions

# References

- Hongbo Zhu, “Arithmetic coding based on probability-aggregation and delayed-subdivision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010, Document JCTVC-A027.

- Hongbo Zhu, “Some improvement of the arithmetic coding based on probability aggregation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 2st Meeting: Geneva, CH, 21-28 July, 2010, Document JCTVC-B091.

- Hongbo Zhu, “Decoding Improvement on the PA-Coder”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration(s)

Hongbo Zhu may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).