| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D343 |
| --- | --- |

| Title: | CE3: Cross-verification of Toshiba's experimental results for NTLF with DCTIF 12/8 by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Elena Alshina Woo-Jin Han Jianle Chen | Email: | elena_a.alshina@samsung.com wjhan.han@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

# Abstract

This contribution provides the cross-verification for Toshiba’s test contributed in CE3. Both performance and encoding/decoding time measurement are verified by Samsung.

- Algorithm description

The proposed tool modifies interpolation part of MC for Luma component. The motion vector accuracy is 1/4 pel so fractional part of MV may have 15 different values (refer to Fig. 1).

Fig. 1 Farctional positions for 1/4 pel MC.

In verified test 2D separable DCTIF is used for all fractional positions.

The proponent has found interesting combination of two DCTIF with different length. For fractional positions “a”, “b”, “c”, “d”, “h”, “l” and “j” DCTIF12 is applied.  For remaining fractional positions DCTIF8 is used. This non-uniform filter length reduces complexity of interpolation for inner fractional positions.

In this test proponent used the following DCTIF12 coefficients:

1/4: {   -2,     5,   -9,    18,  -41,  229,   76,  -30,   16,   -9,     4,   -1,},

1/2: {   -1,     4,  -10,   22,  -48,  161,  161,  -48,   22,  -10,    4,   -1,}.

The bit depth of filetr coefficents is 8.

This NewDCTIF12 coefficents. They are slightly different from variant using in TMuC for HE case.

For DCTIF8 the following 6 bits representation was used:

1/4: {    -4,   16,  -40,  228,   76,  -28,   12,   -4 },

1/2: {    -4,   20,  -48,  160,  160,  -48,   20,   -4 },

Actually this filter is implemented in s/w as 8 bits filter, but theoretically bit depth can be reduced to 6 bits without performance change.

Both filters were implemented by proponent using multiplication-free form (additions and shifts only). This is beneficial both for s/w and h/w realization. And this simplifies a lot cross-verification for number of operations.

- Test results

Proponent provided s/w for verification test at time enough for running verification test.

After code investigation we conclude that test satisfies CE3 requirements.

We tested provided s/w in 2 test configurations: Low Delay –HE, Random access - HE (according to proponent recommendation) using test conditions specified in JCTVC-C500.

Performance totally matches with proponent data, encoding/decoding time measurement difference is under noise level.

Our verification test results are summarized in Tab. 1 .

Table 1 Test results: TMuC 0.9 using NTLF with DCTIF 12/ 8  as Luma interpolation filter vs TMuC 0.9.

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 |
| Class B | -0.3 | -0.2 | -0.1 |
| Class C | 0.1 | 0.0 | 0.1 |
| Class D | 0.2 | 0.3 | 0.4 |
| Class E |  |  |  |
| All | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.7 | -0.1 | -0.6 |
| Class C | -0.6 | 0.1 | 0.0 |
| Class D | -1.4 | -0.4 | -0.1 |
| Class E | -0.7 | -0.6 | -1.2 |
| All | -0.9 | -0.2 | -0.4 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 94% |  |  |

# Complexity measurement

We also checked the number of operation per pixel measurement provided by proponent. We conclude that reported number of operations matches with s/w implementation.

# Conclusions

From these test results we confirm that NTLF improves HM performance in HE case by 0.4%. Encoding time change compare to anchor is under noise level.

As it was reported by proponent the same NTLF concept with coefficients DCTIF12 from TMuC shows slightly less performance (only 0.2% performance improvement vs TMuC 0.9 HE).

Proposed by Toshiba interpolation scheme (NTLF) is an interesting trade-off between complexity and performance.