| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D344 |
| --- | --- |

| Title: | CE3: Experimental results of DCTIF by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE response |  |  |
| Author(s) or Contact(s): | Elena Alshina Jianle Chen Alexander Alshin Nikolay Shlyakhov Woo-Jin Han | Email: | elena_a.alshina@samsung.com jianle.chen@samsung.com alexander_b.alshin@samsung.com n.shlyakhov@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

- Abstract

This contribution provides the simulation results when DCTIF with new coefficients is used for Luma motion compensation TMuC 0.9. NewDCTIF12 shows 0.8% improvement for HM in  HE case. NewDCTIF6 improves performance of HM by 2.8% in LC case. NewDCTIF8 improves LC case by 4.6% with only 0.4% drop in HE case. Additionally it was shown NewDCTIF that reduction of bit-depth for coefficients of NewDCTIF8 to 6 bits brings almost no performance drop.

- Introduction

DCTIF was used for motion compensation in Samsung’s CfP response [1]. This is 2D separable interpolation filter with easy to generate filter coefficients for arbitrary number of taps. DCTIF12 was selected as default interpolation filter for HE in first HEVC test model.

Compare to DCTIF implemented in TMuC DCTIF filter coefficients were slightly changed with noticeable performance improvement for each size: 6, 8 and 12 taps.

Here we present test results for 6, 8 and 12 taps DCTIF in order to provide detail complexity / performance analysis and motivate optimal interpolation filter chose for HEVC test model .

- Algorithm description

Interpolation filter is applied in MC for fractional position values generation. Current MV accuracy for Luma component is 1/4 pel. So 15 fractional positions (refer to Fig. 1) will be interpolated.

Fig. 1 Fractional positions for 1/4 pel MC.

2D separable interpolation scheme for 1/4 accuracy. DCTIF is 2D separable interpolation filter. For fractional positions “ “a”, “b” and “c”” horizontal 1D filetr is used. For fractional positiona “d”, “h” and ”l” vertical 1D filetr is used. For remaining positions first vertical 1D filetr is applied for extended block, and then horizontal 1D filter is used. Block extension is 2M-1 (2M is number of filetr taps).

1D interpolation filetr.  The input of 1D interpolation funtion is array Ref-M+1, Ref-M+2,…,RefM of integer postion values. The output is interpolated value X, which has fractional position  between Ref0 and  Ref1.

(1)

If only one 1D filter is used for current fractional position then  .

If  two stage interpolation will be applied (inner fractional positions) then   for vertical filtration (no left shift is needed), and   for horizontal 1D filter.

DCTIF coefficients.

Because DCTIF coefficients we used for this CE are slightly different from DCTIF implemented in TMuC we’ll call filter we tested here NewDCTIF. But as it will be show below filter coefficients generation algorithm still uses DCT basis.

We tested NewDCTIF with three different sizes during this CE: 6, 8 and 12 taps. For these test we used 8 bits filter coefficients representation. Additionally NewDCTIF8 was tested using 6 bits coefficients representation.

NewDCTIF coefficients we used are summarized in Tab.1-4.

Only filter coefficients for fractional positions  are included into tables. For   fractional positions DCTIF coefficients can be obtained using mirror symmetry:

.

Tab. 1. NewDCTIF12 (8 bits) filter coefficients for 1/4 pel MV accuracy.

|  |  | “>>” | “+” |
| --- | --- | --- | --- |
| 1/4 | {-2, 5, -9, 18, -41, 229, 76, -30, 16, -9, 4, -1,} | 7 | 18 |
| 1/2 | { -1, 4, -10, 22, -48, 161, 161, -48, 22, -10, 4, -1 } | 7 | 15 |

Tab. 2. NewDCTIF8 (8 bits) filter coefficients for 1/4 pel MV accuracy.

|  |  | “>>” | “+” |
| --- | --- | --- | --- |
| 1/4 | { -4, 15, -39, 228, 74, -26, 11, -3 }, | 6 | 16 |
| 1/2 | { -3, 12, -40, 159, 159, -40, 12, -3 } | 5 | 10 |

Tab. 3. NewDCTIF8 (6 bits) filter coefficients for 1/4 pel MV accuracy.

|  |  | “>>” | “+” |
| --- | --- | --- | --- |
| 1/4 | {-1, 4, -10, 57, 19, -7, 3, -1 }, | 4 | 12 |
| 1/2 | { -1, 4, -11, 40, 40, -11, 4, -1 } | 4 | 9 |

Tab. 4. NewDCTIF6 (8 bits) filter coefficients for 1/4 pel MV accuracy.

|  |  | “>>” | “+” |
| --- | --- | --- | --- |
| 1/4 | { 8, -35, 227, 73, -23, 6 } | 5 | 12 |
| 1/2 | { 5, -33, 156, 156, -33, 5 } | 3 | 8 |

Both variants of NewDCTIF we tested here were implemented in multiplication-free from (with shifts and additions only).  The number of operations is needed for each filter representation is included to the most right columns of Tab. 1-4.

DCTIF coefficients generation method.

There are three typical steps of interpolation

- to make decomposition of input vector Ref(xm) using functional basis{k(xm)} :

pm=Ckk(xm)

- and make phase shift during inverse transform:

p()=Ckk(x0+)

Fortunately forward-inverse transform pair can be merged into the one spatial filter

For example if DCT basis is used then filter coefficients are matrix product of forward ( ) and inverse transform ()  matrices:

(1)

If input reference signal Ref = {p-M+1, p-M+2,….,p0, p1,….pM} contains some noise or corrupted due to quantization error (for example) then better results of interpolation can be achieved if smoothing will be applied before interpolation.

One can smooth reference signal by multiplying vector Ref by smoothing matrix S:

;

; .         (2)

;

The matrix S is 3-diagonal matrix (all elements out of main diagonal and two co-diagonals are zeros).

In order to minimize smoothing near interpolated point and “cut” high frequency far from it we chose smoothing parameter   depending on distance (i-) to interpolated pixel:

.                     (3)

Of course for DCT IF and smoothing combination we can pre-calculate combined filter   coefficients using the following matrix product formula:

(4)

During NewDCTIF coefficients (Tab. 1-4) generation we chose parameters  ,  solving optimization problem for minimizing deviation of frequency response curve from desired shape (refer to Fig. 2). The square of domain shadowed by blue on Fig.2  is functional depending on , . The minimum of this functional provides  ,  for NewDCTIF.

| Fig. 2. Frequency Response: left - DCTIF12 (TMuC) , right – NewDCTIF12 |  |
| --- | --- |

Formulas (1), (4) provide interpolation filter coefficients which are real (not integer) numbers. For practical application these filter coefficients are scaled by some factor ( is preferable) and rounded to integer.

Let’s  be scaled filter coefficients:

(5).

Theoretically the sum of filter coefficients is equal to 1:

.

After integerization (5) the normalization condition

(5)

might be broken due to rounding error. In order to keep normalization (5) some filter coefficients were modified 1.

- Test results

All tests described in this part was done using TMuC 0.9 (version available at 2010 Nov, 1) , encoder and decoder configurations as described in JCTVC-C500 [2]. DCTIF with N taps was enabled by setting

InterpFilterType              : 0

DIFTap                            : N

in configuration files.

We run tests for 4 different cases: Random Access High Efficiency (RA-HE), Random Access Low Complexity (RA-LC), Low Delay High Efficiency (LD-HE)), Low Delay Low Complexity (LD-LC).

DCTIF performance improvement.

First set of test results (Tab. 5-8) shows performance improvement when we change DCTIF coefficients in TMuC to NewDCTIF. Because several companies use DCTIF coefficients (4,6,8 or 12 taps) in their proposals this test we perform this test to prove that our modification was done in DCTIF coefficients noticeably improves performance.

As an anchor in all these tests DCTIF (TMuC) was used. These test results were kindly verified by TI. NewDCTIF performance with modified coefficients performance was verified by Toshiba, Sony and TI during this CE.

According to these data DCTIF12 performance was improved recently by 1.0%, DCTIF8 performance was improved by 0.9%, DCTIF6 performance was improved by 0.6%. An average Y BD-rate across 4 test cases is reported here.  NewDCTIF8(6 bits) shows almost the same performance with NewDCTIF8 (8bits).

No interpolation filter structure was changed during these tests. As the number of taps is the same there is no complexity increase in terms of multiplication and adds implementation (even some reduction, because NewDCTIF coefficients sometimes equal to 1, so no multiplication is needed).

For shifts-adds implementation the number of operation increment is less than 7%. Both NewDCTIF6, 8 and 12 were provided for cross-checkers using multiplication-free implementation.

Table 5. NewDCTIF12  vs DCTIF12(TMuC).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class B | -0.3 | -0.1 | -0.1 | -0.5 | -0.1 | -0.1 |
| Class C | -0.2 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class D | -0.5 | 0.0 | -0.1 | -0.4 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.0 | -0.1 | -0.4 | -0.1 | 0.0 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -0.4 | -0.3 | -1.9 | -0.4 | -0.4 |
| Class C | -1.3 | -0.5 | -0.5 | -2.1 | -0.3 | -0.5 |
| Class D | -2.6 | -1.4 | -1.0 | -3.4 | -0.7 | -0.7 |
| Class E | -0.7 | -0.6 | -1.0 | -1.2 | -0.2 | -0.5 |
| All | -1.4 | -0.7 | -0.7 | -2.2 | -0.4 | -0.5 |

Table 6. NewDCTIF8(8 bits)  vs DCTIF8(TMuC).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | -1.0 | 0.1 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -1.3 | -0.1 | 0.0 |
| Class C | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.2 |
| Class D | 0.4 | 0.5 | 0.4 | 0.8 | 0.5 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.2 | 0.1 | -0.3 | 0.1 | 0.1 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.7 | -0.5 | -2.2 | -0.1 | -0.1 |
| Class C | -1.8 | -1.3 | -1.2 | -0.9 | 0.1 | 0.2 |
| Class D | -3.5 | -3.8 | -3.1 | -1.1 | 0.5 | 0.8 |
| Class E | -1.3 | -0.7 | -1.2 | -2.4 | -0.9 | -0.9 |
| All | -1.9 | -1.6 | -1.4 | -1.6 | 0.0 | 0.1 |

Table 7. NewDCTIF8(6 bits)  vs DCTIF8(TMuC)..

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.2 | -0.6 | 0.1 | 0.1 |
| Class B | -0.1 | 0.1 | 0.1 | -0.9 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | -0.3 | 0.1 | 0.1 |
| Class D | -0.1 | 0.2 | 0.3 | -0.1 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.1 | 0.1 | -0.5 | 0.1 | 0.0 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -0.5 | -0.5 | -1.4 | -0.1 | 0.0 |
| Class C | -1.7 | -1.4 | -1.4 | -0.8 | -0.1 | -0.2 |
| Class D | -3.6 | -3.5 | -3.4 | -1.3 | -0.7 | -0.4 |
| Class E | -0.8 | -0.1 | -0.8 | -1.5 | -0.5 | -0.5 |
| All | -1.7 | -1.4 | -1.5 | -1.2 | -0.3 | -0.2 |

Table 8. NewDCTIF6  vs DCTIF6(TMuC)..

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | 0.1 | -1.1 | 0.1 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.8 | 0.1 | 0.1 |
| Class C | 0.0 | 0.2 | 0.1 | -0.2 | 0.2 | 0.2 |
| Class D | 0.0 | 0.3 | 0.3 | 0.0 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | -0.5 | 0.2 | 0.2 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.1 | 0.2 | -1.8 | 0.3 | 0.4 |
| Class C | 0.3 | 0.4 | 0.3 | -1.1 | 0.4 | 0.4 |
| Class D | 0.5 | 0.3 | 0.5 | -0.9 | 0.3 | 0.4 |
| Class E | -0.8 | -1.0 | -1.0 | -3.7 | -1.2 | -0.8 |
| All | 0.0 | 0.0 | 0.1 | -1.7 | 0.1 | 0.1 |

Complexity estimation for NewDCTIF

CE3 [3] uses special form for test results representation. And it was mandatory for all participants to provide information about complexity for each filter tested during this CE.

The numbers of operation per pixel ratio compare to anchor for all variation of NewDCTIF is reported in Table 9. For HE the number of shifts and additions is reported, for LC cases – the number of multiplications and additions.

Recently better implementation of DIF was found by Nokia. So here we report relative number of operations compare to current LC anchor and compare to optimized version of  DIF.

| Filter | vs HE anchor |  | vs LC anchor |  | vs optimized LC anchor |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Worst | Average | Worst | Average | Worst | Average |
| NewDCTIF6 | 68% | 58% | 155% | 129% | 210% | 153% |
| NewDCTIF8(8 bits) | 89% | 80% | 245% | 188% | 331% | 222% |
| NewDCTIF8(6 bits) | 65% | 61% | 212% | 165% | 287% | 195% |
| NewDCTIF12 | 104% | 107% | – | – | – | – |
| NewDCTIF6 restricted on enc. side | – | – | 155% | 78% | 210% | 93% |
| NewDCTIF8 (6bits) restricted on enc. side | – | – | 163% | 97% | 221% | 114% |

Two filters solution for HEVC.

First version of HEVC test model uses DCTIF12 for HE and DIF for LC cases. During this test we replaced DIF by DCTIF6 for LC case and used NewDCTIF12 for HE. Test results are summarized in Tab. 10.

We report here two different decoding time measurement: with “-o” option (first line) and without “-o” option (second line). An average performance improvement due to this change of interpolation filters is 1.8%.

If this interpolation filters scheme will be adopted then decoder should support 4 different filters: for 1/4 and 1/2-pel position, for 6 and 12 taps.

Table 10. NewDCTIF12 for HE and NewDCIF6 for LC vs TMuC 0.9.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0 | 0 | 0.1 | 0.3 | 0.9 | 0.8 |
| Class B | -0.3 | -0.1 | -0.1 | -0.7 | 0.9 | 1.1 |
| Class C | -0.2 | 0 | 0 | -2.7 | -0.1 | -0.3 |
| Class D | -0.5 | 0 | -0.1 | -6.5 | -2.5 | -2.5 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0 | -0.1 | -2.7 | -0.3 | -0.3 |
| Enc Time[%] | 101% |  |  | 108% |  |  |
| Dec Time[%] w “-o” | 90% |  |  | 117% |  |  |
| Dec Time[%]w/o “-o” | 100% |  |  | 114% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -0.4 | -0.3 | -1.4 | -1 | -1.1 |
| Class C | -1.3 | -0.5 | -0.5 | -2.6 | -2.6 | -2.7 |
| Class D | -2.6 | -1.4 | -1 | -6.3 | -6.1 | -6.2 |
| Class E | -0.7 | -0.6 | -1 | -1.4 | -1 | -1.2 |
| All | -1.4 | -0.7 | -0.7 | -2.9 | -2.7 | -2.8 |
| Enc Time[%] | 98% |  |  | 109% |  |  |
| Dec Time[%] w “-o” | 82% |  |  | 117% |  |  |
| Dec Time[%]w/o “-o” | 99% |  |  | 112% |  |  |

We summarized complexity data for this solution  in Table 11.  For LC number of operations contend using multiplication and additions form (because LC anchor is implemented with multiplications); for HE case complexity is counted using multiplication-free form (shifts and adds).

In this solution both HE and LC share the same interpolation filter structure: 2D separable filter. Two sets of filter coefficients should be supported by decoder.

Overall performance improvement for HM  by this solution is 1.81% with moderate complexity increment in LC and almost no additional complexity in HE case.

Table 11. Complexity and performance for proposed two filters solution.

|  |  | LC | HE |
| --- | --- | --- | --- |
| Complexity, Operations | Worst case / pixel | 155% | 104% |
|  | Average of operations / pixel | 129% | 107% |
| Complexity, Instructions | Worst case / block | 170% | 104% |
|  | Average instructions / block | 141% | 107% |
| Performance for each cases |  | -2.80% | -0.82% |
| Average HM performance improvement |  | -1.81% |  |

One interpolation filter solution for HEVC.

For this test we used NewDCTIF8 in both HE and LC cases. Test results are summarized in Tab. 12.

As before we report here two different decoding time measurement: with “-o” option (first line) and without “-o” option (second line).

An average performance improvement of HEVC test model due to this change of interpolation filter  is 2.1%.

Table 12.  DCTIF8 vs TMuC 0.9.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.1 | 0.7 | 1.1 | 0.8 |
| Class B | -0.1 | -0.1 | -0.2 | -1.0 | 0.9 | 1.2 |
| Class C | 0.9 | 0.4 | 0.4 | -4.6 | -0.5 | -0.7 |
| Class D | 1.8 | 1.4 | 1.3 | -10.1 | -4.0 | -3.9 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.4 | 0.4 | -4.2 | -0.8 | -0.8 |
| Enc Time[%] | 102% |  |  | 118% |  |  |
| Dec Time[%] w “-o” | 107% |  |  | 130% |  |  |
| Dec Time[%]w/o “-o” | 99% |  |  | 132% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  |  |  |  | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.1 | -0.2 | -2.2 | -1.4 | -1.7 |
| Class C | 0.7 | 1.0 | 1.1 | -5.3 | -4.2 | -4.4 |
| Class D | 0.9 | 1.0 | 2.2 | -10.7 | -9.4 | -9.6 |
| Class E | -0.4 | -0.5 | -0.7 | -1.3 | -1.7 | -1.5 |
| All | 0.2 | 0.4 | 0.6 | -4.9 | -4.2 | -4.3 |
| Enc Time[%] | 100% |  |  | 116% |  |  |
| Dec Time[%] w “-o” | 103% |  |  | 123% |  |  |
| Dec Time[%]w/o “-o” | 98% |  |  | 128% |  |  |

For some application it is important to storage intermediate data within 16 bits.  For LC case (IBDI off) all NewDCTIF8 calculation will be possible to perform within 16 bits if filter coefficients bit depth will be 6 (not 8 as in test described above).

Test results for NewDCTIF8 (6 bits) with coefficients from Tab. 3 are presented in Tab. 13.

Table 13. NewDCTIF8 (6 bits) vs TMuC 0.9.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0 | 0.1 | 0.3 | 1 | 1.1 | 0.8 |
| Class B | -0.1 | -0.1 | -0.1 | -0.7 | 1 | 1.1 |
| Class C | 0.8 | 0.3 | 0.3 | -5 | -0.5 | -0.8 |
| Class D | 1.3 | 1.1 | 1.2 | -10.8 | -4.2 | -4.2 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.4 | 0.4 | -4.3 | -0.8 | -0.8 |
| Enc Time[%] | 99% |  |  | 117% |  |  |
| Dec Time[%] | 102% |  |  | 132% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.1 | -0.2 | -1.4 | -1.5 | -1.7 |
| Class C | 0.8 | 0.9 | 0.9 | -5.2 | -4.3 | -4.8 |
| Class D | 0.7 | 1.2 | 1.8 | -10.8 | -10.3 | -10.5 |
| Class E | 0 | 0.1 | -0.3 | -0.4 | -1.3 | -1.1 |
| All | 0.3 | 0.6 | 0.6 | -4.5 | -4.4 | -4.5 |
| Enc Time[%] | 97% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 122% |  |  |

In this solution both HE and LC share the same interpolation filter structure (2D separable filter) and the same filter coefficients.

This solution requires decoder to support only 2 different filters: 8 taps filters for 1/4 and 1/2-pel positions. This is minimal filters set for 1/4 pel MC.

Overall performance improvement for HM  by this solution is 2.00% with about 60% increment of  number of operations in LC but 40% reduction of interpolation part complexity in HE case.

Table 14. Complexity and performance for proposed two filters solution.

|  |  | LC | HE |
| --- | --- | --- | --- |
| Complexity, Operations | Worst case / pixel | 212% | 65% |
|  | Average of operations / pixel | 165% | 61% |
| Complexity, Instructions | Worst case / block | 230% | 66% |
|  | Average instructions / block | 177% | 63% |
| Performance for each cases |  | -4.42% | 0.42% |
| Average HM performance improvement |  | -2.00% |  |

# Encoder restriction for DCTIF

Additionally we present test results when on ME stage inner fractional positions of MV were restricted. This mean that only following fractional MV were allowed: “a”, “b”, “c”, “d” , “h”, “l” and “j” (refer to Fig. 1). Only for one of them (position “j”) 2D filtration is needed, remaining positions use only 1D filter.

Using such kind of encoder only restriction for any 2D separable interpolation filter one can get decoder complexity on level lower than diagonal scheme with the same number of taps.

Test results in Tab. 15 show performance improvement of restricted NewDCTIF6 vs DIF in LC case.

Table 15. NewDCTIF6 with encoder restriction vs DIF in LC cases.

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | V | Y | Y | V |
| Class A | 2.2 | 1.8 | 1.5 |  |  |  |
| Class B | 0.9 | 1.2 | 1.3 | -0.1 | 0.9 | 1.2 |
| Class C | -0.5 | 0.7 | 0.5 | -0.7 | -0.7 | -0.7 |
| Class D | -3.6 | -1.1 | -1.1 | -2.9 | -2.1 | -2.4 |
| Class E |  |  |  | -0.2 | 1.1 | 1.5 |
| All | -0.5 | 0.5 | 0.5 | -1 | -0.2 | -0.1 |

| Complexity, Operations | vs DIF | vs DIF optimized |
| --- | --- | --- |
| Average of operations / pixel | 78% | 93% |

An average number of operations per pixel for restricted NewDCTIF6 is 93% even if we compare with improved DIF version. But still Restricted NewDCTIF6 shows -0.74% gain compare to DIF.

For NewDCTIF8(6 bits) encoder restriction reduces decoder complexity to numbers reported in Tab. 16. An average number of operations per pixel for is almost the same (101%) compare with improved DIF version but expected performance improvement  is at least 2%

.

Table 16. NewDCTIF8(6 bits) with encoder restriction vs DIF.

| Complexity, Operations | vs DIF | vs DIF optimized |
| --- | --- | --- |
| Average of operations / pixel | 97% | 114% |

This test is presented here to show that using the same decoder one can control decoder complexity by encoder only restriction. And 2D separable filter with mentioned restriction is even less complex than diagonal interpolation scheme. If for some application the complexity interpolation of diagonal filter scheme complexity is the only one acceptable solution then we can reach this level using decoder supporting only 2D separable filter and with better performance.

# Complexity and performance analysis

This is clear that longer taps filters provide more accurate interpolation and improve MC part. There are three graphs on Fig.3-5. We put an average number of operations per on horizontal axis and Y-BD-rate compare to anchor.  For LC case we count number of multiplications and additions, for HE case we count number of shifts and adds.

Fig. 3-4 are for LC and Fig. 5 is for HE case.

Fig 2. Complexity / performance analysis  for  NewDCTIF 6, 8 and 12 taps compare to LC anchor.

Fig 3. Complexity / performance analysis for  New DCTIF 6,8  and 12 taps compare to optimized LC anchor.

Fig 3. Complexity / performance analysis for NewDCTIF 6, 8 and 12 taps compare to HE anchor.

From this analysis NewDCTIF8(6 bits) shows good trade-off between complexity and performance for both HE and LC cases.

# Conclusions

In this proposal we provide NewDCTIF filter family and prove up to 1% performance improvements for each DCTIF 6, 8 and 12 taps. Also, it was shown that the overall number of operations could be reduced by using of encoder-only strategy in 2D FIR structure. The robustness of the NewDCTIF coefficients with the lower bit-depth was also shown, which is very useful for reducing the actual number of operations and the implementation cost due to its smaller dynamic range.

Thus, we would like recommend using NewDCTIF coefficients instead of original DCTIF coefficients in HEVC test model.

Although, in view of design and implementation simplicity, we believe that it is desirable that both configurations can share the same interpolation filter scheme, the possibility of various filtering structure proposed by other participants in CE3 could be considered according to the characteristics of the filtering structure.

# References

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

- T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C503r5, Guangzhou, Oct. 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).