| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D345 WG11 Number: m19111 |
| --- | --- |

| Title: | Summary of CE4: interpolation for MC (Chroma) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina | Tel: Email: | +82-10-3026-1305 elena_a.alshina@samsung.com |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of CE4: interpolation for MC (Chroma). Ten companies have been registered in CE4 and nine proposed tools have been evaluated on the common condition with cross-verification after all. Detailed results will be reported by proponents.

# Introduction

In this core experiment, interpolation filters for motion compensation of chroma pixels have been discussed.  There are following two purposes:

- Coding performance verification of the proposed interpolation filters for chroma components

- Complexity analysis of the proposed interpolation filters

# Document list

- Summary document

  - JCTVC-D345 [CE coordinator] Summary of CE4 : interpolation for MC (Chroma)

- Proponent's documents

  - JCTVC-D057 [Haricharan Lakshman , Heiko Schwarz , Thomas Wiegand] CE4: Chroma interpolation using MOMS based FIR filters

  - JCTVC-D223 [A. Fuldseth , G. Bjøntegaard] CE4: Interpolation filtering of chroma samples using two-stage averaging

  - JCTVC-D271[Jeong-Pil Kim , Dae-Yeon Kim , Jeongyeon Lim , Yung-Lyul Lee] CE4 : Chroma Interpolation Filtering using High Precision Filter

  - JCTVC-D324 [K. Ugur, J. Lainema (Nokia)] CE4: Results for chroma interpolation filter tests by Nokia

  - JCTVC-D347 [J. Chen, E. Alshina, W.-J. Han (Samsung)] CE4: Experimental results of DCTIF application for Chroma MC by Samsung

  - JCTVC-D349 [P. Chen, M. Karczewicz] CE4: High Precision Filtering for Chroma

- Cross-checking documents

  - JCTVC-D162 [T.Chujoh , K.Kano , T.Yamakage (Toshiba)] CE4: Verification of Samsung's 4-tap DCT-IF Chroma

  - JCTVC-D272 [Jeong-Pil Kim , Dae-Yeon Kim , Heongyeon Lim , Yung-Lyul Lee] CE4 : Cross-check of Samsung’s DCT-IF 6-tab chroma filtering

  - JCTVC-D289 [Jicheng An , Xun Guo] CE4: Crosscheck of Qualcomm’s 1/8 pel accuracy high precision filter interpolation filter for chroma by MediaTek

  - JCTVC-D318 [K. Ugur, J. Lainema (Nokia)] CE4: Cross verification of high precision chroma interpolation filter

  - JCTVC-D346 [E. Alshina, W.-J. Han, J. Chen (Samsung)] CE4: Cross-verification of HHI’s experimental results for Chroma interpolation using MOMS by Samsung

  - JCTVC-D351 [P. Chen, M. Karczewicz] CE4: Cross-check for Cisco’s Proposal

  - JCTVC-D373 [H. Lakshman] CE4: Cross check of Nokia's chroma interpolation filter JCTVC-D324

  - JCTVC-D415 [Jiwook Jung, Eunyong Son, Sehoon Yea] CE4: Cross-check report of Nokia’s Chroma interpolation filter

  - JCTVC-D420 [Lingzhi Liu, Yongbing Lin, Haoping Yu] Huawei & HiSilicon report on CE4: Experimental results of DCTIF application for Chroma MC by Samsung

- Related documents

  - JCTVC-D138 [Bin Li, Jizheng Xu, Gary J. Sullivan, Feng Wu and Houqiang Li] Rounding control for Chroma interpolation.

# Proposed Tools

Nine tools have been proposed.

Table 1  Proposals

| Proponent | Method |
| --- | --- |
| Sejong University, SK Telecom | Chroma Interpolation Filtering using High Precision Filter |
| HHI | Chroma interpolation using MOMS. |
| Nokia Variant 1 | Chroma interpolation using partially directional filters ( diagonal 1/4 and bi-linear 1/8) |
| Nokia Variant 2 | Chroma interpolation using directional filters (diagonal for both 1/4 and 1/8) |
| Cisco | Two-stage averaging filter for chroma |
| Qualcomm | 1/8 pel accuracy high precision interpolation filter |
| Samsung Variant 1 | DCTIF6 (8 bits) filter for chroma |
| Samsung Variant 2 | DCTIF4 (8 bits) filter for chroma |
| Samsung Variant 3 | DCTIF4 (6 bits) filter for chroma |

# Cross-verification

Proponents provided their source code exchange to all participants. Cross-checkers run the software independently and compared their results with proponent’s results.

Table 2. Cross checkers

| Proponent | Cross-checker |
| --- | --- |
| Sejong University, SK Telecom | Nokia |
| HHI | Samsung |
| Nokia | LG, HHI |
| Cisco | Qualcomm |
| Qualcomm | Media Tek |
| Samsung | Sejong University, Toshiba, Nokia, Huawei |

# YUV-PSNR and YUV BD-rate measure

Only chroma interpolation filter was be changed compared to the reference during this CE.

Objective rate-distortion measurements using 4-point BD-PSNR and BD-Rate were provided by proponents. BD-Rate measurements were reported for each color component separately.

Additionally YUV BD-rate measure was used inside this CE. Mean square error was averaged across all color components. Based on this all-color-components averaged PSNRYUV was computed by following formula:

Both for anchor and tested algorithm total bit-rates and PSNRYUV were estimated and 4-point YUV BD-rate was reported as additional performance measurement. All CE participants agreed that this measure uses total bits for all color components and equally takes into account distortion of each color component.

# Experimental results

The software specified by JCTVC-C500 (the version available on Nov. 1) was used for testing in this CE. Participants used common conditions and test sequences as defined by JCTVC-C500. Proposed Chroma interpolation filters was evaluated for

- low delay & high efficiency,

- low delay & low complexity,

- random access & high efficiency,

- random access & low complexity

test configurations.

## Coding efficiency

Since Table 3 shows results of coding efficiency from anchor condition, the smaller values mean better results.

Average across 4 test cases BD-rate is reported for all color components: Y, U and V; additionally YUV-BD-rate is reported. The last column contains decoding time increment reported by proponent.

Note that these values of compression ratios are rough analyses and more accurate values will reported by each proponent.

In Table 3 tests are listed in order complexity increment. This very rough complexity measure is based on number of taps for each filter only. If several proponents use the same number of taps then lower complexity assigned to filter with less bit depth coefficients. More detail for complexity estimation will be reported by each proponent.

Table 3  Summary of average across 4 test cases BD-bitrates (%) for each color component

| Proponent | Y-BD-rate | U-BD-rate | V-BD-rate | YUV-BD-rate | Dec Time[%] | # taps |
| --- | --- | --- | --- | --- | --- | --- |
| Cisco | 0.6% | 9.2% | 10.3% | 3.2% | 98% | 2 |
| Nokia Variant 1 | -0.1% | -1.5% | -1.7% | -0.6% | 97% | 6/2 |
| Nokia Variant 2 | -0.2% | -3.2% | -3.7% | -1.3% | 98% | 6/2 |
| Samsung Variant3: 4 taps 6 bits | -0.4% | -5.4% | -6.3% | -2.2% | 98% | 4 |
| Samsung Variant 2: 4 taps 8 bits | -0.4% | -5.5% | -6.5% | -2.2% | 103% | 4 |
| Sejong University, SK Telecom | -0.4% | -5.9% | -6.8% | -2.4% | 106% | 6/2 |
| HHI | -0.4% | -6.2% | -7.3% | -2.5% | 114% | 6/4 |
| Qualcomm | -0.4% | -4.7% | -5.2% | -1.9% | No data | 6 |
| Samsung Variant 1: 6 taps 8 bits | -0.5% | -6.3% | -7.5% | -2.6% | 105% | 6 |

# Conclusion

In this core experiment, nine proposals have been tested and their performances have been verified. More than 6% Chroma gain was shown in several tests during this CE.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

[2] Elena Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504r5, Guangzhou, Oct. 2010.