| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D346 |
| --- | --- |

| Title: | CE4: Cross-verification of HHI’s experimental results for Chroma interpolation using MOMS by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Elena Alshina Woo-Jin Han Jianle Chen | Email: | elena_a.alshina@samsung.com wjhan.han@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

# Abstract

This contribution provides the cross-verification for HHI’s test contributed in CE4 Both performance and encoding/decoding time measurement are verified by Samsung.

- Algorithm description

The proposed tool modifies interpolation part of MC for Chroma component. Chroma motion vector accuracy is 1/8 pel.

Proponent implemented 2D separable FIR filter for chroma.

There are two different variation of filter length: 4 taps and 6 taps. Longer taps filter is applied in HE, 4 taps filter is used for LC case.

Filter coefficients are represented using 8 bit depth.

In original JCTVC contribution [1] proponent used and described MOMS filter as combination of frame based IIR  and 2D separable FIR. For this test proponent re-designed MOMS to be simple 2D separable FIR filter. We would like to emphasize that this is quite positive tendency. New (FIR only) variant has no disadvantages typically MOMS has:

- No frame size additional buffer is required to storage IIR coefficients with high precision;

- No high precision is needed for FIR coefficients (8 bits depth is much better than 15 bits  bit depth);

- No latency due to frame based IIR operation.

Resulting FIR coefficients proponent obtained using MOMS concept are very similar to DCTIF coefficients. Which also indicates that interpolation filter we have Chroma now is very close to be optimal if two different approaches in filter coefficients generation  provide almost the same results both in terms filter coefficients and performance

- Test results

Proponent provided s/w for verification test at time enough for running verification test.

After code investigation we conclude that test satisfies CE4 requirements.

We tested provided s/w using 4 test configurations: Low Delay -HE, Low Delay -LC, Random-access-HE and Random access LC (according to CE4 agreement) and test conditions specified in JCTVC-C500.

Both Y, U, V and YUV BD-rate was estimated in this test (according to CE4 requirements [2]).

Performance totally matches with proponent data, encoding/decoding time measurement show very similar tendency with proponent data.

Our verification test results are summarized in Tab. 1.

Table 2 Test results: TMuC 0.9 with HHI’s chroma interpolation filter.vs TMuC 0.9.

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -2.5 | -3.3 | -1.0 | -0.1 | -3.7 | -3.9 | -1.5 |
| Class B | -0.2 | -5.2 | -7.0 | -2.1 | -0.1 | -6.9 | -7.0 | -2.2 |
| Class C | -0.2 | -5.6 | -5.4 | -2.1 | -0.2 | -7.8 | -7.7 | -2.8 |
| Class D | -0.3 | -7.3 | -9.1 | -2.8 | -0.2 | -9.1 | -10.8 | -3.0 |
| All | -0.2 | -5.5 | -6.6 | -2.1 | -0.2 | -7.3 | -7.8 | -2.5 |
| Enc Time[%] | 103% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 110% |  |  |  | 116% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class B | -0.5 | -3.8 | -4.9 | -1.7 | -0.9 | -8.4 | -9.8 | -3.3 |
| Class C | -0.6 | -4.3 | -4.7 | -2.2 | -0.9 | -9.9 | -9.2 | -4.5 |
| Class D | -0.8 | -8.5 | -9.1 | -3.1 | -1.3 | -9.9 | -12.0 | -4.1 |
| Class E | -0.2 | -4.4 | -4.6 | -1.5 | -0.3 | 3.8 | -2.2 | -0.2 |
| All | -0.6 | -5.2 | -5.9 | -2.2 | -0.9 | -6.9 | -8.7 | -3.2 |
| Enc Time[%] | 98% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 108% |  |  |  |

Verified tool improves TMuC performance by -0.4% / -6.2% / -7.3 %  / -2.5% cases (according to Y/U/V and YUV BD-rate measure) in average across to 4 tests.

Decoding time measurement was done with writing image out-put onto disk. An average decoding time increment is 9% in LC.

# Conclusions

MOMS was converted for FIR only form for this test. This variant of MOMS is much better for h/w implementation than IIR + FIR form and still shows very good performance.

We slightly doubt about necessity of two different Chroma interpolation filters for HE and LC. HM anchor uses bi-linear for both cases. We hope next version of HEVC test model will also use one filter solution for both configurations for design simplicity.

# References

- T. Wiegand, et. al., " Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, Dresden, Germany, April 2010

- Elena Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504r5, Guangzhou, Oct. 2010