| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D347 |
| --- | --- |

| Title: | CE4: Experimental results of DCTIF application for Chroma MC by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE response |  |  |
| Author(s) or Contact(s): | Jianle Chen Elena Alshina Woo-Jin Han | Email: | jianle.chen@samsung.com elena_a.alshina@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

- Abstract

This contribution provides the simulation results when DCTIF is used for Chroma motion compensation in TMuC 0.9. Two variants of DCTIF with 6 and 4 taps were tested. In terms of YUV BD-rate DCTIF4 provides 2.2% and DCTIF6 provides 2.6% performance improvement of first test model of HEVC without noticeable encoding/decoding time increment.

- Introduction

DCTIF was used for Chroma motion compensation in Samsung’s CfP response [1]. This is 2D separable interpolation filter with easy to generate filter coefficients for arbitrary number of taps and arbitrary MV accuracy.

Here we present test results for 4 and 6 taps DCTIF in order to motivate optimal interpolation filter chose for HEVC test model. DCTIF4 taps was tested in to variants: 8 bits coefficients and 6 bits coefficients.

- Algorithm description

Interpolation filter is applied in Chroma MC part to get fractional position value in one of 63 position (Refer to Fig. 1) .

Fig. 1 Fractional positions for 1/8 pel MC.

2D separable interpolation scheme for 1/8 accuracy. DCTIF is 2D separable interpolation filter. For fractional positions “oa”, “a”, “ob”, “b”, “oc”, “c” and “od” horizontal 1D filetr is used. For fractional positiona “oe”, “d”, “oq”, “h”,”pe”,”n” and”pq” vertical 1D filetr is used. For remaining positions first vertical 1D filetr is applied for extended block, and then horizontal 1D filter is used. Block extension is 2M-1 (2M is number of filetr taps).

1D interpolation filetr.  The input of 1D interpolation funtion is array Ref-M+1, Ref-M+2,…,RefM of integer postion values. The output is interpolated value X, which has fractional position  between Ref0 and  Ref1.

(1)

If only one 1D filter is used for current fractional position then  .

If  two stage interpolation will be applied (inner fractional positions) then   for vertical filtration (no left shift is needed), and   for horizontal 1D filter.

DCTIF coefficients for Chroma MC.

We tested 2 different DCTIF sizes during this CE: 4 taps and 6 taps. Only filter coefficients for fractional positions  are included into tables. For   fractional positions DCTIF coefficients can be obtained using mirror symmetry:

.

The DCTIF6-Chroma coefficients included into Tab. 1. These coefficients were scaled by 256 ().

Table. 1. DCTIF6_Chroma(8 bits) filter coefficients for 1/8 pel MV accuracy.

|  |  |
| --- | --- |
| 1/8 | { 1, -13, 241, 34, -8, 1, } |
| 1/4 | { 2, -22, 217, 72, -15, 2, } |
| 3/8 | { 3, -25, 185, 111, -21, 3, } |
| 1/2 | { 3, -25, 150, 150, -25, 3, } |

The DCTIF4-Chroma (8 bits) coefficients included into Tab. 2. These coefficients were scaled by 256 ().

Table. 2. DCTIF4_Chroma(8 bits) filter coefficients for 1/8 pel MV accuracy.

|  |  |
| --- | --- |
| 1/8 | { -12, 242, 30, -4,} |
| 1/4 | { -18, 218, 65, -9,} |
| 3/8 | { -19, 184, 105, -14,} |
| 1/2 | { -17, 145, 145, -17,} |

The DCTIF4-Chroma (6 bits) coefficients included into Tab. 3. These coefficients were scaled by 64 (). We also provide number of shifts and additions for these filter coefficients in multiplication-free implementation.

Table.3. DCTIF4_Chroma(6 bits) filter coefficients for 1/8 pel MV accuracy.

|  |  | “>>” | “+” |
| --- | --- | --- | --- |
| 1/8 | { -3, 60, 8, -1,} | 3 | 5 |
| 1/4 | { -4, 54, 16, -2,} | 5 | 5 |
| 3/8 | { -5, 46, 27, -4,} | 4 | 7 |
| 1/2 | { -4, 36, 36, -4,} | 2 | 4 |

Let’s us notice that DCTIF4-Chroma (6 bits) and DCTIF4-Chroma (8 bits) were generated using the same algorithm. Just scaling factor is different: 64 and 256 correspondently.

This seems to be difficult to further reduce bit-depth for DCTIF6 because for scaling factor 256 some elements are equal to 1. With less bit-depth these elements will be zero and it will be already not 6 taps filter.

For  DCTIF4_Chroma 6 bits depth also seams to be minimal: further reduction of filter_bits will turn some coefficients to be zeros  .

- Test results

All tests described in this part was done using TMuC 0.9 (version available at 2010 Nov, 1) , encoder and decoder configurations as described in JCTVC-C500. No other changes except replacement of bi-linear interpolation for Chroma MC by 2D separable DCTIF were used during these tests.

Performance measurement was done using three color components (Y,U,V) BD-rates and additionally YUV-BD-rate was estimated [2]. It calculated using total bits and average across all three color components mean square error.

Table 1. DCTIF6 applied for Chroma MC vs TMuC 0.9.

|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | YUV | Y | U |  | V |  | YUV |  |
| Class A | -0.1 | -2.4 |  | -3.2 | -1.0 | -0.1 | -3.5 |  | -4.1 |  | -1.5 |  |
| Class B | -0.1 | -4.8 |  | -6.6 | -1.9 | -0.1 | -7.5 |  | -7.8 |  | -2.5 |  |
| Class C | -0.2 | -5.1 |  | -5.0 | -2.0 | -0.2 | -8.4 |  | -8.2 |  | -3.0 |  |
| Class D | -0.3 | -6.5 |  | -8.1 | -2.5 | -0.2 | -9.6 |  | -11.8 |  | -3.3 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | -0.2 | -5.0 |  | -6.1 | -2.0 | -0.1 | -7.8 |  | -8.5 |  | -2.7 |  |
| Enc Time[%] | 106% |  |  |  |  | 103% |  |  |  |  |  |  |
| Dec Time[%] | 106% |  |  |  |  | 103% |  |  |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |  |  |
|  | Y |  | U | V | YUV | Y |  | U |  | V |  | YUV |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.5 |  | -3.7 | -4.3 | -1.6 | -0.9 |  | -9.3 |  | -10.9 |  | -3.7 |
| Class C | -0.6 |  | -4.2 | -4.5 | -2.1 | -0.9 |  | -10.9 |  | -10.1 |  | -5.0 |
| Class D | -0.8 |  | -8.8 | -10.1 | -3.1 | -1.4 |  | -10.7 |  | -13.5 |  | -4.5 |
| Class E | -0.3 |  | -4.5 | -5.0 | -1.7 | -0.3 |  | 6.2 |  | -0.6 |  | 0.3 |
| All | -0.6 |  | -5.3 | -5.9 | -2.1 | -0.9 |  | -7.1 |  | -9.4 |  | -3.5 |
| Enc Time[%] | 103% |  |  |  |  | 108% |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  | 113% |  |  |  |  |  |  |

Table 2. DCTIF4 (8 bits) applied for Chroma MC vs TMuC 0.9.

|  | Random access |  |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | YUV | Y | U | V | YUV |
| Class A | -0.1 | -2.3 | -3.0 |  | -1.0 | -0.1 | -3.7 | -4.0 | -1.5 |
| Class B | -0.1 | -4.1 | -5.8 |  | -1.6 | -0.1 | -6.9 | -7.1 | -2.3 |
| Class C | -0.2 | -4.5 | -4.4 |  | -1.7 | -0.2 | -7.9 | -7.8 | -2.8 |
| Class D | -0.3 | -5.6 | -6.9 |  | -2.2 | -0.2 | -8.8 | -10.8 | -3.0 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.2 | -4.4 | -5.3 |  | -1.7 | -0.1 | -7.2 | -7.9 | -2.5 |
| Enc Time[%] | 102% |  |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 104% |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV |  | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -2.7 | -3.4 | -1.2 |  | -0.8 | -8.1 | -9.1 | -3.2 |
| Class C | -0.5 | -3.1 | -3.1 | -1.6 |  | -0.9 | -9.8 | -8.9 | -4.5 |
| Class D | -0.7 | -6.4 | -7.2 | -2.3 |  | -1.3 | -9.6 | -11.8 | -4.0 |
| Class E | -0.2 | -3.7 | -4.4 | -1.4 |  | -0.3 | 5.8 | -1.1 | 0.2 |
| All | -0.5 | -3.9 | -4.4 | -1.6 |  | -0.9 | -6.3 | -8.2 | -3.1 |
| Enc Time[%] | 101% |  |  |  |  | 103% |  |  |  |
| Dec Time[%] | 97% |  |  |  |  | 111% |  |  |  |

Table 2. DCTIF4 (6 bits) applied for Chroma MC vs TMuC 0.9.

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -2.3 | -2.9 | -0.9 | -0.1 | -3.7 | -3.9 | -1.5 |
| Class B | -0.1 | -4.1 | -5.6 | -1.6 | -0.1 | -6.8 | -7.1 | -2.2 |
| Class C | -0.2 | -4.3 | -4.4 | -1.7 | -0.2 | -7.8 | -7.7 | -2.8 |
| Class D | -0.3 | -5.6 | -7.0 | -2.2 | -0.2 | -8.9 | -10.8 | -3.0 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -4.3 | -5.3 | -1.7 | -0.2 | -7.2 | -7.8 | -2.5 |
| Enc Time[%] | 98% |  |  |  | 95% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 94% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -2.8 | -3.1 | -1.2 | -0.8 | -8.2 | -9.2 | -3.2 |
| Class C | -0.5 | -2.8 | -3.0 | -1.5 | -0.9 | -9.7 | -8.9 | -4.5 |
| Class D | -0.7 | -5.2 | -5.9 | -2.0 | -1.3 | -9.5 | -11.5 | -4.0 |
| Class E | -0.2 | -4.1 | -4.2 | -1.4 | -0.3 | 5.0 | -1.2 | 0.2 |
| All | -0.5 | -3.6 | -4.0 | -1.5 | -0.9 | -6.4 | -8.2 | -3.1 |
| Enc Time[%] | 98% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 95% |  |  |  | 101% |  |  |  |

Decoding time in Tab. 2-3 is reported for decoder run with out-put image writing.

These test results were kindly verified by Sejong University (DCTIF6), Toshiba (DCTIF4), Nokia and Huawei (DCTIF4(6bits)).

# Complexity and performance analysis

Both DTCIF6 and DCTIF 4 applied for Chroma give over 2% performance improvement of first test model of HEVC (in terms of YUV-BD-rate) and show over 6% chroma gain (in average).

An average number of operations per pixel for DCTIF6 is 26.76, but for DCTIF4 is only 14.97. Here the number of operations in multiplications-adds form is reported.

More over DCTIF6 tested here use 8 bits filter coefficients, but DCTIF4 filter coefficients were rounded to 6 bits depth. Low bit-depth of filter coefficients makes DCTIF4more friendly for h/w implementation.

# Conclusions

Based on these results, we would like to recommend DCTIF4 (6 bits implementation) to be used as unique interpolation filter for Chroma MC in next version of the HEVC test-model.

This will improve performance for each color components by 0.4% (Y) / 5.4% (U) / 6.3% (V) which is equivalent to 2.2% YUV BD-rate reduction without noticeable encoding/decoding time change.

# References

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- Elena Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504r5, Guangzhou, Oct. 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).