| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D348 |
| --- | --- |

| Title: | CE6.a: Cross-verification of intra prediction improvement from Santa Clara University and Hisilicon (JCTVC-D026) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen Junghye Min | Email: | jianle.chen@samsung.com jh643.min@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides the cross-verification for intra prediction improvement method proposed by Santa Clara University and Hisilicon (JCTVC-D026). The cross-check results match those proposed in JCTVC-D055 with similar encoding/decoding time.

# Methods for crosschecking

In the method of JCTVC-D026, intra mode number is represented by a combination of the Group Number and the Index Number. New method of Most Probable Intra mode derivation is developed according to two parts representation of intra mode. The new method is applied to PU with size 8x8, 16x16, 32x32. Plane mode and Bi-linear mode are integrated as UIP non-directional intra modes as well as the improved mode coding method.

In another test, planar mode is added further.

# Experimental experiments

The experiments are conducted with the common conditions as defined by HEVC TM Testing Conditions in JCTVC-C500 [1]. Intra only high efficiency and low complexity configuration are tested according to CE 6 test requirement [2].

BD-rate saving of crosschecking data totally matches with proponent data, encoding/decoding time measurement show very similar tendency with proponent data. Results are summarized in Tab.1 and Tab.2.

Tab.1 Results of improved mode coding with plane and bilinear mode integration

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.2 | -0.5 | -0.3 | -0.4 |
| Class B | -0.2 | -0.2 | -0.3 | -0.5 | -0.4 | -0.4 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.2 |
| Class E | -0.4 | -0.4 | -0.5 | -0.8 | -0.8 | -0.6 |
| All | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 97% |  |  | 102% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |

Tab.2 Results of improved mode coding with plane, bilinear and planar mode integration

|  |  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  | -0.1 | -0.2 | -0.1 | 0.7 | 0.6 | 0.4 |
| Class B |  | -0.2 | -0.3 | -0.3 | 0.7 | 0.5 | 0.3 |
| Class C |  | 0.0 | -0.1 | -0.1 | 1.1 | 0.8 | 0.7 |
| Class D |  | 0.0 | 0.0 | 0.0 | 1.0 | 0.7 | 0.6 |
| Class E |  | -0.4 | -0.3 | -0.5 | 0.7 | 0.1 | 0.4 |
| All |  | -0.1 | -0.2 | -0.2 | 0.9 | 0.6 | 0.5 |
| Enc Time[%] |  | 105% |  |  | 103% |  |  |
| Dec Time[%] |  | 103% |  |  | 98% |  |  |

# Conclusion

The improved intra mode coding method achieves 0.2% BD-rate saving in intra LC configuration and 0.4% BD-rate saving in intra HE configuration when additional non-directional intra mode is jointly integrated. This kind scheme for intra mode coding should be considered for HEVC Test Model if intra mode number becomes large.

# References

- F. Bossen, “Common conditions and software reference configurations”, JCTVC-C500, Guangzhou, October, 2010.

- A. Tabatabai, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506, , Guangzhou, October, 2010.