| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D352 |
| --- | --- |

| Title: | High accuracy averaging for bi-prediction (Verification of D321) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen Sandeep Kanumuri 3240 Hillview Ave Palo Alto, CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | DOCOMO USA Labs |  |  |

_____________________________

# Abstract

This document presents results for high-accuracy averaging for bi-prediction. It is a cross-verification of the proposal described in D321.

# Introduction

Bi-prediction is typically achieved by averaging two predictions which are given at the same bit-depth as the final prediction. For example in the common low-complexity settings of the HEVC software, two predictions, one for each list, are generated with 8-bit precision per sample. They are then averaged to obtain a final prediction with the same 8-bit precision. The limited bit depth of the intermediate predictions from each list can lead to some performance degradation and an adaptive rounding method was introduced, wherein the average of the 2 predictions is computed by either rounding up or down, depending on the picture index.

The proposal described in D321 offers to maintain a 14-bit precision for each prediction signal generated in the motion compensation process, and reduce the bit depth to the desired target only after having averaged the 2 motion-compensated predictions. Adaptive rounding is disabled.

# Technical analysis

Software implementing the proposal described in D321 was provided to us by Nokia. The modifications related to the proposal, delimited by a preprocessor macro, were ported to a clean code base (version 0.9 of the common software) for analysis and testing.

As noted in the previous section, the main difference with the current HM design is that motion-compensated predictions are kept with a higher bit-depth before averaging when bi-prediction is used. This higher bit depth is set to 14 bits per sample and is intended to be independent of internal bit-depth increase (IBDI) settings. The bit depth of the prediction is reduced to the “standard” depth only after the averaging operation that is part of the bi-prediction process.

The table below (constructed based on reading the provided software) shows the bit depth that is used in various stages of the motion-compensation prediction process, where “i” represents the internal bit-depth increase, as configured by the BitDepthIncrement encoder configuration parameter.

| Stage | HM1 | Dxxx |
| --- | --- | --- |
| Reference sample | 8+i | 8+i |
| After filtering first dimension (DCT-IF) | 16+i | 16+i |
| After filtering second dimension (DCT-IF) | 8+i | 10+i (clipped) |
| After filtering when only in one dimension (DCT-IF) | 8+i | 10+i (clipped) |
| After filtering when only in one dimension, or non-separable (DIF) | 8+i | 14 (clipped) |
| After filtering along first direction (DIF) | 8+i | 15 |
| After averaging two directions (DIF) | 8+i | 14 (clipped) |
| After bi-prediction averaging | 8+i | 8+i |

It should be noted that in the case where DCT-IF is used, the provided implementation works properly only when IBDI is set to 4 bits. Indeed the intended bit depth is 14 after filtering, and the table above indicates a depth of 10+i, which works out fine when i=4 as defined in the default software configurations where DCT-IF is used. It should be further noted that this implementation detail can very easily be fixed.

# Results

The coding performance of high-accuracy averaging for bi-prediction was evaluated using the common conditions using version 0.9 of the common software. Results are tabulated below.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | -0.4 | -0.8 | -0.3 |
| Class B | 0.0 | 0.0 | -0.1 | -0.3 | -1.1 | -0.6 |
| Class C | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | -0.1 | 0.2 | 0.8 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.3 | -0.4 | 0.0 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.4 | -0.9 | 0.0 | -0.7 | -0.4 |
| Class C | 0.0 | -0.1 | -0.1 | 0.1 | -0.2 | -0.1 |
| Class D | 0.0 | -0.8 | 0.0 | -0.1 | 0.7 | 0.7 |
| Class E | -0.1 | -3.8 | -3.2 | 0.6 | 0.6 | -1.4 |
| All | 0.0 | -1.1 | -0.9 | 0.1 | 0.0 | -0.3 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |

In the high-efficiency case little difference is observed. Intuitively this makes sense as the precision is only slightly increased from 12 to 14 bits for representing motion-compensated predictions from each list. There seems however to be a benefit for the chroma component, where a significant difference was recorded for the low-delay setting, for class E in particular.

In the low-complexity case results indicate that high-precision averaging performs similarly to adaptive rounding.

In all cases a slight increase in encoding and decoding run times was recorded.

# Conclusion

Document D321 proposes to replace rounding control by high-accuracy averaging for bi-prediction. Results presented in D321 have been verified and show that both solutions offer similar coding performance.