| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D354 |
| --- | --- |

| Title: | CE7: Cross-verification of Qualcomm’s MDDT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | cross-check |  |  |
| Author(s) or Contact(s): | Elena Alshina Vadim Seregin Woo-Jin Han | Email: | elena_a.alshina@samsung.com v.seregin@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

# _____________________________

# Abstract

This contribution provides verification test results for Qualcomm’s MDDT in TMuC 0.9.

- Algorithm description

There is brief summary of verified tool is in Table 1.

Table 1. Brief summary of Qualcomm’s MDDT.

| Applied TU sizes | 4, 8, 16 |
| --- | --- |
| Number of additional transform core for each size | 4 |
| TMuC Quantization matrix re-use | No |
| TMuC Transform re-use | No |
| Scan modification | Yes |
| integer arithmetic | 64 bits |

The proposed tool uses alternative transform. There are 4 types of transform additionally implemented in TMuC. UDI directions are separated into 4 groups. According to the group one of alternative transforms core is applied to Intra residual.

Total size of additional arrays introduced into TMuC for Qualcomm’s alternative transform and quantization is show in Table 2.

Table 2. Additional memory usage for transform part of MDDT.

| Intra TU size | Matrix size | Number of matrices |  |  |
| --- | --- | --- | --- | --- |
|  |  | Transform | Quantization | De-quantization |
| 4x4 | 16 | 8 | 6 | 12 |
| 8x8 | 64 | 8 | 6 | 12 |
| 16x16 | 256 | 8 | 6 | 12 |
| Total | 8736 |  |  |  |

In the original MDDT proposal statistical scanning was used introducing noticeable complexity especially in the decoder side. However, in current MDDT realization, adaptive coefficient scanning (ACS) is exploited replacing previous statistical approach. As part of ACS, three scanning patterns: HM default, horizontal and vertical scans are used, and, instead of explicit signaling, fixed table mapping between scanning index and intra mode is applied for 4x4 and 8x8 transform sizes with intra predicted blocks. In the rest of cases, including inter blocks, only HM default scan is exploited. This fixed table solution doesn’t introduce additional complexity and positively contributes to the performance of MDDT similar to the previous statistics based adaptive scanning.

In this MDDT proposal adaptive scanning is implemented for high-efficiency only.

- Test results

Proponent provided s/w at very late stage. So our verification test is not finished yet. Partially available results are represented in Table 3. This contribution will be updated after verification test will be finished.

Table 3 Test results: TMuC 0.9 with Qualcomm’s MDDT vs TMuC 0.9.

|  |  | HE Intra |  | HE Random access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.0 | -3.6 | -3.6 | -1.3 | -0.8 | -0.6 |
| Class B | -1.6 | -2.3 | -2.3 | -0.8 | -0.8 | -0.8 |
| Class C | -2.3 | -2.3 | -2.3 | -1.1 | -0.9 | -1.0 |
| Class D | -2.3 | -2.1 | -2.1 | -0.9 | -0.7 | -0.4 |
| Class E | -3.1 | -3.6 | -3.4 |  |  |  |
| All | -2.3 | -2.6 | -2.6 | -1.0 | -0.8 | -0.7 |
| Enc Time[%] | 112% |  |  | 106% |  |  |
| Dec Time[%] | 109% |  |  | 103% |  |  |

Verified tool improves TMuC performance in HE Intra-only configuration by 2.3% and in HE random-access configuration by 1.0%.

Decoding time increment by 9% in HE intra-only with 3% in HE random-access seams to results of 16x16 alternative transform. Encoding time increment is 12% in HE intra-only configuration with 6% in HE random-access according to our measurement.

# Conclusions

Low complexity MDDT from Qualcomm shows nice performance improvement for TMuC with moderate encoder or decoder running time increment.

Actually MDDT is still two tools combination: alternative transform and scan modification. Scan modification part of MDDT was tested in CE 11. Only mode dependent scan provides 0.9% gain.

The size of arrays additionally introduced into TMuC for alternative transform part of MDDT is very large compare to other proposals in CE7.

In order to study advantages of alternative transform clearly we suggest separately investigating transform core apart from scanning part, since both parts can be improved independently.