| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D355 WG11 Number: m19123 |
| --- | --- |

| Title: | CE9: Cross-check report from Qualcomm on Modified AMVP and PU-Merge (CE9.3.1c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Core Experiment report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a report of an experiment CE 9.3.1c in core experiment 9 (CE9) on PU based merge as described in JCTVC-C509.

# Introduction

CE9 divide motion vector coding in three categories: prediction unit based Merge and Skip tools, the adaption of the set of motion vector predictors at the slice level, and the temporal motion vector memory compression. This contribution reports the experiment performed respect to subtest 9.3.1c. In 9.3.1c, CU-level Skip and Direct modes and PU-level Merge are enabled. Besides, a modified AMVP is applied on Skip and Direct modes. The modified AMVP only allows MVPs from co-located and corner locations and disable zero vector as an MVP candidate. If the zero vector is not used anymore, the signalling is modified. If the motion vector candidate lists for Skip/Direct are both empty, the direct mode is not signalled, this reduces the number of bits used for the prediction mode signalling. If only one of the two lists is empty, the skip_flag and inter_pred_idc syntax elements are not signalled because they can be inferred from the two lists. For PU-level Merge, only left and above merge modes are allowed.

# Test Results

The experiments are performed under common test condition [2]. The following table summarizes the obtained results for the High Efficiency and Low Complexity profiles:

|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.5 |  | 0.1 |  | 0.3 | -0.8 |  | -0.3 |  | -0.2 |
| Class B | -0.5 |  | 0.2 |  | 0.2 | -0.8 |  | -0.5 |  | -0.4 |
| Class C | -0.4 |  | 0.0 |  | 0.0 | -0.7 |  | -0.3 |  | -0.3 |
| Class D | -0.6 |  | -0.2 |  | 0.1 | -0.8 |  | -0.4 |  | -0.3 |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.5 |  | 0.0 |  | 0.1 | -0.8 |  | -0.4 |  | -0.3 |
| Enc Time[%] | 88% |  |  |  |  | 96% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.1 | 0.3 |  | 0.2 |  | 0.4 | 0.0 |  | -0.1 |  |
| Class C | -0.4 | -0.2 |  | -0.2 |  | 0.2 | 0.0 |  | 0.0 |  |
| Class D | -0.4 | -0.8 |  | -0.3 |  | 0.1 | 0.1 |  | 0.0 |  |
| Class E | -1.3 | -2.5 |  | -2.5 |  | -0.8 | -0.4 |  | -0.6 |  |
| All | -0.5 | -0.6 |  | -0.5 |  | 0.1 | 0.0 |  | -0.1 |  |
| Enc Time[%] | 94% |  |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 102% |  |  |  |  |

# References

- J. Jung and B. Bross, “Core Experiment 9: Motion Vector Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C509, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.