| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D356 |
| --- | --- |

| Title: | Three digits to speed up the reference encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto, CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | DOCOMO USA Labs |  |  |

_____________________________

# Abstract

In this proposal very minor modifications to the software are suggested wherein 3 digits are modified in the code of the common software. The proposed modifications result in little coding performance loss (<0.5%) while significantly speeding up the encoder (by up to 37%).

# Introduction

Even though tremendous improvements were achieved in terms of encoding times between versions 0.8 and 0.9 of the common software, the encoder remains unreasonably slow, taking more than 24 hours to encode some of the test points defined in the common test conditions. Simple changes are suggested to further speed up the encoder without incurring a significant coding performance penalty.

# Proposed encoder modifications

## Disabling inter NxN mode except for 8x8 CUs

Splitting a CU into 4 smaller units can be achieved in 2 ways: either split it into 4 CUs or split it into 4 PUs. It may be argued that this design generates superfluous redundancy in the syntax (although there is not a complete overlap given the available TU splitting options for each case). It is suggested to disable the inter NxN-split mode wherein a CU of size 2Nx2N is split into four PUs of size NxN, except for the smallest CU size of 8x8. This is achieved by changing the preprocessor macro HHI_DISABLE_INTER_NxN_SPLIT from 0 to 1.

## Search range for bipred refinement

In the motion estimation process for bi-predicted partitions, a refinement search is performed for a second list wherein the vector for a first list is kept constant. The implemented search method is a full-search method and has a range of 8 samples. It is suggested to reduce this range to 4 samples.

## Search step for full search

In the motion estimation process a full search is conducted under certain conditions. A variable iRaster defines the step used in this search. It is suggested to increase the value of this variable from 3 to 5, thereby reducing the number of search points in the motion estimation.

# Results

Experiments were conducted to determine the trade-offs between encoding time and coding performance offered by the proposed changes. A very recent version of the software (essentially version 1.0) was used for the experiments.

Reference: revision 442, 0.9-hm branch, default test configurations (as per JCTVC-C500)

Test: revision 442 with all proposed changes, 0.9-hm branch, default test configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.6 | 0.5 | 0.3 | 0.6 | 0.4 |
| Class B | 0.2 | 0.4 | 0.4 | 0.2 | 0.2 | 0.1 |
| Class C | 0.4 | 0.5 | 0.6 | 0.4 | 0.4 | 0.5 |
| Class D | 0.2 | 0.3 | 0.4 | 0.2 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.4 | 0.5 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 80% |  |  | 66% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.4 | 0.1 | 0.2 | 0.2 | 0.3 |
| Class C | 0.2 | 0.3 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.1 | 0.0 | 0.7 | 0.1 | 0.0 | 0.2 |
| Class E | 0.0 | -0.3 | 0.1 | 0.3 | 0.3 | 0.2 |
| All | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 76% |  |  | 63% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Reference: version 0.9, default test configurations (as per JCTVC-C500)

Test: revision 442 with all proposed changes, 0.9-hm branch, default test configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.7 | 0.7 | 0.3 | 0.6 | 0.5 |
| Class B | 0.2 | 0.5 | 0.5 | 0.2 | 0.3 | 0.1 |
| Class C | 0.4 | 0.4 | 0.5 | 0.4 | 0.5 | 0.5 |
| Class D | 0.2 | 0.4 | 0.5 | 0.2 | 0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.5 | 0.5 | 0.3 | 0.4 | 0.3 |
| Enc Time[%] | 76% |  |  | 60% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.4 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class C | 0.2 | 0.3 | 0.2 | 0.2 | 0.0 | 0.2 |
| Class D | 0.1 | 0.0 | 0.7 | 0.0 | 0.1 | 0.1 |
| Class E | 0.0 | -0.3 | 0.1 | 0.2 | 0.4 | 0.3 |
| All | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 74% |  |  | 60% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

Reference: revision 442, 0.9-hm branch, default test configurations (as per JCTVC-C500)

Test: revision 442 with proposed change 2.1, 0.9-hm branch, default test configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 | 0.0 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 91% |  |  | 88% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.1 |
| Class D | 0.1 | -0.7 | 0.2 | 0.1 | 0.0 | 0.0 |
| Class E | 0.0 | -0.2 | -0.3 | 0.1 | -0.2 | -0.1 |
| All | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 89% |  |  | 87% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Reference: revision 442, 0.9-hm branch, default test configurations (as per JCTVC-C500)

Test: revision 442 with proposed changes 2.1 and 2.2, 0.9-hm branch, default test configurations

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 |
| Class B | 0.1 | 0.2 | 0.3 | 0.1 | 0.1 | 0.0 |
| Class C | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.0 | 0.2 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 | 0.0 |
| Enc Time[%] | 84% |  |  | 74% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.5 | 0.2 | 0.1 | 0.2 | 0.2 |
| Class C | 0.2 | 0.3 | 0.1 | 0.1 | 0.1 | 0.2 |
| Class D | 0.1 | -0.5 | 0.4 | 0.1 | 0.1 | 0.3 |
| Class E | 0.1 | -0.4 | -0.1 | 0.2 | 0.4 | 0.2 |
| All | 0.1 | 0.0 | 0.2 | 0.1 | 0.2 | 0.2 |
| Enc Time[%] | 77% |  |  | 66% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Additional data

An additional experiment was conducted in which the numbers of reference frames are doubled in the encoder configuration files (NumOfReference=4, and NumOfReferenceB_Lx=2) in addition to the changes proposed above. The results below were obtained. While the encoding times are back to were they where before the proposed changes, results show that the performance of the reference encoder can be significantly improved without significant impact on encoding time.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -2.0 | -2.1 | -1.4 | -1.3 | -1.3 |
| Class B | -2.6 | -2.2 | -2.2 | -2.1 | -0.9 | -0.6 |
| Class C | -2.0 | -1.2 | -1.3 | -2.5 | -1.4 | -1.2 |
| Class D | -1.2 | -0.8 | -0.4 | -3.0 | -1.0 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -1.9 | -1.5 | -1.5 | -2.3 | -1.1 | -0.9 |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -3.2 | -3.5 | -4.2 | -3.9 | -4.2 | -5.0 |
| Class C | -3.3 | -3.1 | -3.2 | -5.0 | -5.9 | -6.0 |
| Class D | -4.1 | -4.9 | -4.1 | -6.9 | -8.0 | -8.1 |
| Class E | -5.4 | -8.0 | -9.4 | -5.3 | -3.8 | -5.3 |
| All | -3.9 | -4.6 | -4.9 | -5.2 | -5.5 | -6.1 |
| Enc Time[%] | 102% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

In JCTVC-D093 it is reported that significant gains may be achieved by using hierarchical QP assignment in the low-delay configurations. Such a configuration was tested with low-delay configurations by setting RateGOPSize to 4 in addition to increasing the number of reference frames. For proper operation a bug related to the reference list construction had to be fixed in the encoder. It should be noted that the encoder and decoder run-time reductions can be primarily attributed to reductions in bit rates.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -9.9 | -31.2 | -31.7 | -9.8 | -27.1 | -27.1 |
| Class C | -12.9 | -28.1 | -27.2 | -12.8 | -27.8 | -26.5 |
| Class D | -13.5 | -31.3 | -28.5 | -14.4 | -31.4 | -28.5 |
| Class E | -17.2 | -34.2 | -33.6 | -16.8 | -26.2 | -25.9 |
| All | -12.9 | -31.0 | -30.1 | -13.0 | -28.2 | -27.1 |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 86% |  |  | 90% |  |  |

Additional BD-rate gains may be obtained by further setting BQP to 1, thereby reducing the differences between QP values assigned to each picture.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -11.4 | -26.0 | -26.6 | -11.2 | -22.5 | -23.0 |
| Class C | -13.0 | -23.3 | -22.7 | -13.4 | -23.4 | -22.6 |
| Class D | -13.8 | -26.1 | -24.4 | -14.9 | -26.8 | -24.6 |
| Class E | -16.6 | -29.0 | -29.1 | -16.1 | -21.7 | -21.5 |
| All | -13.4 | -25.9 | -25.5 | -13.6 | -23.6 | -23.0 |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 91% |  |  | 93% |  |  |

# Conclusion

Results presented in this proposal show that it is possible to significantly decrease encoding times of the reference encoder (by up to 37%) without incurring a significant penalty in coding performance (<0.5%). This was achieved by changing 3 digits in the source code. It is recommended to include those modifications in the next software release. Additional changes to the default encoder configurations may be considered depending on which trade-off between encoding time and coding performance is most desirable.

It is expected that multi-character changes to the motion estimation implementation in the reference encoder (in particular to non-numeric sections thereof) would offer even better trade-offs between encoding time and coding performance. Research on the topic of motion estimation should thus be encouraged, as availability of reasonably fast software providing excellent coding performance is essential to the success of the HEVC project.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).