| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D357 |
| --- | --- |

| Title: | CE7: Experimental results of fast ROT by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE response |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin Felix C. Fernandes Yinji Piao Woo-Jin Han | Email: | elena_a.alshina@samsung.com alexander_b.alshin@samsung.com felix.f@sta.samsung.com yj1003.piao@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides the simulation results when Rotational transform (ROT) is enabled in TMuC 0.9 against the TMuC 0.9 anchor results. In Intra-Only test case, about 2% (HE) / 3% (LC) gain is provided by ROT. In Random-Access test scenario ROT gain is over 1% on average. No additional changes (like scan modification, for example) typically improving alternative transform performance was applied in this test.

- Introduction

It is well known that the DCT basis functions are sub-optimal for some types of residuals. For example, residual signals with strong directional components cannot be represented efficiently with DCT basis vectors. Typically, alternative transforms work efficiently for intra residuals. There are several known alternative transforms: Mode Dependent Directional Transform (MDDT) [1], DCT/DST [2] switch and Rotational transform (ROT) [3].

The ROT was tested in TE12 and showed more than 2% bit-saving for Intra-Only test and about 1% bit-saving in Random Access (RA) test case. Decoding time increment due to ROT is on a noise level (~5%), encoding time in RA test scenario was also acceptable, but encoding time increment due to ROT was about 2.5 times in Intra test. This overhead appeared large and was mainly due to unnecessary encoder-side operations. In this contribution, we modify encoder-side ROT search process to avoid these unnecessary operations and thus obtain a fast ROT. Section 3 describes the fast ROT test results.

- Algorithm description

In contrast to other known alternative transforms, the ROT does not replace the main transform core. It is applied as a secondary transform for only the low frequency part of transform coefficients matrix (Fig. 1).

Fig 1. ROT application on decoder side.

For 4x4 blocks, the 4x4 inverse ROT (invROT4) is applied. In all other cases, the 8x8 inverse ROT (invROT8) is applied to the 8x8 upper-left square of transform coefficients matrix.

So the input of invROT8 is a 4x4 square matrix with transform coefficients after de-quantization. The input of invROT8 is an 8x8 square matrix with low-frequency part transform coefficients after de-quantization

Mathematically, the ROT is a separable transform. Therefore, the forward 4x4 ROT (ROT4) is matrix left-hand side multiplication of input matrix with “matrix of horizontal rotation” (RHorizontal) and then right-hand side multiplication with “matrix of vertical rotation” (RVertical).

For the ROT4, these mathematical operations are represented in Fig 2. Shadowed squares indicate non-zero elements of RHorizontal and RVertical.

1

1





=

RHorizontal                   input                       RVertical         =         rotated output

Fig. 2. The scheme of ROT4 process.

Taking into account the a-priori, known, structure of the ROT matrix (some elements of ROT matrices are zeros and one non-zero element is equal to 1) the total number of operation for ROT is essentially smaller than for two matrix multiplications. Exact number of operations for ROT process is summarized in Table 1.

Table 1 Number of operations for ROT.

|  | Inside loop |  | per block |  | total | per pixel |
| --- | --- | --- | --- | --- | --- | --- |
|  | “*” | “+” | “*” | “+” |  |  |
| ROT4 | 18 | 12 | 72 | 48 | 120 | 7.5 |
| ROT8 | 36 | 24 | 288 | 192 | 480 |  |
| Two 4x4 matrix products | 32 | 24 | 128 | 96 | 224 | 14 |
| Two 8x8 matrix products | 128 | 112 | 1024 | 896 | 1920 | 30 |

The Rotational transform was designed to allow partial exchange of energy between columns/rows of transform coefficients matrix in order to get better energy compaction and minimize quantization error. Partial energy exchange between columns / rows is controlled by angle parameters k (1).

(1)

For example, if we set  1=/2, 2=0, 3=0 then RVertical will  switch 1 and 2 columns of inputted matrix  with change of sign of 2nd column. By setting 1=, 2=0, 3=0 one gets RVertical  which will change sign of all elements. If 1=2=3=0 then the ROT does not change the input.

Because we assume that the main transform is close to optimal and only small modification of transform coefficients matrix is needed, the parameters k in (1) were chosen to be near 0 or  which correspond to small absolute value change with possible sign change. From (1) it is clear that ROT is an orthogonal transform. Therefore Parseval’s theorem guarantees that total transform-coefficients energy after the ROT is preserved.

Exact values of k are not important because in hardware and software implementations, both RVertical and RHorizontal are pre-calculated, scaled and rounded to integer. The set of k was not changed during this CE. Four sets of non-trivial k provide 4 non-trivial ROT transforms. We also define the “zero” ROT, in which trivially, k=0, so that the ROT is effectively skipped and no transform-coefficients are changes after the DCT. These 5 ROT transforms constitute the dictionary of alternative transforms for this CE. One of these possibilities is chosen on encoder side and signaled to decoder explicitly by encoding indexROT = 0,…,4 for each TU.

If ROT is applied, then no modification of main transform core or quantization part is needed because the ROT re-uses the same quantization matrices that are already implemented in HM).

The entire ROT dictionary with 4 non-trivial alternative transforms requires storage of 72 additional integers for invROT4 and 144 additional integers for invROT8. So in total, 216 integers with absolute values less than 256 should be stored. This is enough for 4 alternative transforms applied for arbitrary TU sizes (from 44 up to TUsizeMax TUsizeMax) and all QP.  For comparison, the de-quantization matrix for only 44 transform core in HM contains 96 elements. Therefore, the memory usage by ROT compared to other alternative transforms and even the main transform core in HM is relatively small.

The ROT is applied for intra residuals only.

On encoder side, all members of ROT dictionary are applied and the best member for the current TU is chosen. In the previous ROT version on encoder side, the entire TU encoding process was repeated inside the ROT search loop: prediction, forward main transform, forward ROT, quantization, de-quantization, inverse ROT, and inverse main transform (7 steps in total). The best ROT transform was decided based on TU Rate-Distortion (RD) cost.

Because the ROT is an orthogonal transform, the distortion can be estimated in frequency domain using Parseval's identity. We exploit this advantage in RDOQ to reduce computational complexity: the best quantization is decided using distortion estimation in frequency domain.  For the test in this CE, the ROT encoder search process was re-designed and merged with RDOQ. In the resulting fast ROT version, only 2 steps are repeated within the ROT search loop: forward ROT and quantization.

No ROT kernel modification was applied during this CE.

- Test results

The fast ROT was implemented on top of TMuC 0.9 (version available at 2010 Nov, 1) and tested using encoder and decoder configurations as described in JCTVC-C500 [?]. No additional change (for example, scan modification) which typically has synergy with alternative transforms was enabled during this test.

Test results are summarized in Table 2. In Intra-Only test, the ROT shows about 2% gain for HE and about 3% gain for LC configuration. For Random-Access test scenario, the ROT gain is over 1% on average. Note that for this scenario, the ROT is applied for intra residuals only.

Table 2 Test results: TMuC 0.9 with fast ROT on vs TMuC 0.9.

|  |  |  | Intra |  |  |  |  |  | Intra-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V | Y |  |  | U |  | V |
| Class A | -2.3 |  | 0.4 |  | 0.2 | -3.5 |  |  | 0.7 |  | 0.5 |
| Class B | -1.8 |  | 1.2 |  | 1.1 | -2.9 |  |  | 1.2 |  | 1.1 |
| Class C | -1.8 |  | 1.6 |  | 1.5 | -2.8 |  |  | 1.4 |  | 1.4 |
| Class D | -2.0 |  | 1.6 |  | 1.6 | -2.9 |  |  | 1.6 |  | 1.6 |
| Class E | -1.7 |  | 0.5 |  | 0.5 | -2.8 |  |  | 0.4 |  | 0.5 |
| All | -1.9 |  | 1.2 |  | 1.1 | -2.9 |  |  | 1.2 |  | 1.1 |
| Enc Time[%] | 159% |  |  |  |  | 219% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  | 105% |  |  |  |  |  |
|  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y | U |  | V |  |  | Y | U |  | V |  |
| Class A | -1.0 | 0.6 |  | 0.6 |  |  | -1.4 | 0.5 |  | 0.3 |  |
| Class B | -0.9 | 1.0 |  | 0.9 |  |  | -1.2 | 0.6 |  | 0.5 |  |
| Class C | -1.0 | 0.8 |  | 0.8 |  |  | -1.2 | 0.5 |  | 0.5 |  |
| Class D | -0.9 | 1.2 |  | 1.3 |  |  | -0.9 | 0.6 |  | 0.7 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | -0.9 | 0.9 |  | 0.9 |  |  | -1.1 | 0.6 |  | 0.5 |  |
| Enc Time[%] | 104% |  |  |  |  |  | 103% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  |  | 101% |  |  |  |  |

Decoding time increment due to ROT is on a noise level (no more than 5%). Encoding time increment in RA test scenario is negligible. Encoder complexity in Intra only test is 1.6 and 2.2 for HE and LC cases, respectively.

While evaluating the encoder complexity, it is important to remember that Intra-Only complexity in real-world implementations will be significantly lower than x1.6 and x2.2 because the TMuC software does not accurately simulate actual implementation platforms. In practice, significant encoding-time reduction will be achieved because actual implementations exploit parallelism. To understand this reduction, we address hardware and software implementations separately.

In hardware implementations, an Application-Specific Integrated Circuit (ASIC) is designed for a specific video encoder. Now, for the ROT with dictionary-size of N, the encoding-time overhead arises from a loop with (N+1) iterations. In the first iteration, the RD cost from the trivial “zero” skipped ROT is estimated. In the remaining N iterations, the RD costs from each non-trivial ROT in the dictionary are estimated. To reduce encoding time, ASICs are usually designed to implement loop iterations simultaneously in parallel, whenever each iteration is independent of other iterations. For the ROT implementation, each RD cost estimate is clearly independent of all others. Therefore, (N+1) separate ASIC modules can be implemented to evaluate the (N+1) RD costs simultaneously. Hence the ROT encoding-time overhead would reduce by a factor of (N+1). Thus, for N = 4, we expect the Intra-HE encoding time to reduce from x1.6 to x1.1 and similarly, the Intra-LC encoding time to reduce from x2.2 to x1.3.

In software implementations, Single Instruction, Multiple Data (SIMD) platforms are widely used to exploit parallelism for encoding-time reduction. On a SIMD platform, several processing units are available to perform independent operations simultaneously, in parallel. Therefore, since the (N+1) iterations in the ROT search are independent, each iteration can be assigned to an individual processing unit. These (N+1) processing units would compute (N+1) independent RD costs in parallel, thus reducing the ROT encoding-time overhead by a factor of (N+1). Therefore, for software implementations with N=4, we also expect the Intra-HE and Intra-LC encoding times to reduce from x1.6 to x1.1 and from x2.2 to x1.3, respectively.

Note that the preceding hardware and software encoding-time reductions are easily achieved by exploiting parallelism, because each iteration of the ROT search loop is independent of all other iterations. In addition to reductions from parallelism, we also expect encoding-time reduction from hardware and software optimizations made within each parallelized loop iteration.  Since the current TMuC implementation has no such optimizations, we anticipate that the Intra-HE and Intra-LC encoding-times in an actual hardware or software implementation would be less than 1.1x and 1.3x, respectively.

Additional complexity reduction for ROT is possible if lifting (described in JCTVC-C-096) is applied. Lifting was not tested in this CE in order to minimize changes in ROT compared to the original design. Only the encoder search was modified.

However, with lifting, significant hardware complexity reduction is possible, as explained here briefly. In a lifting factorization of the ROT, each ROT matrix in the dictionary is first factored into Givens rotation matrices. These Givens rotation matrices are then each replaced by a triplet of integer lifting matrices. While retaining overall ROT transform invertibility, the multipliers in each lifting matrix can be approximated so that fewer hardware elemental adders are required in a ROT implementation. From JCTVC-C096, with BD-rate of -0.003%, the lifting factorization obtained 46% and 21% reduction in elemental-adder count of forward and inverse ROT implementations, respectively.

# Restricted ROT search test results

Additionally, we present test results when ROT search is enabled for only the Best UDI mode. This encoder-only modification cuts half of ROT gain with 3 times encoder complexity reduction in HE and 6 times encoder complexity reduction in LC. This modification is applied only if all frames are encoded as Intra (no performance/complexity change in RA test case).

Intra only test results for this modification are summarized in Table 4.

Table 4 Test results: TMuC 0.9 with fast ROT restricted on encoder side vs TMuC 0.9.

|  |  |  | Intra |  |  |  |  |  | Intra-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V | Y |  |  | U |  | V |
| Class A | -1.3 |  | 0.8 |  | 0.7 | -2.1 |  |  | 0.7 |  | 0.5 |
| Class B | -1.2 |  | 1.2 |  | 1.0 | -1.6 |  |  | 1.1 |  | 0.9 |
| Class C | -0.9 |  | 1.3 |  | 1.2 | -1.3 |  |  | 1.5 |  | 1.5 |
| Class D | -0.9 |  | 1.3 |  | 1.3 | -1.3 |  |  | 1.7 |  | 1.7 |
| Class E | -1.0 |  | 0.7 |  | 0.8 | -1.6 |  |  | 0.3 |  | 0.5 |
| All | -1.1 |  | 1.1 |  | 1.1 | -1.5 |  |  | 1.1 |  | 1.1 |
| Enc Time[%] | 119% |  |  |  |  | 118% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 103% |  |  |  |  |  |

# Comparison with TE12 results

Rotational transform was tested in TMuC 0.7 as part of TE12 [JCVTV-C-202], [JCVTV-C-036]. True anchor for this tool evaluation is MDDT off and ROT off.  Full frame test results are summarized in Table 5. This information is provided here as supplementary data in order to show that fast ROT has almost the same performance as previous version with essentially smaller encoding complexity.

Table 5 Test results: TMuC 0.7 ROT on MDDT off vs TMuC 0.7 ROT off MDDT off.

|  |  |  | Intra |  |  |  |  |  | Intra-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V | Y |  |  | U |  | V |
| Class A | -2.1 |  | 1.1 |  | 2.0 | -2.4 |  |  | -0.5 |  | -0.1 |
| Class B | -1.9 |  | 0.6 |  | 1.2 | -2.2 |  |  | -1.3 |  | -1.0 |
| Class C | -2.3 |  | 0.0 |  | -0.1 | -2.5 |  |  | -2.0 |  | -2.1 |
| Class D | -2.1 |  | -0.1 |  | -0.2 | -2.2 |  |  | -1.9 |  | -2.1 |
| Class E | -2.1 |  | 3.6 |  | 2.8 | -2.6 |  |  | 0.6 |  | -0.5 |
| All | -2.1 |  | 0.9 |  | 0.9 | -2.3 |  |  | -1.2 |  | -1.3 |
| Enc Time[%] | 242% |  |  |  |  | 259% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  | 104% |  |  |  |  |  |
|  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y | U |  | V |  |  | Y | U |  | V |  |
| Class A | -0.9 | 0.5 |  | 0.9 |  |  | -1.0 | -0.4 |  | -0.2 |  |
| Class B | -0.9 | 0.0 |  | 0.4 |  |  | -0.9 | -0.8 |  | -0.9 |  |
| Class C | -1.0 | -0.5 |  | -0.5 |  |  | -1.0 | -0.7 |  | -0.8 |  |
| Class D | -0.8 | 0.0 |  | -0.2 |  |  | -0.7 | -0.8 |  | -0.6 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | -0.9 | -0.1 |  | 0.1 |  |  | -0.9 | -0.7 |  | -0.7 |  |
| Enc Time[%] | 105% |  |  |  |  |  | 107% |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |  | 100% |  |  |  |  |

# Conclusions

Based on these results, we recommend the ROT for the next version of the HEVC test-model.

# References

- M.Karczewicz, P.Chen, R.Joshi, X.Wang, W-J.Chien, R.Panchal, “Video coding technology proposal by Qualcomm Inc”, JCTVC-A121, Dresden,  Germany, April 2010.

- A. Saxena, F. C. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, JCT-VC-C108, Guangzhou, October 2010.

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- F. C. Fernandes, “Low-Complexity Rotational Transform”, JCT-VC-C096, Guangzhou, October 2010.

- “TE12.4: Experimental results of MDDT and ROT by Samsung and Qualcomm.” JCT-VC-C202, Guangzhou, October 2010

- “Alternative performance measurement of MDDT and ROT in TMuC”, JCT-VC-C36, Guangzhou, October 2010

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).