| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D358 WG11 Number: m19127 |
| --- | --- |

| Title: | CE10: Samsung’s crosscheck on number of intra prediction directions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Junghye Min Jianle Chen | Email: | jh643.min@samsung.com jianle.chen@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the cross-check result CE10: on number of intra prediction directions. The results of 12 tests cases are verified and match with those of all other cross-checkers.

# Introduction

An increased number of directions for intra prediction was introduced in the TMuC and was integrated into the HEVC Test Model[1]. The purpose of this test is to characterize the performance and complexity of using different numbers of intra prediction directions. The characterization was performed by investigating the coding efficiency improvement in terms of BD-BR [2] performance, and by determining the complexity increase for both encoding and decoding using variety of combinations of intra prediction directions for each block size.

# Test Configuration

## Test points

The test points are identified in the table below with respect to the number of intra prediction directions up to the one defined for the HEVC Test Model default configurations [3]. The test has been conducted using TMuC0.9 with modifications to realize the limitation of number of intra prediction directions.

Table 1. Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 |

Test #12: To evaluate the upper limit of the performance of the intra prediction method of the test model,

the reference without fast mode decisions, which means that all intra prediction directions are evaluated using RD cost, is also tested.

## Software

TMuC0.9 has been used with modification for the CE tests. The modifications for test cases #1 to #9 and #11 are only modifications of the tables shown in Table 3 and 4 based on Table 1.

For the test case #10, as maximum index for 64x64 intra prediction is 3, mode mapping table for most probable mode calculation has been modified to limit the mapped mode less than 3.

For the test case #12, the best intra prediction mode is selected by evaluating R-D cost for all modes.

## Test Sequences, Bit Rates and Coding Conditions

Test conditions are the intra-only configuration on both High Efficiency and Low Complexity conditions and test sequences as defined in C500[3], using the reference configuration files also provided in [3].

# Experimental results

Experimental results of all test cases in listed in Table 4 with BR rate saving and encTime measurement.

Table 4. Test results on the common test sequences

|  | Block size and number of directions |  |  |  |  | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | EncTime | Bdrate | EncTime |
| Reference | 17 | 34 | 34 | 34 | 5 |  |  |  |  |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 1.8 | 94% | 1.9 | 94% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 5.1 | 86% | 6.3 | 79% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0.6 | 91% | 1.1 | 100% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0.6 | 96% | 1.2 | 101% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 1.2 | 90% | 2.2 | 93% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0.4 | 90% | 0.3 | 100% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0.7 | 97% | 0.5 | 100% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0.1 | 97% | 0.0 | 99% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0.1 | 95% | -0.0 | 103% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0.0 | 93% | 0.0 | 101% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 2.6 | 93% | 3.4 | 94% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0.2 | 201% | -0.7 | 224% |

# Conclusion

In CE10 summary document, best number of intra prediction directions for each block size is made according to coding efficiency and complexity of 12 test Cases. We recommend considering it for the next version of HEVC Test Model.

# References

- T. Wiegand, “HEVC text specification Draft 1”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C403, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.