| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D360 |
| --- | --- |

| Title: | CE11: Low-complexity adaptive coefficients scanning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Woo-Jin Han | Email: | v.seregin@samsung.com jianle.chen@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In this document Adaptive Coefficient Scanning (ACS) with three scanning pattern is investigated and tested. The scanning index for every Transform Unit (TU) is explicitly signalled to the decoder side in the method proposed in JCTVC-C205 [1]. In the optimized method, mode dependent coefficient scanning proposed in JCTVC-D393 [2] is used to derive scan mode for intra 4x4 and 8x8 blocks to reduce encoder complexity. Experimental results shows optimized ACS provides 1.0%, 0.6% and 0.6 % BD-rate gain in high efficiency (HE) configurations and 0.1%, 0.2% and 1.1% BD-rate gain in low complexity (LC) configurations, respectively for intra-only, random access, and low-delay test conditions.

# Adaptive Coefficients Scanning

Adaptive Coefficient Scanning (ACS) was firstly introduced in A124 [3], following by JCTVC-C205 [1], and consists of three scanning patterns: conventional zigzag, horizontal and vertical patterns, where scanning pattern index is signalled TU-wise to the decoder side for luma blocks. And, luma scanning index of the same block is applied for chroma colour component transform unit. The overhead is signaled for each luma TU with non-DC coefficient.

The optimized ACS applies mode dependent coefficient scanning proposed in JCTVC-D393 [2] to reduce burden of overhead bits as well as encoding complexity. The mode dependent scanning index derivation method is used to derive scan mode for intra 4x4 and 8x8 block instead of explicit signalling. Only scan index for inter block are selected by best RD cost and explicitly signalled to decoder side.

The following mapping table for 4x4 and 8x8 is used.

IntraDirToScanIdx[34] = {1, 2, 0, 0, 1, 1, 0, 2, 2, 0, 0, 1, 1, 0, 0, 2, 2, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0, 0, 2, 2, 2, 2, 0, 0, },

where the array index is luma intra mode direction.

# Experimental results

The proposed algorithms were implemented on top of TMuC 0.9 and performance was evaluated according to CE11 description [4]. Both original ACS with explicit signalling and optimized ACS are compared to default context-adaptive scanning method scan in high efficiency configuration and zigzag in low complexity configuration.

Following tables represent total average BD-rate saving for each testing configuration with encoder and decoder running time. It also should be mentioned here that running time may have some deviation as we ran simulation tests on the computer cluster.

Tab.1 shows the results of original ACS with explicit signalling for all TU type. In this test, optimal scanning pattern among zigzag, horizontal and vertical scan is selected and signalled for each TU with non-DC coefficients. Results of LC configuration are marked as gray since they are not cross-verified by cross-checker.

Tab.1 Explicit signalling ACS

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Intra | -1.2 | -1.6 | -1.6 | 121% | 100% |
| HE Random access | -0.5 | -0.9 | -0.8 | 113% | 98% |
| HE Low delay | -0.6 | -0.9 | -0.9 | 114% | 102% |
| LC Intra | -0.1 | -0.7 | -0.8 | 137% | 99% |
| LC Random access | -0.1 | -0.6 | -0.7 | 111% | 100% |
| LC Low delay | -1.0 | -2.2 | -2.2 | 109% | 100% |

Tab.2 shows the results of optimized ACS. In this test, optimal scanning pattern among HM default scan, horizontal and vertical scan is derived from mapping table for intra 4x4 and 8x8 block and signalled for inter TU with non-DC coefficients. Results shows that the consistent gain was achieved by proposed method in 6 configurations with small encoding time increase.

Tab.2 Optimised ACS

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Intra | -1.0 | -1.2 | -1.2 | 102% | 102% |
| HE Random access | -0.6 | -0.8 | -0.7 | 113% | 104% |
| HE Low delay | -0.6 | -0.9 | -0.8 | 108% | 98% |
| LC Intra | -0.1 | -0.3 | -0.4 | 98% | 100% |
| LC Random access | -0.2 | -0.5 | -0.4 | 113% | 104% |
| LC Low delay | -1.1 | -2.1 | -2.3 | 105% | 97% |

# Conclusion

Optimised adaptive coefficient scanning method was proposed in this contribution according to the trade-off between complexity and performance. We recommend adopting the optimised ACS as the consistent coefficient scanning method for both high efficiency and low complexity configuration in next version of HEVC Test Model.

# References

- V. Seregin and J. Chen, “Low-complexity adaptive coefficients scanning”, JCTVC-C205, JCT-VC Meeting, Guangzhou, Oct. 2010

- Y. Zheng, M. Coban, et al, “CE11: mode dependent coefficient scanning”, JCTVC-D393, JCT-VC Meeting, Daegu, Jan. 2010

- K. McCann, et al, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, 1st JCT-VC Meeting Document JCTVC-A124, Apr. 2010.

- V.Sze, “CE11: Coefficient scanning and coding”, JCTVC-C511, JCT-VC Meeting, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

Samsung Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).