| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D362 WG11 Number: m19137 |
| --- | --- |

| Title: | CE12: Summary Report of Adaptive Motion Vector Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien 5775 Morehouse Dr. San Diego, CA 92130, USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a summary of CE12: Adaptive motion vector resolution. Eight companies have been registered in CE12 and three proposed tools have been evaluated on the common condition with cross-verification after all. Detailed results will be reported by proponents.

# Introduction

In this core experiment, the performance and value of adaptive motion vector resolution (AMVRES) is evaluated in HEVC Test Model under Consideration 0.9 (TMuC 0.9). In AMVRES, motion vector resolution is signaled adaptively. Two out of four original proposals were evaluated. Four organizations participated in cross-check. The characterization was performed by investigating the coding efficiency improvement in terms of BD-BR [2] performance, and by determining the complexity by encoding and decoding time.

# Document list

- Summary document

  - JCTVC-D362 [W.-J. Chien] Summary of CE12 :Adaptive Motion Vector Resolution

- Proponent's documents

  - JCTVC-D394 [Qualcomm] CE12: Adaptive Motion Vector Resolution proposed by Qualcomm.

  - JCTVC-D325 [Nokia] CE12: Nokia proposal on adaptive MV resolution with directional filters

- Cross-checking documents

  - JCTVC-D069 [Sharp] CE12: Verification of Qualcomm's contribution on AMVRES

  - JCTVC-D153 [Sejong Univ./SKT] CE12: Cross-check result of Nokia’s adaptive MV resolution with directional filters by Sejong Univ./SKT

  - JCTVC-D153 [MediaTek] CE12: Crosscheck of Nokia’s adaptive MV resolution by MediaTek

  - JCTVC-D361 [Samsung] CE12: Cross-verification of Qualcomm’s experimental results of AMVRES by Samsung

# Proposed Tools

Seven tools have been proposed.

Table 1  Proposals

| Proponent | Method |
| --- | --- |
| Qualcomm | Adaptive motion vector resolution |
| Nokia | Adaptive MV resolution with directional filters |

# Cross-verification

Proponents provided their source code exchange to all participants. Cross-checkers run the software independently and compared their results with proponent’s results.

Table 2  Cross checkers

| Proponent | Cross-checker |
| --- | --- |
| Qualcomm | Samsung Sharp |
| Nokia | MediaTek Sungkyunkwan Univ. Sejong Univ. SK Telecom |

# Experimental results

All experiments for all sequences based on the common conditions, JCTVC-C500 [1] have been conducted. On this common condition, the following evaluation criteria are defined.

1.	Coding performance: objective rate-distortion measurements using PSNR, BD-PSNR, BD-Rate.

2.	Complexity: encoding/decoding times, including more detailed analysis on software.

## Coding efficiency

Since Table 3 and  show results of coding efficiency from anchor condition, the smaller values mean better results.

These values are calculated by using 4-point BD-rate.

Note that these values of compression ratios are rough analyses and correct values will reported by each proponent.

Table 3  Summary of BD-bitrate (%) on low complexity anchor

| Proponent | Random access | Low delay | Average |
| --- | --- | --- | --- |
| Qualcomm | -1.3% | -1.2% | -1.2% |
| Nokia | -0.6% | -0.6% | -0.6% |

Table 4  Summary of BD-bitrate (%) on high efficiency anchor

| Proponent | Random access | Low delay | Average |
| --- | --- | --- | --- |
| Qualcomm | -1.3% | -1.3% | -1.3% |
| Nokia | N/A | N/A | N/A |

## Complexity analyses

These results of encoding and decoding time on Table 5and Table 6 have been conducted by each proponent independently.

Table 5  Summary of complexity analyses on low complexity anchor

| Proponent | Encoding time (%) |  | Decoding time (%) |  |
| --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay |
| Qualcomm | 104% | 103% | 102% | 102% |
| Nokia | 109% | 98% | 100% | 106% |

Table 6  Summary of complexity analyses on high efficiency anchor

| Proponent | Encoding time (%) |  | Decoding time (%) |  |
| --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay |
| Qualcomm | 109% | 107% | 102% | 92% |
| Nokia | N/A | N/A | N/A | N/A |

## Additional results

Table 7  Summary of BD-bitrate (%) on low delay anchor

| GPB =0 | Low delay | Low delay low complexity |
| --- | --- | --- |
| TMuC0.9 | 6.2% | 2.9% |
| Qualcomm | 1.6% | 1.6% |

Table 8  Summary of complexity analyses on low delay anchor

| GPB =0 | Encoding time (%) |  | Decoding time (%) |  |
| --- | --- | --- | --- | --- |
|  | Low delay | Low delay low complexity | Low delay | Low delay low complexity |
| TMuC0.9 | 58% | 46% | 92% | 83% |
| Qualcomm | 59% | 48% | 94% | 86% |

# Conclusion

In this core experiment, two proposals have been tested and their performances have been verified.

# Reference

- W.-J. Chien, “Core Experiment 12: Adaptive Motion Vector Resolution”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C512, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.