| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D365 |
| --- | --- |

| Title: | Fast Integer Transforms for the HM, and Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoon Mi Hong Il-Koo Kim Kyo-Hyuk Lee Wei Dai Madhu Krishnan Pankaj Topiwala | Email: | yoonmi.hong@samsung.com ilkoo.kim@samsung.com kyohyuk.lee@samsung.com daisy@fastvdo.com madhu@fastvdo.com pankaj@fastvdo.com |
| Source: | Samsung Electronics Co., Ltd. and FastVDO LLC |  |  |

_____________________________

# Abstract

The current HM (TMuC-0.9) utilizes integer transforms of sizes 4-pt to 32-pt, designed using Chen's factorization of the DCT, which provide good decorrelation performance and moderate complexity.  This contribution summarizes the performance and the complexity of the existing transform cores and introduces new integer transforms and several variants, which offer substantial computational savings with effectively no performance loss.  The current 4-pt and 8-pt transforms are initially retained, while the more complex 16-pt and 32-pt transforms are systematically simplified, even including multiplication-free designs. The multiplication-free designs afford exact invertibility in limited bitdepth arithmetic, yet have identical performance. In addition, we have developed a simplified approach to quantization matrices – just replacing them with scalar values.  This can be done for both the existing HM transforms, as well as the new transforms in our design.  In sum, we believe the transform and quantization designs in the HM can be significantly improved computationally, with no noticeable loss of performance.

# Introduction

H.264/AVC supports the DCT-like integer transforms for 4×4 and 8×8 blocks. However, its direct extension to large sizes such as 16×16, 32×32 and 64×64 is not easy because of the dedicated design for 4×4 and 8×8 only. Recently, integer transforms for 16×16, 32×32 and 64×64 sizes based on Chen’s fast algorithm was proposed and implemented in the current HM [1]. While Chen’s factorization has fully recursive and regular butterfly structure which is beneficial to hardware design, it is not as fast as the algorithm based on Loeffler’s [2][3]. New integer transforms based on Loeffler’s algorithm was proposed in [6], which are less complex than the current large size transforms. While those transforms show significant performance over small size transforms, they do carry additional computational complexity. It is therefore sensible to consider ways to reduce the complexity of large-size transforms, along with their quantization schemes. This contribution develops new integer transforms with reduced complexity, as well as the potential to reduce memory requirements by simplifying quantization and dequantization matrices. The complexity of the transforms can be substantially reduced by introduced lifting-based implementations, and especially by using a multiplication-free design [4]. These designs provide identical performance to the designs with multiplications, but provide dramatically lower complexity, and exact invertibility in low bitdepth arithmetic. For these reasons, we advocate the mult-free designs for HM.

# Current large size transforms in HM with simplified quantization/dequantization matrices

As the current core transforms are scaled DCTs with integer arithmetic, post/pre-scaling matrices are integrated into the quantization and dequantization processes. Six different integer matrices of size 16×16 and 32×32 are pre-calculated from the post/pre-scaling matrices scaled by 2CoreShift  according to , and stored for 16×16 transform and 32×32 transform, respectively. However, for current large size transforms, as the variance among each coefficient position within one matrix is very small, those matrices can be replaced by one simple representative value for each QP. The performance difference due to this fact is negligible and shown in section 6.

# New integer transforms of large size

New integer 16-point and 32-point transforms which are based on Loeffler’s algorithm were proposed in [6] to reduce the complexity while maintaining coding efficiency. Current 16-point and 32-point transforms in HM are based on Chen’s algorithm and all multiplication factors appeared in the butterfly structure are approximated to the type of q/2p where p is a precision value adjusted according to dynamic range limitation and coding efficiency. New integer transforms are also developed by the same way, so the resulting transforms as well as the current large size transforms are not exactly orthogonal, which does not perfectly reconstruct the residual data. However, the effect of non-orthogonality is insignificant in terms of the performance. In order to investigate the effect of non-orthogonal transforms, the performance of the new integer transforms with/without lifting scheme is compared for lower QPs and shown in Figure 1. As illustrated in Figure 1, non-orthogonal transform does not degrade the performance significantly.

The coding gain of the new integer transforms also reach the original DCT. Since the variance of the quantization/dequantization matrices is also small for new integer transforms, simplified quantization/dequantization matrices are possible as well.

Figure 1. Performance comparison: integer transform without lifting vs with lifting

# New integer transforms with lifting scheme

One possible solution to achieve perfect reconstruction is to apply the lifting scheme [4]. While new integer transforms without lifting scheme are not orthogonal, those transforms combined with lifting scheme are bi-orthogonal regardless of integer approximation of multiplication factors, and they are exactly invertible. Moreover, lifting scheme reduces or even eliminates the number of multiplications, and some scaling operations can be absorbed in the quantization and dequantization step. More detailed complexity analysis will be given in the next section. The inverse transform process with lifting scheme was introduced in [4][7]. While lifting-based approaches certainly provide the lowest complexity transforms, one tradeoff to consider when applying the lifting scheme is that when many multiplication factors are integrated into the quantization step, the variance among the coefficient position in the quantization/dequantization matrices can increase. This makes it difficult to produce simple scalar values for quantization/dequantization matrices, and as a result, those transforms require storing six different integer matrices of size 16×16 and 32×32 for quantization and dequantization, respectively. We will investigate this tradeoff further in the CE that we recommend.

# Complexity analysis

Dynamic range analysis from various measurements and number of operations for new integer transforms are described in [6]. If the lifting scheme is additionally applied to the proposed transforms, the dynamic range calculated from matrix norms increases as shown in Table 1. Dynamic range can be increased according to the number of stages in the butterfly structures. New 16-point and 32-point integer transforms have 5 and 7 stages for processing 1-D transform, respectively. Then, after 2-D 16×16 transform and 32×32 transform, 10 bits and 14 bits can be increased for the worst case, respectively. New integer transforms combined with lifting scheme show the same increment of dynamic range since lifting scheme does not increase the number of the stages. From the view-point of the method proposed in [5], lifting scheme does not increase dynamic range as well.

Table 1. Dynamic range estimation for 16-point and 32-point transforms with/without lifting

|  | Chen 16T | Loeffler 16T | Loeffler 16T + lifting | Chen 32T | New integer 32T | New integer 32T + lifting |
| --- | --- | --- | --- | --- | --- | --- |
| Max. column sum norm | 10.28 | 10.29 | 12.20 | 20.52 | 20.51 | 21.53 |
| Max. row sum norm | 11.25 | 11.25 | 16 | 22.63 | 22.63 | 32 |

The number of operations is reduced further if the lifting scheme is applied to the low complexity transform. Lifting scheme decomposes a plane rotation into 3 lifting scheme, which requires 3 multiplications and 3 additions instead of 4 multiplications and 2 additions for one single rotation.  Even though the total number of operations is the same as 6, smaller number of multiplications has benefit for multiplication free design with bit-shifts and additions. Moreover, some of operations can be integrated into the quantization process, which leads to much less complexity. Number of operations required to compute 2-D inverse transforms is shown in Table 2.  Note that for large transforms, thousands of multiplications can be fully eliminated. This is especially noteworthy if one considers the opposite extreme – a fully matrix-multiplication approach, as in Table 3 (e.g., 32K mults for 32-pt transform).

Table 2. Number of operations for inverse transforms

|  | 16x16 Chen transform (now in HM 0.9) | 16x16 New Integer transform | 16x16 New Integer Transform +lifting | 16x16 New Integer Transform +lifting +Mult-free | 32x32 Chen transform (now in HM 0.9) | 32x32 New integer transform | 32x32 New integer +lifting | 32x32 New integer +lifting +Mult-free |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mults | 1408 | 1344 | 768 | 0 | 7424 | 6656 | 4224 | 0 |
| Adds | 2368 | 2304 | 2496 | 3776 | 12416 | 11904 | 12928 | 22976 |
| Shift | 832 | 768 | 864 | 2784 | 4224 | 3712 | 4480 | 17152 |
| Total | 4608 | 4416 | 4128 | 6560 | 24064 | 22272 | 21632 | 40128 |

Fast transform with butterfly structure has more advantages over matrix multiplication implementation. For 16-point Chen transform, only 44 multiplications and 74 additions are required while matrix multiplication requires 256 multiplications and 240 additions. When the transform size becomes larger, fast algorithm shows significant complexity reduction compared to matrix multiplication. Number of multiplications and additions required to compute 2-D inverse transforms is shown in .

Table 3. Number of operations for inverse transforms calculation

|  | 16x16 matrix multiplication | 16x16 Chen transform | 16x16 Loeffler transform | 32x32 matrix multiplication | 32x32 Chen transform | 32x32 New integer transform |
| --- | --- | --- | --- | --- | --- | --- |
| Multiplication | 4096 | 1408 | 1344 | 32768 | 7424 | 6656 |
| addition | 3840 | 2368 | 2304 | 31774 | 12416 | 11904 |

Software processing cycles can be compared. Let’s assume that 1 clock is required for processing 1 multiplication and 1 addition, respectively. Then the fast transform with butterfly structure is usually 2~4 times faster than matrix multiplication. Table 4 shows the number of cycles according to the number of multipliers and adders.

Table 4. Software processing cycles

|  | 16x16 matrix multiplication | 16x16 Chen transform | 16x16 Loeffler transform | 32x32 matrix multiplication | 32x32 Chen transform | 32x32 New integer transform |
| --- | --- | --- | --- | --- | --- | --- |
| 1 multiplier, adder | 7936 | 3776 | 3648 | 64542 | 19840 | 18560 |
| 2 multiplier, adder | 3968 | 1888 | 1824 | 32271 | 9920 | 9280 |
| 16 multiplier, adder | 496 | 236 | 228 | 4034 | 1240 | 1160 |

Regarding hardware resources, processing cycles can also be compared. The proposed fast transforms have butterfly structures which have several stages. Chen’s 16-point transform has 6 stages and Loeffler’s 16-point transform has 5 stages. In each stage, certain number of multiplications and additions are processed. For example, Chen’s 16-point transform needs at most 16 multipliers and 16 adders in each stage, which requires totally 194 (= (6*16+1)*2) cycles. However, matrix multiplication needs 520 ((= 256+4)*2) processing cycles. Assuming the same hardware resources, processing cycles are compared according to the number of multipliers and adders in Table 5.

Table 5. Hardware processing cycles

|  | 16x16 matrix multiplication | 16x16 Chen transform | 32x32 matrix multiplication | 16x16 Chen transform |
| --- | --- | --- | --- | --- |
| 16 multiplier, adder | 520 | 194 | 4116 | 1028 |
| 32 multiplier, adder | 260 | 97 | 2058 | 514 |
| 64 multiplier, adder | 130 | 49 | 1029 | 257 |
| 128 multiplier, adder | 65 | 25 | 515 | 129 |

# Experimental results

Simplified quantization and dequantization matrices for the existing transform cores and new integer transforms are implemented in the HM (TMuC-0.9) software. Simulation results are presented for the full-frame tests with six different configurations. The performance change was negligible for both transforms as shown in Table 6 and Table 7. New transform cores with/without lifting scheme were implemented, and the results are shown in Table 8 and Table 9.

Table 6. Performance of simplified Quantization/Dequantization matrices for HM

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.11 | 0.12 | 0.09 | 0.12 | 0.09 | 0.10 |
| Class B | 0.16 | 0.02 | 0.08 | 0.12 | 0.01 | 0.01 |
| Class C | 0.03 | 0.07 | 0.03 | 0.01 | 0.02 | 0.05 |
| Class D | 0.02 | 0.06 | 0.02 | 0.00 | 0.01 | 0.01 |
| Class E | 0.10 | -0.06 | -0.21 | 0.06 | 0.06 | 0.05 |
| All | 0.09 | 0.04 | 0.01 | 0.06 | 0.03 | 0.03 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.00 | 0.03 | 0.39 | 0.03 | 0.13 | 0.13 |
| Class B | 0.04 | 0.00 | -0.14 | 0.03 | -0.03 | -0.03 |
| Class C | 0.00 | 0.01 | 0.00 | 0.01 | -0.08 | -0.08 |
| Class D | -0.01 | 0.21 | 0.31 | 0.02 | 0.08 | -0.03 |
| Class E |  |  |  |  |  |  |
| All | 0.01 | 0.06 | 0.09 | 0.02 | 0.01 | -0.02 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.01 | 0.02 | -0.15 | -0.06 | -0.06 | -0.14 |
| Class C | -0.05 | -0.04 | 0.03 | 0.01 | -0.05 | 0.05 |
| Class D | 0.02 | -0.28 | 0.15 | -0.02 | 0.28 | 0.01 |
| Class E | -0.09 | 0.09 | -0.61 | -0.17 | -0.42 | -0.08 |
| All | -0.03 | -0.06 | -0.12 | -0.05 | -0.04 | -0.04 |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

Table 7. Performance of simplified Quantization/Dequantization matrices for new integer transform

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.12 | 0.14 | 0.01 | 0.05 | 0.07 |
| Class B | 0.05 | 0.02 | 0.05 | 0.03 | -0.01 | -0.03 |
| Class C | 0.00 | 0.03 | 0.01 | 0.00 | 0.00 | -0.01 |
| Class D | 0.00 | 0.04 | 0.05 | -0.01 | -0.01 | 0.02 |
| Class E | 0.00 | -0.02 | -0.11 | -0.01 | 0.00 | -0.01 |
| All | 0.01 | 0.03 | 0.02 | 0.01 | 0.00 | 0.00 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.18 | 0.28 | -0.01 | 0.12 | 0.07 |
| Class B | 0.00 | -0.01 | 0.18 | -0.02 | 0.04 | 0.04 |
| Class C | -0.01 | -0.01 | 0.03 | -0.02 | -0.11 | -0.01 |
| Class D | -0.04 | -0.13 | 0.18 | 0.00 | -0.24 | 0.22 |
| Class E |  |  |  |  |  |  |
| All | -0.02 | -0.02 | 0.15 | -0.01 | -0.06 | 0.08 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.02 | 0.05 | -0.05 | -0.06 | -0.05 | 0.00 |
| Class C | -0.01 | -0.04 | 0.16 | 0.01 | -0.16 | -0.10 |
| Class D | 0.04 | -0.48 | 0.25 | -0.06 | 0.10 | -0.08 |
| Class E | -0.12 | 0.09 | -0.30 | -0.13 | 0.34 | -0.07 |
| All | -0.02 | -0.10 | 0.03 | -0.05 | 0.03 | -0.06 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 8. Performance of new integer transforms

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.03 | -0.08 | -0.06 | -0.02 | -0.03 |
| Class B | -0.03 | -0.05 | -0.04 | -0.01 | -0.03 | -0.03 |
| Class C | -0.01 | 0.00 | -0.01 | -0.01 | 0.00 | 0.02 |
| Class D | -0.01 | 0.01 | 0.02 | -0.01 | 0.02 | 0.01 |
| Class E | -0.04 | -0.04 | -0.08 | -0.02 | -0.04 | -0.01 |
| All | -0.02 | -0.02 | -0.03 | -0.01 | -0.01 | -0.01 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.26 | 0.29 | -0.02 | 0.15 | 0.04 |
| Class B | -0.03 | -0.05 | -0.18 | 0.00 | -0.04 | 0.04 |
| Class C | 0.03 | 0.04 | 0.05 | -0.03 | 0.04 | 0.02 |
| Class D | 0.01 | -0.08 | 0.14 | 0.00 | -0.10 | 0.00 |
| Class E |  |  |  |  |  |  |
| All | 0.00 | 0.01 | 0.03 | -0.01 | -0.01 | 0.02 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.01 | -0.05 | 0.07 | 0.00 | -0.01 | -0.08 |
| Class C | -0.03 | -0.13 | -0.02 | 0.02 | -0.20 | -0.02 |
| Class D | 0.01 | -0.45 | -0.24 | -0.05 | 0.02 | 0.03 |
| Class E | -0.18 | 0.07 | -0.05 | 0.00 | 0.39 | 0.44 |
| All | -0.03 | -0.15 | -0.05 | -0.01 | 0.03 | 0.06 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

Table 9. Performance of new integer transforms + lifting

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.05 | -0.08 | 0.04 | 0.01 | 0.02 |
| Class B | -0.02 | 0.00 | 0.02 | 0.02 | -0.01 | -0.01 |
| Class C | 0.00 | 0.01 | 0.00 | 0.00 | 0.01 | 0.02 |
| Class D | 0.00 | 0.01 | -0.02 | 0.00 | 0.01 | -0.02 |
| Class E | -0.05 | -0.01 | -0.13 | 0.02 | 0.04 | 0.07 |
| All | -0.02 | 0.00 | -0.03 | 0.01 | 0.01 | 0.01 |
| Enc Time[%] | 103% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.08 | 0.22 | -0.01 | 0.05 | 0.01 |
| Class B | -0.01 | 0.07 | 0.05 | -0.01 | -0.01 | 0.04 |
| Class C | -0.02 | -0.02 | -0.02 | 0.00 | -0.09 | -0.01 |
| Class D | 0.00 | 0.13 | 0.09 | 0.00 | -0.23 | 0.07 |
| Class E |  |  |  |  |  |  |
| All | -0.01 | 0.06 | 0.06 | 0.00 | -0.08 | 0.03 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.00 | -0.11 | -0.21 | 0.03 | -0.09 | -0.12 |
| Class C | 0.01 | 0.09 | -0.11 | 0.06 | -0.16 | 0.06 |
| Class D | 0.03 | -0.28 | 0.22 | -0.03 | 0.25 | -0.14 |
| Class E | -0.20 | 0.10 | -0.49 | 0.04 | -0.06 | 0.03 |
| All | -0.02 | -0.06 | -0.13 | 0.02 | -0.01 | -0.05 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

# Conclusions

The improvement of large size transforms in various aspects is described in this proposal. Simplified quantization and dequantization matrices reduce memory requirements. The complexity of transform cores themselves can be reduced by newly developed integer transforms and their variants with lifting scheme. Those variants with/without lifting scheme have pros and cons regarding invertibility, complexity, and the possibility of simplified quantization and dequantization. The designs using lifting and multiplication-free offer the greatest savings in complexity, while providing excellent performance.  But the quantization structure needed for that approach is not mature at this time. We therefore propose adoption of simplified quantization and dequantization matrices to the current HM and, as a next step, creation of CE to investigate those issues in detail, and return with better recommendations on the transform and quantization structures.

# References

- K. McCann, W. J. Han, I. K. Kim, J. H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y. M. Hong, M. S. Cheon, and N. Shlyakhov,  Samsung's response to the call for proposals on video compression technology,  in 1st JCT-VC meeting , no. JCTVC-A124, Apr. 2010.

- W.-H. Chen, C. H. Smith, and S. C. Fralick, A fast computational algorithm for the discrete cosine transform, IEEE Trans. Commun., vol. COMM-25, pp. 1004-1009, Sep. 1977.

- C. Loeffler, A. Lightenberg, and G. Moschytz, Practical fast 1-D DCT algorithms with 11 multiplications,  Proc. IEEE ICASSP, vol. 2, pp. 988–991, Feb. 1989.

- J. Liang and T. D. Tran, Fast multiplierless approximations of the DCT with the lifting scheme, IEEE Trans. Signal Process., no. 12, pp. 3032-3044, Dec. 2001.

- X. Wan, Y. Wang, and W. Chen, Dynamic range analysis for the implementation of fast transform, IEEE Trans. Circuits Syst. Vid. Technol., vol. 5, pp. 178–180, Apr. 1995.

- Yoon Mi Hong, Min-Su Cheon, Il-Koo Kim, Low complexity 16x16 and 32x32 transforms and partial frequency transform, in 3rd JCT-VC meeting, no. JCTVC-C209, Oct. 2010.

- J. Liang, T. Tran, P. Topiwala, “A 16-bit architecture for H.26L, treating DCT transforms and quantization,” ITU VCEG doc M16, Austin, TX, April, 2001.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. and FastVDO LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).