| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D366 WG11 Number: m19141 |
| --- | --- |

| Title: | CE5: Improved intra prediction mode coding with LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE report |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Xianglin Wang, Wei-Jung Chen (Qualcomm) Arild Fuldseth (Cisco) | Tel: Email: | +1-858-658-5673 martak@qualcomm.com arilfuld@cisco.com |
| Source: | Qualcomm, Cisco |  |  |

_____________________________

# Abstract

In this report, coding results of an improved intra prediction mode coding scheme are reported. Results show that based on the CE test conditions, an average coding gain of 0.5% can be achieved.

- Introduction

In the last JCT-VC meeting in Guangzhou, it was reported in proposals [1][2] that the overhead in signaling intra prediction mode can be reduced with more suitable VLC table selection and appropriate symbol combination with low complexity entropy coding (LCEC). This document reports further results based on the CE test conditions and TMuC 0.9.

# Proposed Approach

Coding of intra prediction modes is modified for 4x4 blocks (17 modes), 16x16 and 32x32 blocks (34 modes). Let us consider for example 4x4 block mode coding. If the chosen intra prediction mode iDir is equal to the most probable mode mostProbMode, code number 0 is assigned to this mode. Otherwise if iDir>mostProbMode prediction mode index iDir is modified: iDir = iDir -1, and the codeword number n is found for iDir using a mapping table. There is a single mapping table used for 4x4 blocks. Finally, the VLC codeword is assigned to the code number using one of the 2 VLCs tables. Which table is used depends whether intra prediction modes of its left and above neighboring blocks are same. The same idea is also applied to 16x16 and 32x32 blocks, but with one set of tables (single mapping table and 2 VLC tables) shared between these two blocks sizes. The mapping table is made adaptive. The same adaptation process is same as used for other syntax elements when encoded with low complexity entropy coding (LCEC).

# Simulation Results

Based on test conditions specified in CE5, simulations are performed using all three low complexity configurations. Detailed results can be found in the associated excel data sheet.

The results show the proposed scheme provides an average coding gain of 0.5% with all intra configuration, and 0.2% with random access configuration. Due to lack of intra blocks, there is only very slight coding gain with low delay configuration. With all intra configuration, the coding gain is rather consistent for all test sequences.

# Conclusion

This contribution presents CE results on intra prediction mode coding. Based on the results, we recommend this improved scheme for LCEC intra mode coding to be adopted into TM.

# References

[1] M. Karczewicz, W.-J. Chien, X.Wang “Improvements on VLC”, JCTVC-C263, Guangzhou, China, Oct 2010.

[2] A. Fuldseth, G. Bjontegaard, A. Hallapuro, K. Ugur, J. Lainema, “Recent improvements on the low complexity entropy coder (LCEC) in TMuC”, JCTVC-C185, Guangzhou, China, Oct 2010.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Cisco Systems Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).