| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D368 |
| --- | --- |

| Title: | CE2: Overlapped Block Motion Compensation for Geometry Partition Block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is related to core experiment 2: flexible motion partitioning. Overlapped block motion compensation (OBMC) [1] has been presented in previous JCTVC meetings. In this contribution, we describe an improved OBMC method and provide its performance on geometry partition (GEO).

# Introduction

OBMC [1] uses overlapped blocks in motion compensation to reduce blocking artifact along macroblock boundaries. When more flexible motion partition methods such as geometry partition are used, OBMC can improve coding efficiency by smoothing out the partition boundaries and reducing the high frequency energy, resulting in better transform coding efficiency and visual quality.

# OBMC for Geometry partition

In geometry partition, a 2Nx2N block can be partitioned into two parts by a straight line, and the partition line can be of arbitrary orientations.  In this case, it is highly possible that a block transform covers pixels from two partitions. Since the two partitions may have different motion vectors, pixels at the block boundary may have large discontinues, which may generate visual artifacts and also reduce coding efficiency.

Last meeting, we presented an OMBC method in [1]. In this contribution, an improved OMBC method is presented. Figure 1 shows a geometry partitioned block where white pixels are pixels belonging to Partition 1 and grey pixels are pixels belonging to Partition 2. For every pixel, we check its 4-connected neighborhood. If at least one neighborhood pixel belongs to the other portion, the current pixel is classed as a pixel in Transition Region A. For pixels do not belong to Transition A, we check its distance  2 diamond (diamond-2) neighborhood. If at least one neighborhood pixel belongs to the other portion, the current pixel is classed as a pixel in Transition Region B.

For pixels in Transition Regions (both A and B), the motion compensation is performed using a weighted sum of the motion predictions from the two motion vectors. For pixels in Region A, the weights are 3/4 for the prediction using the motion vector of the partition containing the current pixel and 1/4 for the prediction using the motion vector of the other partition.  For pixels in Transition Region B, the weights are 7/8 and 1/8.

Figure 1 Overlapped Motion Compensation for Geometry Partition

Figure 2: 4-connected neighborhood (left) and diamond-2 neighborhood (right)

# SIMULATION RESULTS

We implement OBMC on top of TMuC 0.9 with simplified geometry partition (simplified geometry partition is released in CE2 [2]). The coding efficiency is evaluated in RandomAccess (RA) and LowDelay (LD) configurations for high efficiency case based on the common conditions defined in JCTVC-C500[4]. The coding efficiency is measured by using 4 point BD-Rate

## R-D Performance

The experimental results are shown in Table 1. Compared to geometry partition without OBMC, the average bit rate reduction in Random Access and Low Delay are 0.6% and 0.6%, respectively.

Table 1 Bit rate reduction from OBMC (compared to geometry partition without OBMC)

|  |  | Random access |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -1.2 | -1.0 |  |  |  |
| Class B | -0.5 | -0.7 | -0.9 | -0.4 | -0.7 | -0.6 |
| Class C | -0.8 | -1.3 | -1.1 | -0.6 | -1.0 | -0.9 |
| Class D | -0.5 | -0.9 | -0.5 | -0.4 | -0.6 | -0.7 |
| Class E |  |  |  | -1.1 | -0.8 | -0.5 |
| All | -0.6 | -1.0 | -0.9 | -0.6 | -0.8 | -0.7 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2 provides the performance for different sequences.

Table 2 . Per Sequrnce bit rate reduction of OBMC

|  |  | RA | LD |
| --- | --- | --- | --- |
| Class A | Traffic | -0.5 |  |
|  | PeopleOnStreet | -1.1 |  |
| Class B | Kimono | -0.3 | -0.3 |
|  | ParkScene | -0.7 | -0.5 |
|  | Cactus | -0.5 | -0.9 |
|  | BasketballDrive | -0.4 | -0.4 |
|  | BQTerrace | -0.3 | -0.1 |
| Class C | BasketballDrill | -0.9 | -0.7 |
|  | BQMall | -1.1 | -0.7 |
|  | PartyScene | -0.4 | -0.3 |
|  | RaceHorses | -0.7 | -0.6 |
| Class D | BasketballPass | -0.6 | -0.4 |
|  | BQSquare | -0.4 | -0.1 |
|  | BlowingBubbles | -0.5 | -0.2 |
|  | RaceHorses | -0.7 | -0.7 |
| ClassE | Vidyo1 |  | -0.9 |
|  | Vidyo3 |  | -1.5 |
|  | Vidyo4 |  | -0.9 |
|  | Class A | -0.8 |  |
|  | Class B | -0.5 | -0.4 |
|  | Class C | -0.8 | -0.6 |
|  | Class D | -0.5 | -0.4 |
|  | Class E |  | -1.1 |
|  | All | -0.6 | -0.6 |

As supplemental information, in  Table 3 we show the total gain of geometry partition with OBMC compared to TMuC 0.9

Table 3  Performance of Geometry Partition with OBMC

|  |  | Random access |  |  | Low Delay |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.9 | -3.1 | -2.8 |  |  |  |
| Class B | -1.4 | -2.1 | -2.2 | -1.6 | -2.7 | -2.9 |
| Class C | -2.4 | -3.4 | -3.3 | -2.1 | -3.0 | -3.3 |
| Class D | -2.0 | -2.6 | -2.4 | -1.9 | -3.2 | -3.1 |
| Class E |  |  |  | -4.0 | -4.4 | -4.1 |
| All | -2.0 | -2.7 | -2.6 | -2.2 | -3.2 | -3.3 |
| Enc Time[%] | 123% |  |  | 131% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  |

## Visual Improvement

By smoothing motion partition boundary, OBMC can suppress artifacts and improve visual quality.  An example is shown in Figure 3.

Figure 3 Frame 332 BasketballDrill (LD, QP37): left (Without OBMC), right (with OBMC)

- P. Chen, R. Panchal, W. Chieng and M. Karczewicz, “Overlapped block motion compensation in TMuC”, JCTVC-C251, JCT-VC Meeting, Guangzhou, CN, Oct. 2010.

- E. Francois , P. Bordes, L. Guo, and M. Karczewicz, “CE2: Simplified Geometry Block Partitioning”, JCTVC-D230, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).