| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D369 |
| --- | --- |

| Title: | CE5: Efficient coefficient coding method for 16x16 and 32x32 transforms in LCEC mode |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Sunil Lee Il-Koo Kim Woo-Jin Han Jeong-Hoon Park Jianle Chen | Email: | sunil.lee@samsung.com ilkoo.kim@samsung.com wjhan.han@samsung.com jeonghoon@samsung.com jianle.chen@samsung.com |
| Source: | [Samsung Electronics Co., Ltd.] |  |  |

_____________________________

# Abstract

This contribution is Samsung’s response to the Core Experiment 5 on the low complexity entropy coding improvements. In this contribution, a new coefficient coding method for 16x16 and 32x32 transforms in LCEC mode, which has been proposed in JCTVC-C210, is presented. Tandberg, Ericsson, and Nokia’s coefficient coding method, which is currently implemented in the HEVC Test Model, has been extended for the efficient coding of the coefficients from 16x16 and 32x32 transforms. A simplified version of the proposed method which does not require any additional variable-length code tables is also presented. Experimental results show that the proposed method provides good coding gain in intra and random access configurations without noticeable complexity increments.

# Introduction

The current coefficient coding method of the HEVC Test Model (HM) in Low Complexity Entropy Coding (LCEC) mode is based on Tandberg, Ericsson, and Nokia’s (TEN) method [1], [2]. Although TEN’s proposal also used large spatial transforms including 16x16, 32x32, and 64x64, only 8 low frequency basis vectors were used even when transforms larger than 8x8 were applied. Since at most 64 transform coefficients from 8x8 low frequency band needed to be coded, TEN’s variable-length code (VLC) engine for coefficient coding was designed for size of up to 8x8 or 64 transform coefficients. Contrary to the TEN’s partial transform, the spatial transforms of the HM proposed by Samsung [3] exploit all the basis vectors, thus the transform coefficients from the whole frequency band need to be coded. However, since TEN’s LCEC lacks an efficient method to code the coefficients from large transforms, only 64 low frequency coefficients are coded in the current HM, which means that high frequency coefficients from 16x16 and 32x32 transforms are truncated without any rate-distortion consideration in LCEC mode. Since this kind of truncation may lead to the decrease in the overall performance in LCEC mode, especially when there exist many high frequency coefficients outside 8x8 low frequency band, an efficient coefficient coding method for 16x16 and 32x32 transforms in LCEC mode should be proposed.

In the last meeting in Guangzhou, we proposed an efficient coefficient coding method for large transforms including 16x16, 32x32, and 64x64 in LCEC mode [4], and this proposal was decided to be included in the Core Experiment 5 (CE5) on the LCEC improvements [7], along with other proposals, e.g. [5] and [6]. This contribution is Samsung’s response to the CE5. In this contribution, we proposed an efficient coefficient coding method for 16x16 and 32x32 transforms based on our previous proposal. A simplified version of the proposed method, which can achieve the considerable coding gain by simply extending the existing coefficient coding method for 4x4 and 8x8 transforms without introducing any additional VLC tables, is also presented.

# Algorithm description

In [4], we proposed a LCEC coefficient coding method which can code all the transform coefficients from the whole frequency band. In addition to this full coefficient coding method, we also proposed to use our method only for 16x16 transform, where only the coefficients from 16x16 low frequency band are coded in 32x32 and 64x64 transforms. Since it was already shown in [4] that the full coefficient coding does not provide meaningful performance improvement over the extension only to 16x16 transform, we decided to implement our method only for 16x16 transform in HM, which means that only 16x16 low-frequency coefficients are coded when the transform size is 32x32. Please refer to [1] and [4] for the details of this method.

In this contribution, a simplified version of the proposed method is also presented. In this method, the coefficient coding method for 8x8 transform is also used for 16x16 transform. Only one exceptional case which cannot be covered by the method for 8x8 transform is the zero-run longer than 63, since the maximum length of the run in 8x8 transform is 63, thus the VLC table defined for run-level coding in 8x8 does not include the codeword for the run longer than 63. In the proposed simplified method, this problem is resolved by using the direct coding method which has been used to further extend the coding method for 16x16 to 32x32 and 64x64 transforms in [4]. In this method, when the length of the preceding zero-run is longer than 63 in 16x16 or 32x32 transforms, an escape code which is not included in the VLC table for run-level coding in 8x8 transform is written first, and then the remaining run value is simply written. The run-level coding in 16x16 and 32x32 transform in the proposed simplified method can be summarized as follows.

- If run <= 63

  - Encode the preceding run and levelFlag as in 8x8 transform

- Else

  - Encode the run of 64 and levelFlag as a combined event using the escape codes (specifically, 127 for levelFlag=0 and 128 for levelFlag=1).

  - Encode the remaining run (specifically, run-64).

# Experimental results

The common test condition for low-complexity configuration specified in [7] is used for all the experiments. The detailed experimental results are also available in the attached Excel file.

Table 1. BD-rate saving of the 16x16 partial coefficient coding using the method proposed in [4]

| Config. Class | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5 | -1.0 | -1.4 | -0.9 | 0.4 | -0.1 |  |  |  |
| Class B | -2.1 | -1.3 | -1.7 | -1.4 | 0.1 | -0.3 | 0.1 | 2.4 | 1.9 |
| Class C | -0.6 | -0.4 | -0.6 | -0.5 | 0.0 | 0.0 | 0.1 | 1.0 | 1.1 |
| Class D | -0.5 | -0.5 | -0.8 | -0.3 | -0.3 | -0.3 | 0.0 | 0.8 | 0.5 |
| Class E | -1.9 | -2.2 | -1.9 |  |  |  | 1.1 | 3.3 | 1.8 |
| Average | -1.4 | -1.0 | -1.3 | -0.8 | 0.0 | -0.2 | 0.3 | 1.8 | 1.3 |
| Enc Time | 116% |  |  | 114% |  |  | 105% |  |  |
| Dec Time | 101% |  |  | 108% |  |  | 98% |  |  |

## Performance of the 16x16 partial coefficient coding using the method proposed in [4]

In this experiment, the performance of the 16x16 partial coefficient coding based on the method proposed in [4] is evaluated and summarized in Table 1. Average BD-rate savings are 1.4%, 0.8%, and -0.3% in intra, random access, and low delay configurations, respectively. As shown in the table, the noticeable performance improvement is achieved in intra and random access configurations, where large transforms are frequently selected and there exist many high frequency coefficients. On the other hand, the proposed method does not provide the BD-rate saving in low delay configuration, where large transform is not selected often and most TUs include only few non-zero coefficients in the low frequency band. The complexity of the proposed method measured in terms of encoding and decoding time shows that the proposed method slightly increases the encoder complexity, especially in the intra configuration, while the increase in the decoder complexity is relatively small.

Table 2. BD-rate saving of the 16x16 partial coefficient coding using the simplified method

| Config. Class | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8 | -0.3 | -0.7 | -0.7 | 0.6 | 0.2 |  |  |  |
| Class B | -1.5 | -0.5 | -0.9 | -0.8 | 0.2 | 0.0 | -0.5 | 0.4 | 0.1 |
| Class C | -0.5 | -0.2 | -0.4 | -0.2 | 0.2 | 0.0 | 0.0 | 0.2 | 0.2 |
| Class D | -0.4 | -0.4 | -0.6 | -0.2 | -0.3 | -0.2 | -0.2 | 0.4 | 0.0 |
| Class E | -1.5 | -1.7 | -1.5 |  |  |  | -0.1 | 1.0 | 1.1 |
| Average | -1.1 | -0.6 | -0.8 | -0.5 | 0.1 | 0.0 | -0.2 | 0.5 | 0.3 |
| Enc Time | 112% |  |  | 108% |  |  | 107% |  |  |
| Dec Time | 99% |  |  | 101% |  |  | 100% |  |  |

## Performance of the 16x16 partial coefficient coding using the simplified method

In Table 2, the performance of the 16x16 partial coefficient coding based on the simplified method is summarized. Average BD-rate savings are 1.1%, 0.5%, and 0.2% in intra, random access, and low delay configurations, respectively. As the 16x16 coefficient coding based on the method proposed in [4], the noticeable performance improvement is observed in intra and random access configurations. Although the performance is slightly degraded in intra and random access configurations as compared with the method based on the previous proposal, the gap is not so large, and the performance is even improved in low delay configuration. The encoding complexity of this simplified method is lower than that of the method based on the previous proposal [4], and the decoding complexity is similar to the HM.

# Conclusion

In this proposal, a new coefficient coding method for 16x16 and 32x32 transforms in LCEC mode is presented as a response to the CE5. The experimental results show that the proposed method can effectively improve the coding performance when large transforms with many high frequency coefficients are selected, e.g. for the sequences in class B. We propose to consider the proposed method in the HM since the proposed method is a straightforward extension of the existing coefficient coding method in LCEC mode and provides the coding gain without noticeable complexity increase.

# References

- K. Ugur, K. R. Andersson, and A. Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson,” JCTVC-A119, Dresden, Germany, April 2010.

- T. Wiegand, W.-J. Han, J.-R. Ohm, and G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 1,” JCTVC-C403, Guangzhou, China, October 2010.

- Ken McCann, Woo-Jin Han, and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- Sunil Lee, Min-Su Cheon, and Il-Koo Kim, “Efficient coefficient coding method for large transform in VLC mode,” JCTVC-C210, Guangzhou, China, October, 2010.

- A. Fuldseth, G. Bjontegaard, A. Hallapuro, K. Ugur, and J. Lainema, “Recent improvements of the low complexity entropy coder (LCEC) in TMuC,” JCTVC-C185, Guangzhou, China, October 2010.

- M.Karczewicz, W.-J. Chien, and X.Wang, “Improvements on VLC,” JCTVC-C263, Guangzhou, China, October 2010.

- F. Bossen, “Common test condition and software ,” JCTVC-C500, Guangzhou, China, October 2010.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).