| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D370 WG11 Number: m19145 |
| --- | --- |

| Title: | CE5: Improved coding of inter prediction mode with LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chen, Xianglin Wang, Marta Karczewicz | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com xianglin@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports CE results on modified coding of inter prediction mode with low complexity entropy coding (LCEC). The average gain is 1.9% for random access configuration and 2% for low delay configuration.

- Introduction

Inter prediction modes currently include split flag, skip flag, merge flag, direct flag, Inter_2Nx2N, Inter_Nx2N, Inter_2NxN, Inter_NxN, Intra_2Nx2N, and Intra_NxN. According to the current software, the first three flags are coded in the following order. If split flag is 1, no more mode information is sent; otherwise, an adaptive loop filter flag and a skip flag are sent. If the skip flag is 1, no more mode information is sent; otherwise, a merge flag is sent. If merge flag is 1, no more mode information is sent; otherwise, a direct flag is sent. If the direct flag is 1, no more mode information is sent; otherwise, a VLC code is sent to indicate which mode is chosen among all other modes. Such a coding scheme is applied to CU prediction mode regardless of CU depth.

In the last JCT-VC meeting in Guangzhou, it was reported in proposals [1] that coding efficiency in signaling inter prediction mode can be improved through a joint coding of the prediction modes. This document reports further results based on the CE test conditions and TMuC 0.9.

# Proposed Approach

According to this proposal, split flag, skip flag, merge flag and direct flag (including prediction direction) are grouped and coded together with a symbol Inter_Partition. Unary codes are used. When Inter_Partition in coded, it indicates that additional information about prediction modes will follow. Mapping between symbol and codeword index is adaptive and CU depth dependent. The adaptation scheme is the same as used for other syntax elements. When Inter_Partition is coded, additional code is sent to signal one of other prediction modes for the current CU, e.g. Inter_2Nx2N or Inter_NxN, etc. A fixed VLC table at each CU depth is used for this purpose.

Additionally, we found that when only small number of symbols is coded, LCEC performance can be further improved by replacing current direct-swapping based adaptation scheme with counter based adaptation. Counter is used to track the statistics and adapt the mapping between symbols and codeword indices accordingly. After encoding of each symbol the counter for this symbol is increased. If the current symbol’s counter is greater than the counter of the symbol with the codeword index smaller by one, the codeword indices of these symbols are swapped. We added such adaptation for coded block flag coding and {inter prediction direction, ref index} pair coding.

# Simulation Results

Based on test conditions specified in CE5, simulations are performed using all three low complexity configurations. Detailed results can be found in the associated excel data sheet.

The modifications of inter prediction mode coding provide an average 1.8% coding gain for random access configuration and 1.6% for low delay configuration. If in addition current direct-swapping adaptation scheme is replaced with counter based adaptation for coded block flag and {inter prediction direction, ref index} pair coding, the overall average coding gain is 1.9% for random access configuration and 2% for low delay configuration. In all the cases mentioned above, coding gain is consistent for all test sequences.

The proposed coding scheme has nearly no impact on encoding and decoding speed. In many cases, the decoding is found to be even faster.

# Conclusion

This contribution presents CE results on inter prediction mode coding. Based on the results, we recommend the improved scheme for LCEC inter mode coding to be adopted into TM. We also recommend that for some short VLC table, e.g. the one for CBP coding and (inter prediction direction, ref index) coding, counter based adaptation is used for better coding performance.

# References

[1] M. Karczewicz, W.-J. Chien, X.Wang “Improvements on VLC”, JCTVC-C263, Guangzhou, China, Oct 2010.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).