| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D371 WG11 Number: m19147 |
| --- | --- |

| Title: | Crosscheck of TI's matrix multiplication specification (JCTVC-D036) and DCT + Hadamard large transform (JCTVC-D037) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi, Y. Zheng, and J. Sole | Email: rajanj@qualcomm.com Tel.: +1-858-658-4511 |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of two of TI’s proposals related to transforms. The proposals were matrix multiplication specification (JCTVC-D036) and DCT + Hadamard large transform (JCTVC-D037). The BD-results from our crosscheck matched those provided by TI.

# Introduction

This contribution provides verification results for two transform related proposals from TI, namely, matrix multiplication specification (JCTVC-D036) and DCT + Hadamard large transform (JCTVC-D037). In “matrix multiplication specification”, all the transforms in the current Test Model from 4×4 to 32×32 are replaced by matrix multiplication. Matrix for a smaller block size is derived by subsampling the 32×32 matrix. Partial downshifts are introduced after first 1-D transform to reduce the bit-depth for the transpose buffer.

In “DCT + Hadamard large transform” (JCTVC-D037) the 32×32 transform is replaced by a combination of 2×2 Hadamard and 16×16 DCTs. For a 1-D transformation, first a 2-point Hadamard is performed on the adjacent samples. Then, two 16-point DCTs are performed, one on the low frequency coefficients from the Hadamard transform and another on the high frequency coefficients from the Hadamard transform. It should be noted that the proposed DCT + Hadamard transform is used only for inter blocks. For intra blocks, existing 32×32 transform is used.

- Results

The coding efficiency was crosschecked for both proposals in Intra, random access (RA) and low delay (LD) configurations for both the high efficiency (HE) and low complexity (LC) cases. Tables 1 and 2 show the summary of results for matrix multiplication specification (JCTVC-D036) and DCT + Hadamard large transform (JCTVC-D037), respectively. More detailed coding results can be found in the Excel attachments. No YUV files were written when measuring the decoding times. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++.

For matrix multiplication specification, the BD-results were virtually unchanged from TMuC0.9. Similarly, encoding and decoding times were virtually the same as in TMuC0.9. For the DCT + Hadamard large transform, the worst case BD-rate degradation of 0.4 was for the LD HE case. The encoding times were very close to TMuC0.9. We observed an increase in decoding times, especially for the low delay cases. Looking at the code, the DCT + Hadamard large transform should be at least as fast as the matrix multiplication specification.

Table 1: BD-rate results for matrix multiplication specification

Table 2: BD-rate results for DCT + Hadamard large transform

# Conclusions

We have verified TI’s results for matrix multiplication specification (JCTVC-D036) and DCT + Hadamard large transform (JCTVC-D037). The BD-rate results matched for both matrix multiplication specification and DCT + Hadamard large transform.